3GPP TSG-SA WG2 Meeting #171	S2-2508289

# October 13-17, 2025 Wuhan, China

Source:	Skylo Technologies, ViaSat, Boost Mobile Network, EchoStar, Verizon

Title:	Interim Conclusion for KI#2

Agenda item:	20.1.1

Document for:	Agreement

Work Item / Release: FS_5GSAT_Ph4_ARC /Rel-20

Abstract of the contribution: This paper proposes Interim conclusions for KI#2

# 1. Discussion

As specified in Subclause 7.4.2 of TS 22.261 [1], the Call Setup Time (CST) for voice over GEO links (1–3 kbps) shall be less than 30s. This allowance accounts for the inherent propagation delay of GEO links.

In practice, ITU-T Rec. E.807 [2] identifies 10s as the maximum CST most users will tolerate before perceiving the service as unreliable. This benchmark is critical for the primary GEO voice use case: emergency communications in remote or off-grid areas. In such scenarios, long CSTs are not acceptable. A delayed connection may cause the caller to abandon the attempt, assuming the call has failed, with potentially life-threatening consequences. Therefore, while the standard permits 30s, the practical requirement for GEO voice, especially for emergency calls, is rapid connection, ideally within the 10s QoE target.

Table 1 provides CST results calculated according to Annex A of TR 23.700-19 [3] and Reference [4]. Cases 1-3 are based on text-based SIP signalling over IP with B2BUA deployment. The IMS message sizes follow Sol#12 (four small messages or two large messages, no pre-condition) and Sol#15 (two large messages, no pre-condition). Cases 4-7 are based on binary-encoded signalling in Sol#4 and Sol#20, with IMS message sizes according to Sol#20.

The analysis shows clear advantages of binary-encoded signalling with non-IP delivery and B2BUA:

- At 1 kbps, binary-encoded signalling achieves 9.58s CST, about half of SIP signalling (18.01 s).

- In the 1-3 kbps range, the CST when using binary-encoded signalling remains below 10s, meeting the QoE benchmark in ITU-T Rec. E.807 [2].

- SIP clear-text signalling results in CSTs between 10.62s and 20.84s, which may not be acceptable for emergency call scenarios over GEO.

Table 1: Call Set Up Time (in Seconds) Analysis for Different Bit Rate Values

|  | Case 1: SIP (4 small message, IP, with B2BUA) | Case 2: SIP (2 big messages, IP, with B2BUA) | Case 3: SIP (2 small messages, IP, with B2BUA) | Case 4: Binary-encoded (IP, no B2BUA) | Case 5: Binary-encoded (IP, with B2BUA) | Case 6: Binary-encoded (non-IP, no B2BUA) | Case 7: Binary-encoded (non-IP, with B2BUA) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1kbps | 18.01 | 20.84 | 13.59 | 16.07 | 10.35 | 13.39 | 9.58 |
| 2kbps | 13.86 | 14.99 | 11.36 | 13.89 | 9.74 | 12.55 | 9.36 |
| 3kbps | 12.48 | 13.04 | 10.62 | 13.16 | 9.54 | 12.27 | 9.29 |

These results indicate that binary-encoded signalling offers clear benefits in GEO scenarios. As such, it may be appropriate to consider binary-encoded alternatives alongside SIP clear-text signalling, to ensure users receive the best QoE that can be technically achieved.

Based on above observations, we propose the following high-level principles for KI#2

- UNI interface should be optimized to support voice call over GEO.

- Due to the transmission constraints over satellite link, a binary-encoded optimized IMS protocol shall be used.

# 2. Proposal

It is proposed to include the following changes in TR 23.700-19 V1.0.0.

# 3. Reference

[1] 3GPP TS 22.261: "Service requirements for the 5G system".

[2] ITU-T Rec. E.807: “Definitions, associated measurement methods and guidance targets of user-centric parameters for call handling in cellular mobile voice service”.

[3] 3GPP TR 23.700-19: “Study on Integration of satellite components in the 5G architecture”.

[4] 3GPP SA2 tdoc: available at https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_171_Wuhan_2025-10/INBOX/DRAFTS/Rel-19%2620%205GSAT/Discussion%20on%20GEO%20voice%20signalling.pdf

* * * First Change * * * *

# 7	Interim Agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed principles for KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

To supports KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following principles are concluded:

Principle 1: The existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused to trigger UE-SAT-UE communication, select UPF(s) onboard satellite(s) and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

Principle 2: The following existing 5G VN features apply for KI#5:

-	A single SMF or a single SMF set is supported.

-	The UEs are in the same 5G VN group.

-	Local switch within UPF on-board satellite, N6 and N19 tunnel between UPFs on-board different satellites are supported.

-	Both IP PDU Session type and Ethernet PDU Session type are supported.

-	Only unicast UE-SAT-UE communication is supported.

Principle 3: The SMF determines whether to continue or to fall back the UE-SAT-UE communication due to serving satellite change, based on the information about the availability of ISL(s) between satellites serving the UEs as well as the 5G VN group supporting UE-SAT-UE communication. How SMF gets the information is per implementation, e.g. based on UPF report or from transport network via a proprietary interface.

Principle 4: PSA UPF on ground and UL CL/BP/local PSA UPF onboard satellite is supported as baseline. To support service continuity, the following apply:

-	For the UE-SAT-UE communication continuity case, the SMF reuses existing procedures for Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session as described in clause 4.3.5.7 of TS 23.502 [3] for the UE.

-	For the ground fallback case the SMF coordinates existing procedures at both UEs for Removal of additional PDU Session Anchor and Branching Point or UL CL as described in clause 4.3.5.5 of TS 23.502 [3]. The SMF firstly removes UL CL/L-UPF of peer UE and configure PDR&FAR and N6/N19 routing in ground PSA UPF(s) (if two UEs are using different PSA UPFs) during the satellite handover of local UE, then removes UL CL/L-UPF of local UE after satellite handover of local UE.

Principle 5: For PSA UPF on satellite, SSC mode 2 or 3 can be used to support service continuity. It is assumed that all the 5G VN group members are accessing NTN (no DN connectivity while UE-Sat-UE with PSA UPF).

### 7.1.x	Agreed principles for KI#2 on IMS enhancement for GEO NB-IoT NTN access

The following high-level principles apply for KI#2

- UNI interface should be optimized to support voice call over GEO.

- Due to the transmission constraints over satellite link, a binary-encoded optimized IMS protocol shall be used.

NOTE: the actual protocol selection is left to CT1.

* * * End of Changes * * * *