3GPP TSG-SA WG2#171	S2-2508296

Wuhan, China, 13 – 17 October, 2025	(revison of S2-2506604)

Source:	OPPO

Title:	KI#2, New solution on DO-A procedure

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This paper proposes a new solution for WT#2 on DO-A procedure.

1	Discussion

This paper proposes a basic DO-A procedure initiated by the AIoT Device to send data to the AIoTF for routing to the AF for WT#2.

In the approved SI (SP-250834), the WT#2:

WT#2: Study the support of DO-A Capable AIoT Devices, including:

-	Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).

-	Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.

-	Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF.

NOTE 4:	topology 2 aspect of WT#2 has dependency on WT#1.

This solution has the following assumptions:

- AIoT Device is pre-configured or configured by the network with DO-A routing information.

- AIoT Device sends the DO-A routing information along with the DO-A message for AIOTF to discover the proper AF.

- The NG-RAN can select the AIOTF according to the AIOT information in the temp ID if a temp ID is included in the D2R AS message.

Based on the assumptions above, the following procedures are provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-30 V0.1.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | Key Issue #1 | <Key Issue #2> |
| #1 |  | X |
| #2 |  | X |
| #3 |  | X |
| #4 |  | X |
| #X |  | X |

* * * * Next Change (All text new) * * * *

6.X	Solution #X: DO-A procedure with configured routing information

6.X.0	High-level solution Principles

This solution aims to resolve KI#2 to support DO-A Capable AIoT Devices, the following principles are followed in this solution:

-	This solution has reused the architecture defined in clause 4.2.2 in TS 23.369 [X]. This solution is applied to both direct connectivity and indirect connectivity.

-	The AIoT Device is not required to perform the registration procedure first before sending the DO-A data.

-	AIoT Device sends the DO-A routing information along with the DO-A message for AIOTF to find the proper AF.

6.X.1	Description

This solution resolves KI#2 for support of DO-A Capable AIoT Devices.

This solution has the following assumptions:

1)	AIoT Device is pre-configured or configured by the network with DO-A routing information.

2)	AIoT Device sends the DO-A routing information along with the DO-A message for AIOTF to find the proper AF.

3)	The NG-RAN selects the correlated AIOTF according to AIOTF information in the temp ID if a temp ID is included in the D2R AS message.

6.X.2	Procedures

Figure 6.x.2-1: Procedure for DO-A procedure with configured routing information

0.	DoA routing information (e.g. AF ID or AIOTF allocated DO-A correlation ID) at the AIoT device side or configured by the network via e.g. AIoT Command procedure defined in clause 6.2.2 in TS 23.369 [X]e.

1.	The AIoT Device decides to trigger a DO-A message (e.g. detecting DO-A trigger broadcast signal, etc.).

Editor's note:	DO-A trigger broadcast signal is further defined in RAN WGs.

2.	The AIoT Device sends the AS D2R message (Temp ID, NAS DOA Data Transfer Request message (DO-A data, AIoT device ID, routing information)) to the NG-RAN. AIoT device ID may be a permanent Device ID or Temp Device ID. The temp ID is also provided in the AS D2R message if the AIOTF has allocated a Temp ID to AIoT Device during or after the Command procedure, or during the previous DO-A Data Transfer procedure.

3. The NG-RAN allocates an NGAP ID for the AIoT device and forwards the NAS DO-A Data Transfer Request message in Initial DO-A message to the AIOTF. If a temp ID is included AS D2R message, the NG-RAN can select the AIOTF according to the temp ID. If the temp ID is not included in AS D2R message or the AIOTF derived from the AIOTF in the temp ID can not be selected, the NG-RAN can select an AIOTF which can serve the AIoT Device.

4. Upon receiving the Initial DO-A Message, the AIOTF performs a device check with device profile data retrieved from the ADM to validate the AIoT device.

Editor's note:	Details for device check for DO-A Data Transfer procedure will be determined by SA WG3.

5. After the successful device check, the AIOTF derives the corresponding AF according to the routing information in the NAS DO-A Data Transfer Request message. The AIOTF sends the Naiotf_AIoT_Data Transfer message including the DO-A data and AIoT Device ID to the NEF.

6. The NEF invokes the Nnef_AIoT_Data Transfer service to forward the DO-A data to the AF.

7. The AF sends the Nnef_AIoT_Data Transfer_Response to NEF with AIoT device ID and the response to the DoA data, which may include further data or acknowledgments related to the DO-A Data Transfer process.

8. The NEF invokes Naiotf_AIoT_Data Transfer_Response to AIOTF. The message includes the DO-A data response and related AIoT device ID.

9. The AIOTF allocates an NGAP ID for the AIoT device and sends DL NAS message to NG-RAN with the DO-A data response. In this step, the AIOTF may also allocate a Temp ID to AIoT Device NAS DOA Data Transfer Response message.

10. The NG-RAN sends the AS R2D message to the AIoT device including the NAS message.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

AIoT Device:

-	Support to initiate DO-A message transfer procedure and send routing information along with the DO-A data.

AIOTF:

-	Support to route the DO-A message to proper NEF/AF according to the routing information.

NG-RAN:

-	Support to select the AIOTF based on the temp ID if provided by the AIoT device.

-	Support to establish NGAP connection with AIOTF for DO-A message transfer.

* * * * End of Changes * * * *