SA WG2 Meeting #171	S2-2508317

# Wuhan, China, 13 – 17 October 2025                           (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Interim Conclusions for KI#1

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes some interim conclusions for KI#1.

1	Introduction

This paper proposes some interim conclusions for KI#1.

2	Proposal

It is proposed to update the TR 23.700-30 as followings.

# * * * * Start of change (All Text New) * * * *

8.1	Conclusion on Key Issue #1

The following are interim conclusions for KI#1 on Inventory Procedure.

Figure 8.1-1 describes the Inventory Procedure.

Figure 8.1-1: Inventory procedure

1.	UE performs registration procedure and includes the UE Reader capability in the Registration Request message to AMF. The AMF determines whether the UE is authorized to act as UE Reader based on UE Reader capability and subscription data from UDM, and provides authorization information to UE and NG-RAN.

2.	The AF invokes Nnef_AIoT_Inventory (AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], UE Reader list) service operation request to the NEF.

3.	The NEF may further authorize the AF request. The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request.

4.	The NEF invokes the Naiotf_AIoT_Inventory (AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], UE Reader list) service operation towards to the selected AIOTF(s).

5.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization. The AIOTF generates a Correlation ID and determines assistance information. The AIOTF sends the AIoT Inventory Service Response to the AF via the NEF.

6.	AIOTF performs UE Reader Selection, based on, e.g. Target Area information, AF provided UE Reader list, or stored UE Reader list.

7.	The AIOTF sends Namf_AIoT_MessageDelivery message (NGAP AIoT information, NG-RAN ID, AIOTF ID, Message Type for NGAP AIoT information) to the AMF and further to NG-RAN.

Inventory Request message (NGAP AIoT information) includes the Correlation ID, the AIoT Identification Information to be included in the paging message, assistance information, and UE Reader list.

7a.	AIoT radio resource allocation for UE Reader.

8.	NG-RAN sends DL RRC message (Inventory Request) to UE Reader.

9-10. UE Reader sends Inventory Response to AIOTF via the NG-RAN.

11.	UE Reader(s) will execute the inventory operation. If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT identity.

12.	UE Reader sends an UL RRC message (AIOTF ID, NGAP AIoT information) to NG-RAN. UE Reader sends one or more Inventory Report (NGAP AIoT information) messages to the NG-RAN including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

13. NG-RAN sends an NGAP message (AIOTF ID, NGAP AIoT information) to an AMF and further to AIOTF.

14.	The AIOTF reports the progress of the AIoT inventory request to the NEF, and the NEF informs the AF of the outcome of the AIoT_Inventory request.

# * * * * End of Changes * * * *