3GPP TSG SA WG2 Meeting #171	S2-2508328

Wuhan, China, 13 – 17 October 2025

Source:	TCL

Title:	[WT#3.2, AI agent communication] WT scope and related key issues

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the 6G network for AI working task scope and related key issues.

- Justification

The use case that AI-Agents communicate with each other to complete a complex task is identified in TR 22.870 from SA1. These use cases require the 6G network to support Agent-to-agent communication. In order to support this functionality, 6G network shall standardize APIs for agents to register their attributes, expose their capabilities, and establish connections between agents. Also, there shall be a mechanism for multiple agents to collaborate on a task. The following potential requirement can be seen in TR 22.870:

[PR 6.6.6-3] Based on regulatory requirements, operators’ policy and user consent, 6G network shall support mechanisms for 3rd party AI agents to provide/register their attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) to 6G network, and discover other authorized 3rd party AI agents to achieve collaborative tasks.

[PR 6.8.6-2] The 6G system shall support secure interoperability between AI Agents and between AI Agents and applications to achieve a collaborative task.

The connection between agents deployed on UE and the 6G CN network can be considered as part of the generic UE-CN communication mechanism and will be scoped under WT#1.1. This paper focuses on the discussion for communication between agents outside the 6G CN.

2. Discussion

The 6G CN shall provide the service for AI Agents to register themselves in the network. The network function shall store and manage the attributes of the agents. The centralized agent capability management in 6G CN enables a straightforward mechanism for agents to discover each other based on their capabilities.

Considering the agents deployed on the UE, the mobility management of the agents due to the mobility of UE shall be provided as a function by the 6G CN. 6G CN shall be able to identify the location of a dedicated agent so that any communication with this agent will be possible. The mapping relationship between agent and UE can transform the problem from connecting an agent to connecting a UE, which is a traditional function provided by the network.

The connection between agents may neither be on the control plane nor on the user plane due to the transmission characteristics of the data to be transferred between agents. Compared to the traditional signal or data flow, agents are more likely to transfer tokens between each other. The token communication shows different characteristics from traditional voice or media data transmission. In addition, the purpose of the agent communication is different from the traditional ones. In a traditional communication scenario, data transmission itself is the purpose. For agent communication, the task to be finished behind the communication, such as intent, model update or compressed reasoning state, is the actual purpose. These characteristics raise new requirements and new dimensions on communication quality control.

3. Proposal

It is proposed to include the following changes in TR 23.801.

**** First Change****

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g., AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.2 6G Network for AI:

-   Study whether and how to support the AI agent on UE to register in the network and to discover another AI agent on a different UE.

NOTE 1: Coordination with SA6 on the application layer aspects may be needed.

-   Study whether and how to enable communication for AI agents on different UEs via the 6G network.

NOTE 2: The AI agent(s) on UE is assumed in the UE but not part of the MT, but it can leverage MT functions. It is also assumed that any solution that suggested using UE to Core Network interaction is not replacing NAS protocol.

NOTE 3: The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

- Study whether and how the 6G network (except RAN) can provide AI services(e.g. AI inferencing, model training and model management) to users (via AF or UE).

- Study whether and how to provide overall quality control for the task accomplished by the agent communication or the AI service provided.

NOTE 4: Coordination with SA3 and SA5 may be needed on AI inferencing and AI training.

NOTE 5: Coordination with WT#6 for computing may be needed.

NOTE 6: Charging aspects of AI services may be coordinated with SA5 Charging group.

Study whether and how the UE/AF can provide AI traffic (e.g., AI tokens) characteristics (e.g., error tolerance) to the 6G network so that the 6G network, if needed, (e.g. via enhancing policy control and user plane management) can perform traffic handling and provide adaptive QoS for AI considering the AI traffic characteristics (e.g. AI token) via user plane.

NOTE X: Coordination with SA1 on the AI traffic characteristics may be needed..

- Study whether and how the 6G system can support AI agent enabled third party AFs.

- Study whether and how to enhance network capability exposure functionalities to support AI Agent on UE/AF.

NOTE X: Coordinated with WT#1.2 on policy and QoS control may be needed.

NOTE X: Further update to align with SA1 requirements on support of AI applications and services may be needed.

**** Second Change****

5.X. Key Issue #X: AI Agent Management

For AI-agent deployed on the UE, 6G CN shall have a mechanism for capability registration, agent discover, mobility management and connection establish. The following issues shall be solved:

- Where and how in the 6G CN to store the capability information, deployment information of the AI agents?

- How to expose the capability of the AI agents to enable the potential users to discover them?

- How to do the agent mobility management together with the UE mobility management?

5.X. Key Issue #X: Session Management for AI Agents

The purpose of the communication between AI-agents is different from the traditional communication scenario. The task to be completed by the agent communication shall be also considered and managed by 6G CN. The task quality control can be generally split into communication quality control and computing quality control. 6G CN shall take both into consideration and provide mechanism for overall task quality control. Following issues shall be studied:

- Define the procedures to establish connections and route messages for agents

- How to identify and locate the agents involved in the communication in the 6G CN?

NOTE: The security and authentication issue for the agents may be studied in SA3.

NOTE: The integrity check problem for the task assigned to the agents need to be studied in SA3

- How to keep task level session continuity during mobility for multiple agents? How to synchronize task state between agents?

- Define the tool for quality control of the task accomplished via the communication done by the connectiosns

- How to control the connection quality to fulfil the high-level AI task quality control? Which network function or element shall be responsible for such control?

- How to design the parameters for the QoS of these new connections on both communication and computing aspect?

- How to dynamically configure the QoS parameter on both aspects to finally fulfil the task level overall quality control during task performing?

**** End of Change ****