3GPP TSG-SA2 Meeting #171	S2-2508364

Wuhan, CN, Oct 13 – 17, 2025                                          (was)

Source:	Nokia

Title:	Proposed agreement of principles for Key Issues 5

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#5

# 1.	Introduction

This document discusses aspects related to several KIs and proposes principles for conclusions by considering the scope, architectural assumptions and the solutions

## 1.1 Discussion for KI#5:

Exposure of sensing results to the SC(AF) is provided by the SF/NEF based on the communication path the sensing request has reached the core network (with or without NEF) from the AF. Using the existing mechanisms of exposure framework, the core network provides the sensing results to the AF.

Proposal for KI#5:

Following content for KI#5 is proposed to be captured as principles for agreement.

- SF provides the sensing results to the NEF and then NEF using the exposure framework provides the sensing results to the AF.

- The trigger and preparation of sensing results are based on the agreed type of a sensing service request received from the AF, i.e., one time, evet-based or subscription based.

- Sensing request from the SC includes sensing service type, sensing target area, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS), sensing results configuration (one-time, event-based, subscription-based).

Note: The parameters that can be considered between the SC and the SF will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.5.1	Agreed Principles for KI#5

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#5:

- SF provides the sensing results to the NEF and then NEF using the exposure framework provides the sensing results to the AF.

- The trigger and preparation of sensing results are based on the agreed type of a sensing service request received from the AF, i.e., one time, evet-based or subscription based.

- Sensing request from the SC includes sensing service type, sensing target area, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS), sensing results configuration (one-time, event-based, subscription-based).

Note: The parameters that can be considered between the SC and the SF will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

# * * * * End of change * * * *