3GPP TSG-SA2 Meeting #171	S2-2508470

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2506696)

Source:	Huawei, HiSilicon

Title:	KI#3, Sol #16: Update to include the selection criteria

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Update solution #16 for KI3 including the clarification of sensing entities, removal of EN and addition of the selection criteria details.

1. Introduction

This pCR proposes the update for KI3 Sensing Entity Discovery and Selection, specifically for the following aspects:

-	Clarify that the sensing entities should be gNB as per the determined scope of Rel-20;

-	Remove the EN;

-	Add the details regarding gNB selection criteria;

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6.16	Solution #16: Sensing entity discovery and selection assuming Sensing Entities are registered with the Sensing Function

6.16.0	High-level solution principles

The gNB discovery and selection functionality is supported by the Sensing Function (SF). For each Sensing Service the SF selects a gNB to handle Sensing Service.

NOTE 1:	Whether SF is divided into Sensing Control Function and Sensing Processing Function will be clarified in Clause 8.

For a Sensing Service request the SF performs the following functionality for gNBdiscovery and selection:

- SF may have obtained gNB information based on gNB association procedure, which provides gNB ID, sensing capability information and the Sensing Service Area.

-	When receiving a Sensing Service request, the SF may perform an independent discovery procedure to multiple gNBs to discover the best suitable gNB for the specific Sensing Service.

NOTE 2:	The SF can determine to perform gNB discovery procedure based on the Sensing Service characteristics. For example, if the Sensing Service is applied for long time or when the sensing object is rather static.

-	The SF performs selection of the appropriate gNB(s) for a specific Sensing Service by applying one of the following options:

-	Option 1: the SF may select a primary gNB based on the Target Sensing Service Area and delegates further decisions to the primary gNB.

-	Option 2: the SF may select one or more gNB to act as Sensing Entities based on the Target Sensing Service Area.

NOTE 3: No inter-gNB coordination will be studied.

-	If SF selects multiple gNBs, the SF processes the 3GPP Sensing Data received from multiple gNBs to generate Sensing Result, such as the trajectory information of Sensing object.

-	The SF performs re-selection of the appropriate target gNB(s) as Sensing Entities based on the supported Sensing Area of the gNB(s) and Sensing Result (location information) of the sensing object.

The gNB performs the following functionality:

-	It registers with a SF and provides the sensing capabilities and radio serving area to the SF.

NOTE 4: The Sensing capabilities and the radio serving area of the gNB may be pre-configured to the SF via OAM.

-	In case of Option 1, the gNB may serve as primary gNB to be the anchor point for configuring TRP and to collecting the sensing data from one or multiple gNB(s). The gNB receives a Sensing Service request.

NOTE 5:	The purpose of the primary gNB is to hide the sensing radio topology from the CN and may distribute the sensing measurement to one or multiple Transmission Reception Points (TRPs).

-	In case of Option 1 and 2, the gNB may receive from the SF the Sensing Service request indicating whether to act as Sensing Transmitter and/or Sensing Receiver and the  configuration information. In case of Option 1 the primary gNB may select different TRP based on the requested role.

-	In case of Option 1 and 2, the gNB prepare the sensing data for transmission to the CN. In case of option 1 It collects the reports from one or multiple Sensing Receivers and send to the SF.

6.16.1	Description

For the purposes of the present solution the following definition applies:

Sensing Service Area: a logical concept of an area where the gNB can obtain the 3GPP Sensing Data without interacting with other Sensing Entities.

Sensing Service type: indicating the Sensing scenarios and it includes Object Detection within a Target Sensing Service Area, Border Crossing Detection, Path Monitoring and Path Tracking.

This solution addresses key issue#3 on gNB and Sensing Function Discovery and (Re-)Selection.

In addition to the principles described in the High-level solution principles, this solution assumes that the SF can take into consideration the specific Sensing Service type and requested KPIs when selecting whether to apply Option 1 or Option 2 for the SE selection. For example, if the sensing object is static and large enough, the SF may apply Option 2 and exactgNB. In another example, when the sensing object and target area are small, the SF may not be abler to select the exact frequency band because the SF may not know the exact radio conditions (e.g. interference) and the SF may apply Option 1 and select a primary gNB which further selects the TRP(s).

6.16.2	Procedures

The Figure 6.16.2-1 shows the signalling flow for the gNBdiscovery and selection procedure.

It is shown that only gNB A establishes an association with the SF, but it is also assumed that TRP B and C has also established an association in advance.

Figure 6.16.2-1: gNB discovery and selection procedure

1.	The gNB may select of one or more SFs where to register its capability and to establish an association (e.g. transport network layer association, TNLA) should be established. The gNB may use one of the following methods for SF selection:

a)	The OAM system may (pre-)configure the gNB with a list of SF IDs to which the association needs to be established.

b)	In case of Indirect Connectivity when the gNB communicates with the SF via the AMF, the gNB may either (1) use an existing N2 transport association with an AMF or (2) select a new AMF supporting Sensing Service communication to which a new N2 transport association is established.

2.	The gNB may send gNB association request to the SF, which includes SE ID, sensing capability information, supported  Sensing Service area, supported Sensing Service type.

NOTE 1:	The full details list of capabilities is FFS and will be determined in coordination with RAN WG3 in the normative phase.

3.	Void.

4.	The SF sends gNB association response to the gNB..

NOTE 2:	As an alternative, the associating capability(ies) of gNB(s) can be (pre-)configured by OAM, and step 1-4 may be performed only when needs, for example when the gNB sensing capabilities are changed.

5.	The SF receives a Sensing Service request message from the from NEF/AF, e.g. using the Nsf_SensingService_Request service. The Sensing Service request message includes, target sensing area and Sensing Service KPIs.

6a.	The SF may perform a discovery procedure for appropriate gNB for the Sensing Service request. Considering the Sensing Service characterstics received in step 5, the SF selects the candidate gNB(s) which is(are) considered as most appropriate to perform the Sensing Service taken into account the gNB capabilities previously received in step from 1 to 4 or known as configured via OAM. The SF sends a discovery request message to multiple gNB(s) inlcuding Sensing Service ID, sensing area, requested KPIs.

6b.	If 6a is received,  the gNB(s) send a response mesage including the Sensing Service ID, the capabilities if needed,  and an indication of  capability and confidence level for performing the requested Sensing Service as per discovery request message.

NOTE 3:	Whether step 6 is needed will be clarified in Clause 8.

7.	Considering the Sensing Service characterstics received in step 5 and the optional information provided in step 6 (if any), the SF selects one or more gNB which are considered the appropriate to perform theSensing Service.

8.	The SF sends a Sensing request message to the selected gNB(s) inlcudng the  Sensing Service ID, sensing area, sensing KPIs, target reporting information (e.g. CP or UP reporting and the target address) for the sensing data.

6.16.3	Impacts on Services, Entities and Interfaces

Impacts to the Sensing Function:

-	perform a discovery procedure to multiple sensing entities to discover the best suitable entity for the specific Sensing Service, e.g. the gNB can provide to the SF a confidence level for the specific Sensing Service.

-	select one or more appropriate gNB(s) for a specific Sensing Service as follows:

-	Option 1: select a primary gNB and delegates further sensing decisions to the primarygNB.

-	Option 2: select one or more gNB(s) to act as Sensing Entities.

Impacts to thegNB:

-	establishing association with the SF and providing the sensing capabilities and radio serving area.

-	In Option 1: if capable to serve as primary gNB, receiving a  Sensing Service request and selecting the type of sensing mode and which are the most appropriate sensing transmitter and sensing receiver.

-	In Option 2: receiving from the SF the Sensing Service request indicating to act as Sensing Entity, configuration information.

-	collecting and preparing the sensing data for transmission to the SF.

Impacts to the AMF:

-	In case of Indirect Connectivity when the gNB communicates with the SF via the AMF, the AMF supports forwarding of signalling between the gNB and SF.

# * * * * End of changes * * * *