3GPP TSG-SA WG2 Meeting #171	S2-2508530

# October 13-17, 2025 Wuhan, China

Source:	Skylo Technologies

Title:	New Sol to KI#1

Agenda item:	20.8.1

Document for:	Agreement

Work Item / Release: FS_SMS2EC_ARC/Rel-20

Abstract of the contribution: This paper propose a solution to KI#1

# 1. Discussion

KI#1 aims to study solutions to:

-	identify whether an SMS should be routed to a PSAP;

-	identify the type of emergency service (police, ambulance, fire brigade, etc.) required;

-	address both the case of the UE detecting and the UE not detecting an SMS that is to be routed to Emergency Response Centre;

-	consider the scenario where the serving PLMN does not support routing an SMS to an Emergency Response Centre.

The proposed solution addresses the above aspects.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-65 V0.1.0.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X | X |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

## 6.X		Solution #X: Support of Emergency SMS indication and header for SMS to PSAP

### 6.X.0			High-level solution Principles

The following high-level principles are proposed for the support of SMS to Emergency Centre service:

- An Emergency Header is introduced within the SMS-SUBMIT TPDU, specifically within the TP-UDH. The Emergency Header may include emergency-related information such as the requested emergency service type, callback identifiers, or other parameters required for accurate routing to a PSAP.

- When the UE determines that an SMS is an emergency message, the UE composes the SMS-SUBMIT TPDU including the Emergency Header and submits it using a Service Centre (SC) address configured for emergency SMS handling. The SC address for emergency SMS may be pre-provisioned in the UE or UICC or may be obtained from the network. The SC address is carried in the SMS-SUBMIT RPDU (Relay Protocol Data Unit).

- The UE may also indicate towards the network that the MO data transport contains an emergency SMS.

- The serving network forwards the SMS to the SMSC. The SMSC applies emergency-specific processing and delivers the message to the appropriate PSAP, supporting use cases such as Text-to-9-1-1 [x].

- For cases where the UE does not detect that the SMS is an emergency SMS, the message is composed as a normal SMS without an Emergency Header. The serving network then inspects the Destination Address (TP-DA) of the SMS. If the TP-DA matches an entry in the network’s provisioned emergency-number list, the SMS is classified as an emergency SMS. The network shall then internally flag the SMS as an emergency message and ensure that it is routed to the correct PSAP in accordance with applicable regulatory requirements.

### 6.X.1			Description

#### 6.X.1.1	UE Detected and Non UE Detectable Emergency SMS

To support reliable detection and routing of emergency SMS, the UE may include an emergency header within the TP-UDH of the SMS-SUBMIT TPDU. The emergency header may carry critical information such as a callback number, emergency service type, and location information. Provision of such information at the point of message origination enables the network to more reliably determine whether a MO SMS requires emergency handling, and to ensure accurate routing to the appropriate Public Safety Answering Point (PSAP). This mechanism reduces the risk of misrouted or delayed emergency messages and thereby improves user safety.

The Destination Address TP-DA shall correspond to an emergency short code (e.g. 112, 911), and TP-UDHI shall be set to “1” by the UE. The emergency header in TP-UDH may contain:

- indication that the TP-UD contains an emergency message;

- callback number, if available;

- location information, if available;

- emergency service type (e.g. police, ambulance, fire);

- callback token (e.g. ESQK), if available;

- UE location information, e.g. civic address, if available.

The serving network may download additional emergency numbers to the UE to enable the UE to detect an emergency SMS.

In the case of non-UE detectable emergency SMS, the message will be handled as a normal SMS. The network shall inspect the TP-DA of the SMS-SUBMIT TPDU. If the TP-DA matches an entry in the provisioned emergency-number list, the message shall be flagged as emergency traffic and routed to the appropriate PSAP in accordance with regulatory requirements.

The network may also allocate or substitute a callback identifier (e.g., ESQK) in the TP-OA to support reliable PSAP-to-UE callbacks while preserving user privacy and ensuring regulatory compliance.

The SMSC or Emergency SMS Router shall inspect the SMS-SUBMIT TPDU. If an emergency header is present in the TP-UDH, the network shall extract the included information (e.g. callback number, emergency service type, and location information) and classify the message as an emergency SMS. The network may query the location retrieval functionalities in the network to obtain authoritative location information, apply local routing policies, and ensure that the message is delivered to the correct PSAP in the serving country or region.

### 6.X.2			Procedures

#### 6.X.2.1 	Roaming Handling

For emergency messages initiated by a roaming user, the principle of Local Breakout is maintained, prioritizing service from the VPLMN based on local regulations. If the UE does not know its own location, the UE may retrieve its location information from the VPLMN, e.g. LRF lookup and the VPLMN routes the message directly to the local PSAP. The emergency header in the message is used to choose the correct service-type PSAP.

If the VPLMN lacks the capability or regulatory approval to route the message, it forwards the emergency traffic (including the TPDU, ULI, and UE identity/SUPI) to the HPLMN emergency gateway. The HPLMN allocates the ESQK, routes the message to its PSAP, and preserves the mapping for subsequent callbacks.

#### 6.X.2.2			Callback Handling

When the PSAP replies, it sends the message back to the TP-OA (MSISDN or ESQK). If the address is the ESQK, the network maps the ESQK back to the serving node (MME/AMF) and delivers the MT SMS via the SMS-GMSC/SMSF to the UE using standard MT SMS over NAS procedures. The presence of the callback token in the original message's emergency header allows the PSAP to correlate the message session with any subsequent voice sessions.

### 6.X.3	Impacts to Services, Entities and Interfaces

UE:

- Support the inclusion of an emergency header in the TP-UDH of SMS-SUBMIT when originating emergency SMS

- Indicate towards the network that the MO data transport contains an emergency SMS Network:

SMSC/SMS Router/SMS-IWMSC/SMS-GMSC

- Inspect SMS-SUBMIT TPDU for presence of emergency header in TP-UDH

* * * Second Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[7]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[8]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[9]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[x]		ATIS J-STD-110: “Joint ATIS/TIA Implementation Guideline for J-STD-110, Joint ATIS/TIA 	Native SMS/MMS Text to 9-1-1 Requirements and Architecture Specification”.

* * * End of Changes * * * *