3GPP TSG-WG SA2 Meeting #171	S2-2508569

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion of KI#2

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: It proposes to add conclusion of KI#2.

1. Introduction/Discussion

There are following topics for further consideration for KI#2:

-	Whether the UPF can be the consumer of the user plane traffic pattern and behaviour analysis or not.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

Bullet 1:

Only in Use Case 1, there are solutions mentioning UPF as the consumer. However, there are still ENs in related solutions. It is proposed to solve this after solving the ENs.

Proposal 1: It is proposed to limit Bullet 1 to Use Case 1 and solve it after solving related ENs.

Bullet 2:

This depends on the detailed input & output discussion. As also discussed in S2-2506808, the purpose of Use Case 1 is to know whether traffic in the user plane is abnormal or not, optionally, together with the abnormal type of the traffic (so called Exception ID in some solutions). Thus the input should contain The normal behaviour of the traffic (e.g. including traffic pattern, fingerprint or possible UEs from AF in some solutions) and the exact behaviour of the traffic in the network (so called traffic info in some solutions). the exact behaviour of the traffic in the network requires SMF/UPF Event to report to NWDAF. Other than those are already supported by QoS Monitoring (i.e., the packet delay of the traffic, the data rate of the traffic, and the congestion information of the traffic), all required new information needs to define new SMF/UPF Events. For Use Case 1, the buffered time of the traffic, the data burst of the traffic, and SSL/TLS fingerprint (for encrypted traffic) can be useful.

For Use Case 2, it is already agreed the output should be traffic pattern (i.e. Maximum Burst Size, Periodicity, (Requested) 5GS Delay, (Requested) Maximum Bitrate, (Requested) Guaranteed Bitrate and (Requested) Packet Error Rate), which has already been defined in TS 23.501 and TS 23.503. So the input should be the exact behaviour of the traffic from the SMF/UPF which is related to traffic pattern for the requested service. The packet delay of the traffic, the data rate of the traffic, and the congestion information of the traffic have already been supported by the current UPF Event (QoS Monitoring as described in TS 23.501 [2]). But the Periodicity related information (i.e. Packet Interval) requires new UPF event.

Proposal 2: It is proposed to capture above inputs & outputs and remove Bullet 2.

Bullet 3:

It has already been agreed that to reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information for input data filtering to UPF. Combining notification to NWDAF at UPF for multiple events is an optimization in Stage 3. Instructions from NWDAF to UPF for pre-processing in the UPF requires NWDAF to generate local data pre-processing instructions. New N4 rules should be defined, which is complicated. Thus, it is proposed to not have too many per packet level reporting (e.g. duplicated packet) but only to report a static value for a time period. Then, there is no need to introduce such mechanism.

Proposal 3: It is proposed to remove Bullet 3 directly.

Bullet 4:

For Use Case 1, the consumer can be SMF , AF, OAM and PCF. When the PCF is the consumer, it may update the PCC rules, e.g. gate control for abnormal traffic. When the SMF is the consumer, the SMF can still modify N4 rules, but it should not disobey what PCF instructs. When the AF is the consumer, it may correct the abnormal traffic flow. But this is out of scope of this specification. Similar as OAM. So it is proposed to remove AF and OAM in the agreed principles but they are still permitted from standard perspective.

For Use Case 2, it is agreed the consumer is PCF. The PCF can use the output of NWDAF as AF Request.

Proposal 4: It is proposed to capture above consumer actions and remove Bullet 4.

Base on the above discussion, the input & output and the consumer action of the 2 use cases are quite different and not covered by current Analytics. It is proposed to have 2 new separated Analytics.

Proposal 5: It is proposed to have 2 new separated Analytics for 2 Use Cases.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change * * * *

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information for input data filtering to UPF.

7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, and PCF. The consumer may take the output Analytics into account. The analytics consumer may include the Analytics Filter Information such as DNN and S-NSSAI in the request.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics. The output contains:

-  Whether traffic in the user plane is abnormal or not; optionally together with the abnormal type of the traffic, including DDoS, abnormal data rate, and/or abnormal traffic bursts, etc.

-  The affected traffic (if available), including affected Application address distribution (e.g., Application ID, IP-3-tuple, IP-5-tuple), affected PDU Session ID and/or affected UPF ID.

-	To derive the output analytics, the following input data may be collected to support the NWDAF-based analytics:

-	The Application ID and/or Packet Filter Set, identified the service data flow(s) of the target application(s);

-  The normal behavior of the traffic from the AF, including traffic pattern, SSL/TLS fingerprint, the possible  UE address of the traffic.

-  The exact behavior of the traffic from the SMF/UPF, including the packet delay of the traffic, the data rate of the traffic, the congestion information of the traffic, the buffered time of the traffic, the data burst of the traffic, SSL/TLS fingerprint, number of duplicated packets.

NOTE: the packet delay of the traffic, the data rate of the traffic, and the congestion information of the traffic have already been supported by the current UPF Event (QoS Monitoring as described in TS 23.501 [2]). Other information requires new UPF event.

-  The consumer actions may take the output Analytics into account.

-  When the PCF is the consumer, it may update the PCC rules.

-  When the SMF is the consumer, it may update the N4 rules or reselect the UPF for a PDU Session. But it  shall not generate or update a N4 rule conflicting with the parameters provided by the PCF (e.g. QoS and charging related parameters in PCC rule).

7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF. The analytics consumer may include the Analytics Filter Information such as application description (e.g., SDF Template), UE Address, DNN and S-NSSAI in the request.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics on traffic pattern information. The output contains:

-  The Application ID and/or Packet Filter Set, identified the service data flow(s) of the target application(s);

-  The traffic pattern information defined in TS 23.501 [2] and TS 23.503 [4], i.e., Maximum Burst Size, Periodicity, (Requested) 5GS Delay, (Requested) Maximum Bitrate, (Requested) Guaranteed Bitrate and (Requested) Packet Error Rate.

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow of the collected QoS monitoring information.

-  The exact behavior of the traffic from the SMF/UPF, including the packet delay of the traffic, the data rate of the traffic, the congestion information of the traffic, N6 delay and packet interval.

NOTE: the packet delay of the traffic, the data rate of the traffic, N6 delay and the congestion information of the traffic have already been supported by the current UPF Event (QoS Monitoring as described in TS 23.501 [2]). Other information requires new UPF event.

-	The consumer actions may take the output Analytics into account, e.g. generate or update PCC rules. The PCF can act as described in clause 6.1.3.22 of TS 23.503 [4], i.e., the output of NWDAF can be considered as AF session with required QoS.

# * * * * Second change * * * *

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	Whether and how the UPF can be the consumer of the user plane traffic pattern and behaviour analysis for Use Case 1 or not.

# * * * * Third change * * * *

8	Conclusions

8.2	Conclusion for KI#2

The Agreed Principles for KI#2 as described in clause 7.1.2 apply in the conclusion. In particular:

-  A new Analytic ID is introduced for Use Case 1. The details can be seen in clause 7.1.2.2.

-  A new Analytic ID is introduced for Use Case 2. The details can be seen in clause 7.1.2.3.

# * * * * End of changes * * * *