3GPP TSG-WG SA2 Meeting #171	S2-2508676

# Wuhan, China, 13 - 17 October 2025										(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6442 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LP-WUSPS capability handling updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core, TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on LS S2-2508182/R2-2506261, RAN2 expecting NAS provides a mechanism to enable/disable the entire LP-WUS function. It is proposed to add a disabling of the entire LP-WUSPS handling logic. The existing CN based subgroup support is applicable when there is no disabling of the LP-WUSPS feature (either due no support of the new handling logic or the new handling logic allows the use the LP-WUSPS). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text to reflection the disable handling for LP-WUSPS function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to disable RAN based subgroup handling for LP-WUSPS function. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.12a.1, 5.4.12a.2, 5.4.12a.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.4.12a	LP-WUS with Paging Subgrouping Assistance

#### 5.4.12a.1	General

The RAN and UE may use Low Power Wake UP Signal with Paging Subgrouping (LP-WUSPS) to reduce the power consumption of the UE in RRC_IDLE and RRC_INACTIVE over NR (see TS 38.304 [50]).

Editor's note:	The terminology of "LP-WUS" and "LP-WUSPS" will be further aligned with the terminology used in specifications at RAN Side.

The LP-WUSPS can be based on a LP-WUS Subgroup ID allocated by the AMF, or is determined by the RAN on its own as defined in TS 38.304 [50] and TS 38.331 [28] if the LP-WUSPS is not disabled as specified in clause 5.4.12a.3.

The AMF, when determining its paging strategy (see clause 5.4.3), should take into consideration whether a gNB is using LP-WUSPS determined by the RAN.

If paging subgroups for LP-WUS are being allocated by the AMF, then all AMFs connected to one gNB (including the AMFs in different PLMNs using 5G MOCN network sharing) shall use a consistent policy in allocating UEs to the paging subgroups. The AMF may configure up to 32 different LP-WUS Subgroup IDs.

NOTE:	Because the UE uses the AMF allocated LP-WUSPS across all cells in its TAI-list, and, different, overlapping TAI lists can be allocated to different UEs, then to avoid UE power consumption increasing, it is likely to be necessary that all AMFs with an overlapping coverage area use a consistent policy in allocating UEs to the LP-WUSPS.

The NG-RAN node and the UE determine per cell which LP-WUSPS method to use as defined in TS 38.304 [50] and TS 38.331 [28].

#### 5.4.12a.2	Core Network Assistance for LP-WUSPS

To support the LP-WUSPS, the LP-WUS Paging Subgrouping Support Indication and the LP-WUSPS Assistance Information are used by the AMF and NG-RAN to help determine whether LP-WUSPS applies to the UE and which paging subgroup is used when paging the UE (see TS 38.300 [27]).

NOTE 1:	This CN based subgroup handling is applicable when there is no disabling of the LP-WUSPS function specified in clause 5.4.12a.3 (e.g. either due to no support of the disabling of the LP-WUSPS function or the LP-WUSPS function is not disabled as the result of the handling in 5.4.12a.3).

In the Registration Request message, the UE includes the LP-WUS Paging Subgrouping Support Indication if the UE supports AMF LP-WUSPS Assistance Information for LP-WUS Paging Subgrouping. If the UE includes the LP-WUS Paging Subgrouping Support Indication, the UE may also include the paging probability information to assist the AMF. An AMF supporting LP-WUSPS stores the UE provided LP-WUS Paging Subgrouping Support Indication and the AMF determined AMF LP-WUSPS Assistance Information in the UE context. The AMF may use local configuration, the UE's paging probability information if provided, information provided by the RAN (e.g. any of the Information On Recommended Cells And RAN nodes For Paging), and/or previous statistical information for the UE to determine AMF LP-WUSPS Assistance Information. The AMF LP-WUSPS Assistance Information includes the Paging Subgroup ID.

NOTE 2:	To minimise MT voice call setup latency, the AMF could allocate Paging Subgroup IDs for LP-WUS taking into account whether or not the UE is likely to receive IMS voice over PS session calls.

NOTE 3:	To avoid MT traffic for more mobile UEs causing more stationary UEs to be woken up, the AMF could allocate Paging Subgroup IDs for LP-WUS taking into account the UE's mobility pattern.

If the AMF has determined AMF LP-WUSPS Assistance Information for the UE, the AMF stores it in the UE context in AMF and provides it to the UE in every Registration Accept message.

If the AMF has determined AMF LP-WUSPS Assistance Information, the AMF shall provide it to NG RAN when paging the UE. In addition, in order to support LP-WUSPS for UEs in RRC_INACTIVE mode, the AMF shall provide the AMF LP-WUSPS Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

When the NG-RAN is using AMF allocated subgroups for LP-WUSPS, both the UE and NG-RAN use the AMF LP-WUSPS Assistance Information to determine the paging subgroup to apply as defined in TS 38.300 [27].

The AMF may use the UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]) and N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to update the AMF LP-WUSPS Assistance Information in the UE and NG-RAN.

NOTE 4:	If the serving AMF does not provide AMF LP-WUSPS Assistance Information, then the CN based subgroup for LP-WUSPS is not applied.

#### 5.4.12a.3	Support on the disabling of LP-WUSPS

To support the disabling of the entire LP-WUSPS function (including both subgroup ID determined by AMF and by RAN), the UE should provide the LP-WUSPS Disabling capability in Registration Requestion message which is used by the AMF and NG-RAN to determine whether LP-WUSPS applies to the UE (see TS 38.300 [27]).

In the Registration Request message, the UE should provide the LP-WUSPS Disabling capability. An AMF supporting LP-WUSPS stores the UE provided LP-WUSPS Disabling capability and the AMF determined usage of LP-WUSPS in the UE context.

If the AMF has determined that the LP-WUSPS is disabled for the UE, the AMF provides the indication to the UE in Registration Accept message. The AMF shall provide indication to NG-RAN when paging the UE. In addition, in order to support LP-WUSPS handling for UEs in RRC_INACTIVE mode, the AMF shall provide the indication to NG-RAN as part of the RRC Inactive Assistance Information.

The AMF may use the UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]) and N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to update disabling of the LP-WUSPS.

NOTE:	If the serving AMF does not provide the indication of disabling of LP-WUSPS, then the subgroup for LP-WUSPS is considered supported (based on subgroup ID determined either by AMF or by RAN) for a LP-WUSPS supporting UE.

# * * * End of changes * * * *