SA WG2 Meeting #171	S2-2508703

13 – 17 October, 2025, Wuhan , China

Source:	InterDigital Inc., Philips

Title:	[WT#1.2.x, QoS] QoS Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content on WT 1.2.x QoS Framework.

# Discussion

At SA2 #170, S2-2508122 on the QoS Framework was postponed. This paper proposes two changes to the proposal from SA2 #170.

Proposal #1: Support of QoS Control based on dynamic conditions

The proposal from SA2 #170 included “Whether and how to enhance QoS control mechanisms to…. to improve QoS for applications with dynamic requirements”.

The 5G system relies on static service classification rules. For example, the QoS Rules provide the UE with a static set of rules for mapping specific traffic to specific QoS Rules. This approach can be inefficient in terms of resource utilization and is not well-suited for dynamic service environments. For example, it can result in traffic getting mapped to QoS Flows that provide treatment that is better than what is required or vice versa. It also results in a QoS configuration that does not consider the tradeoffs between energy efficiency, QoE, and radio resource utilization. On the other hand, QoS Rules can be updated, but this not a fast process (e.g. NAS signaling and/or the AF invoking PCF APIs). There is no mechanism in place that allows QoE measurements, energy efficiency, radio resource utilization, and local UE conditions (e.g. speed) to influence, in real time, the QoS treatment that is received by the UE’s traffic flows. Real time QoE measurements, radio resource utilization information, and/or energy efficiency considerations could be used to influence how traffic is mapped to QoS flows so that the application and network can better adapt network resource utilization based on the needs of the application.

Thus, it is not only requirements that might be dynamic, but metrics such as QoE, radio resource utilization information, and energy efficiency are also dynamic.

Thus, this paper proposes the following addition:

Whether and how to enhance QoS control mechanisms to…. to consider dynamic conditions (e.g. QoE, radio resource utilization, and energy efficiency).

Proposal #2: Support of QoS Control based on user identity

Requirements for identifying UE applications and users have existed since Rel-16 (e.g. to TS 22.101 and TS 22.115 as part of the UIA (SP-180328) work item). In Rel-19, SA2 studied how to add support for human user identities (TR 23.700-32 as part of the FS_UIA_ARC (SP-231804) study). SA2 was not able to conclude on how to best add support for identifying the user of a UE. One challenge that companies could not agree on was how to best add support for user identities in the Rel-19 5G System in a backward compatible way. This 6G study presents an opportunity because SA2 can consider supporting an identity service from the first release of 6G.

By allowing for the creation and utilization of identities that identify a user of a UE or an application instance that runs on the UE, operators will be able to provide enhanced user experience, optimized performance, and offer services to users and application providers. For example, network settings can be adapted, and services can be offered to application providers and users according to their needs, which may be different from the default needs of the UE subscription that was used to establish the connection. These identities will allow the operator to charge and provide service differentiation based on the identifier.

Thus, this paper proposes the following addition:

User specific QoS adaptation based on user specific identity.

**** First Change ****

# Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs.

NOTE 1:	The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms

  - to better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network.

  - to improve QoS for applications with dynamic requirements, and

  - to consider dynamic conditions (e.g. QoE, radio resource utilization, and energy efficiency).

- Whether and what to monitor in relation to QoS Framework.

- Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

  - UE provided information to the network for QoS collaboration

  - Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

  - User specific QoS adaptation based on user specific identity.

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

- Study how to ensure interworking with 5GS.

NOTE 3: 	The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: 	Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: 	Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs.

NOTE 6: 	This WT will coordinate with SA WG4 and RAN WGs.

**** End of Changes ****