3GPP TSG-WG SA2 Meeting #171	S2-2508743

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	WT#3.2, WT and KI for NET4AI

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: WT#3.2 scope has been discussed in SA2#170 meeting and S2-2508131 is the final version used for further discussion. This contribution would like to further slightly update WT#3.2 and provide the KIs accordingly.

1. Introduction/Discussion

1.1	Summary of Proposal

This proposal would like to take the version of S2-2508131 as baseline and further slightly update the WT and proposes the KI accordingly.

For WT#3.2 scope, the updated parts are summarized below:

- Update NOTE 5 to mention the coordination with WT#5 for data framework.

- Remove “Further discussion part:” to make the last bullet as part of WT#3.2 and add AI tokens and error tolerance as the example of traffic and characteristics respectively.

- Editorial update

For KI part, following are the additions compared with the proposed WT#3.2 scope:

- Regarding AI agent on UE aspects, the KIs related with the discovery, authorization, identity and profile management of AI Agent on UE and the KIs related with dynamic group communication are proposed.

- Regarding AI inference as service, the KIs related with the optimization to coordination between data transmission and AI inference processing are proposed.

- Regarding AI training as service, the KIs related with how to generate and expose training data or how to train the provisioned AI model and expose the AI model are proposed.

- Regarding QoS handling based on AI traffic characteristics aspect, add AI tokens and error tolerance as an example of AI traffic and the characteristics respectively. And the “via user plane” is removed to avoid confusion.

Clause 1.2 provides justification to study the above aspects.

1.2	Justification on Aspects Studied on 6G Network for AI

AI is becoming more and more widely used. It is worthwhile to consider how 6G network can enable various AI applications and guarantee the required performance to improve the user experience. The 6G Network for AI can be generally categorized into following four aspects:

-	Support of AI agent on UE

-	6G network provides AI inference as a service

-	6G network provides AI training as a service

-	6G network performs traffic handling and provides QoS considering the AI traffic characteristics

1.2.1	Support of AI Agent on UE

Current WT text focuses on AI agent on UE discovery and the communication enabled for AI agents on different UEs.

As discussed in S2-2507155, in order to enable the discovery and group communication of AI agent on UE via 6G network, the 6G network shall be visible to AI agent on UE. This involves following steps and aspects:

Firstly, how to identify and authorize each AI agent on UE by 6G network. Regarding whether to extend based on existing identification mechanism or introduce new type of identity according to the characteristics dedicated to AI agent on UE, this can be discussed and studied during study phase.

In addition, how to enable the dynamic group communication via 6G network shall also be studied since the group communication for AI agent on UE is usually triggered by task collaboration which is dynamic and on demand. Therefore, existing DNN/S-NSSAI based group communication may not be applicable.

Additionally, following are some SA1 requirements related with communication for AI agent on UE in TR 22.870 which are also aligned with the above-mentioned study aspects:

| AI agent Identity related SA1 PRs | [PR 6.7.6-2] Based on regulatory requirements, user consent, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support security identification for 3rd party AI agents provided by authorized 3rd party associated with a user (e.g. AI agents belonging to a customer) |
| --- | --- |
|  | PR 6.7.6-3] Based on regulatory requirements, operators’ policy and user consent, 6G network shall support mechanisms for 3rd party AI agents to provide/register their attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) to 6G network, and discover other authorized 3rd party AI agents to achieve collaborative task. |
|  | NOTE: The 3rd party AI agent refers to the 3rd party AI agent application on UE. [PR 6.41.6-1] The 6G network shall support a mechanism to authenticate and authorize 3rd party AI agent. |
|  | [PR 6.55.6-1] Based on regulatory requirements and operators’ policy, 6G network shall support dynamic identification of 3rd party AI agents. NOTE: Dynamic means the identification of this AI agent can be temporary assigned based on task, and this identification can be assigned by different operators based on different tasks. |
| Group communication related PRs | [PR 6.7.6-4] Based on regulatory requirements and operators’ policy, the 6G network shall provide means to support efficient and secure communication (including multi-modality exchange) between multiple 3rd party AI agents on UEs over a target area. NOTE: This requirement can apply to 3rd party AI agents of same users or different users. It is expected that the required communication service would be provisioned in the range of minutes to days, depending on use case. Lower for temporary task and higher for long term task. |
|  | [PR 6.9.6-2] The 6G system shall support secure interoperability between AI Agents and between AI Agents and applications to achieve a collaborative task. [PR 6.23.6-4] Based on user consent and operator's policy, the 6G network shall support a secure mechanism to provide communication service between AI applications (e.g., AI agent applications) on multiple UEs for a collaborative task. |
|  | [PR 6.52.6-2] The 6G network shall be able to provide a suitable means to allocate network resources (e.g., network slice) to a group of trusted third parties (e.g., applications running on multiple UEs/robots or third party AI agents), based on its availability and capability, considering dynamic changes of traffic demand and QoS characteristics. |
| AI agent discovery | [PR 6.46.6-1] Based on the user consent and operator’s policy, the 6G network shall be able to support the discovery of 3rd party AI agent (application) on the UE. |

Proposal 1: based on the above analysis, the following aspects are to be studied as part of KI regarding AI agent on UE:

-	Whether and how to support the AI agent on UE to discover another AI agent on a different UE

-	Study an identity and profile management for AI agent on UE to identify an AI agent on UE.

-	Study how to support authorization of AI agent on UE.

-	Whether and how to enable communication for AI agents on different UEs via the 6G network

-	Study how to support dynamic group communication for AI agents on different UEs.

1.2.2	6G network provides AI inference as a service

As specified in S2-2507149, offloading AI inference task to server side is becoming increasingly necessary considering the constrains on terminal (e.g., battery, power consumption, memory size etc.). 6G network providing AI inference as a service can control both the data transmission and the AI inference processing. Consequently, better performance can be achieved if these two parts are coordinated with each other compared with handling them independently. This would be beneficial to both APP provider and user experience. Below are some possible optimizations can be studied and performed in 6G network.

Firstly, if 6G network can have the visibility of the application related KPIs - such as E2E latency, first response latency (e.g., TTFT), AI inference speed (e.g., TPOT) – the 6G network can coordinate the AI inference processing node (e.g., the computing site mentioned in WT#6) with transmission nodes to improve the user experience. One possible example of such coordination is by using the AI inference processing status and transmission status to ensure that the AI inference prompts can be sent and processed within proper time. This coordination enables that AI inference results can be sent back to the terminal within the latency requirement to guarantee the user experience without overprovisioning the computing resources and transmission resources.

In addition, when the input data cannot be transferred for AI inference offloading, the first part(s) of AI inference processing (e.g., the first layer(s) of the Neural Network) can be executed at the UE. In this case, how to split the AI inference processing may lead to different E2E latency as the amount of output data in different layers may vary significantly. Hence, 6G network can determine the optimal split considering several factors such as transmission status, UE status and AI inference processing status.

If 6G network supports the optimization of AI inference as a service, the following benefits are foreseen:

- Application Provider can provide user with E2E latency guarantee to enhance user experience without further enhancing and developing specialized mechanisms due to the Mobile Network environment, since 6G network can be responsible to coordinate the transmission and AI inference processing part. Otherwise, without such optimization, if Application Provider would like to achieve E2E latency guarantee, it shall further develop the Application to adapt the AI inference processing based on the transmission status exposed by network, but still would have to rely on uncoordinated and unsynchronized procedures executed at the mobile network and inside the Application Provider This will reduce the changes of the Application Provider ensuring the E2E latency to the users.

- Instead of overprovisioning computing resources and transmission resources to guarantee the E2E latency performance, such optimization can help increase the number of users can be served simultaneously while maintaining same level of configuration of resources without scarifying user experience.

- UE mobility management (e.g., the transmission path switching due to mobility) and the AI inference processing will be handled jointly, leading to optimal routing (e.g., de-tour) to compensate for the unawareness of UE mobility by Application Provider. In this case, the user experience can also be guaranteed.

Proposal 2: based on the above analysis, the following aspects can be studied as part of the KI regarding 6G network providing AI inference as a service:

-	For 6G network providing AI inference as a service to the applications (AF or in UE), the following aspects are further considered:

-	How the 6G network is made aware of the AI inference requirements and application related KPIs (e.g., E2E latency, first response latency, AI inference speed, etc.)

-	Whether and how to coordinate the network transmission and AI inference processing to ensure/improve the performance of AI inference service based on the application related KPIs

-	Whether and how to enable the distribution of the AI inference process (i.e., split AI inference processing) to fulfill the application related KPIs.

1.2.3	6G network provides AI training as a service

Autonomous vehicles and embodied robots are becoming widely used. These terminals are equipped with an AI system having sensors and actuators to perceive surrounding environment and generate actions based on the perceived environmental data and AI model inference processing. In order to make decision accurately, the AI model running on the terminals should be trained/updated with the training data representing different situations/scenarios.

As specified in S2-2507150, existing method to generate the training data is to collect the environmental data manually by driving cars across roads, streets etc. This could lead to high labour cost. 6G network can take its advantages e.g., 1) its wide-area deployed infrastructures making MNO acquire data e.g., via sensing or positioning cost-effectively and 2) its capability to gather different types of data such as positioning of UE, perceived environmental data from UE (e.g., camera, radar data), or other data from NFs to generate the high quality of training data.

For example, delivery company can own the embodied robots to deliver parcel. The AI model of the embodied robots can provide regular route guidance. When the embodied robot encounters with a new area or a new scenario (e.g., unexpected obstacles (e.g., due to accident/road repair) on the path), the embodied robot may have no idea how to conduct route guidance due to the AI model is not adapted to the new area/scenario. In this case, the embodied robot can request the 6G network to provide the training data (with requested data structure) associated with the area/scenario for (re-)training of its AI model. Based on the received request, the 6G network can collect the data from the current/representing area/scenario, generate the training data (synthetic data) meeting the requested training data structure, and online expose/send the generated training data to the embodied robot side for its AI model training. Alternatively, if the embodied robot has limited power or capability to train the AI model locally, it can provision its AI model to the 6G network and request the 6G network to train the model with specific service requirements (e.g., AI model training method to be used, target accuracy that the trained AI model should reach when training completed). In this case, after generating the training data, the 6G network can further train the provisioned AI model online and expose the trained AI model to the embodied robot directly.

Proposal 4: based on the above use case and analysis, the following aspects can be studied as part of the KI regarding 6G network providing AI training as a service:

-	For 6G network providing AI training as a service to the applications (AF or in UE), the following aspects are further considered:

-	Whether and how to generate and expose AI training data to AF or UE based on the AI model training requirement (e.g., requested training data structure, requested location and/or time of the data collected, requested location and/or time to expose the AI training data etc.)

-	Whether and how to train the AI model provisioned by AF or UE and expose the trained AI model based on the AI model training requirement (e.g., requested AI model training method, requested target accuracy, requested location and/or time to expose the trained AI model etc.)

1.2.4	6G network performs traffic handling and provides QoS considering the AI traffic characteristics

According to clause 7.10 of TS 22.261 [1], R18 AIML has studied the QoS requirements for the use cases of split AI/ML inference between UE and Network Server/Application, AI/ML model downloading and the Federated Learning between UE and Network Server/Application function. However, in 6G, the new use case can be real-time AI inference services due to the new technology e.g., generative AI, Large Language Models (LLMs) and AI-accelerated xPUs.

As described in S2-2507306, the tokens exchanged between the users/client and the AI Inference servers could be error tolerant, with different priorities, etc. considering the correlation among the packets (i.e., carrying the tokens) belong to the same inference processing. Based on these characteristics, for example, the 6G network can deliver the data unit even with few packets’ transmission failure due to traffic congestion/unstable coverage, enhancing the overall inference quality. Therefore, higher radio resource efficiency with more experience-satisfied users can be achieved. The potential QoS enhancements would enable the rational scheduling within a data unit while still meeting the inference performance via ensuring the critical packets transmission.

Besides, potential requirements of QoS enhancements for AI applications considering the traffic characteristics of the error tolerance are also described in PR [6.28.6-1] in TR 22.870 [2].

Therefore, it is proposed to study the potential QoS enhancements considering the AI traffic (e.g., AI tokens) characteristics (e.g., error tolerance) in 6G. Therefore, the original wording of the last bullet has been update slightly to reflect these. Additionally, “via user plane” is removed since this wording would lead to confusion. How UE/AF provide the AI traffic characteristics depends on solutions.

Proposal 4: the following aspect is to be studied as part of KI:

Whether and how the UE/AF can provide AI traffic (e.g., AI tokens) characteristics (e.g., error tolerance) to the 6G network so that the network can perform traffic handling and provide QoS considering the AI traffic characteristics.

1.3	Reference

[1]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[2]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements; Stage 1".

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g., AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.2 6G Network for AI:

- Study whether and how to support the AI agent on UE to discover another AI agent on a different UE.

NOTE 1: Coordination with SA6 on the application layer aspects may be needed.

- Study whether and how to enable communication for AI agents on different UEs via the 6G network.

NOTE 2: This AI agent is assumed in the UE but not part of the MT, but it can leverage MT functions. It is also assumed that any solution that suggested using UE to Core Network interaction is not replacing NAS protocol.

NOTE 3: The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

- Study whether and how the 6G network (except RAN) can provide AI inference and AI training as services to applications (AF or in UE).

NOTE 4: Coordination with SA3 and SA5 may be needed on AI inference and AI training.

NOTE 5: Coordination with WT#5 for data framework and WT#6 for computing may be needed.

NOTE 6: New service charging part may be coordinated with SA5 Charging group.

- Study whether and how the UE/AF can provide AI traffic (e.g., AI tokens) characteristics (e.g., error tolerance) to the 6G network so that the network can perform traffic handling and provide QoS considering the AI traffic characteristics.

NOTE X: Coordinated with SA1 on the AI traffic characteristics and coordinated with WT1.2 for QoS.

# * * * * Second change * * * * (All New Text Below)

5.X. Key Issue #X: 6G Network for AI

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The AI services studied in 6G network include communication for AI agents on different UEs, AI inference and AI training etc. This key issue aims at studying whether and how to support such AI services in 6G network and how the 6G network can ensure the performance of the AI services based on the AI service requirements and traffic characteristics. Following aspects are considered to be addressed:

-	Whether and how to support the AI agent on UE to discover another AI agent on a different UE

NOTE 1: Coordination with SA6 on the application layer aspects may be needed.

-	Study an identity and profile management for AI agent on UE.

-	Study how to support authorization of AI agent on UE.

-	Whether and how to enable communication for AI agents on different UEs via the 6G network

-	Study how to support dynamic group communication for AI agents on different UEs.

NOTE 2: This AI agent is assumed in the UE but not part of the MT, but it can leverage MT functions. It is also assumed that any solution that suggested using UE to Core Network interaction is not replacing NAS protocol.

NOTE 3: The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

-	Whether and how the 6G network (except RAN) can provide AI inference as services to applications (AF or in UE). Following aspects are further considered:

-	How the 6G network is made aware of the AI inference requirements and application related KPIs (e.g., E2E latency, first response latency, AI inference speed etc.)

-	Whether and how to coordinate the network transmission and AI inference processing to ensure/improve the performance of AI inference service based on the application related KPIs

-	Whether and how to enable the distribution of the AI inference process (i.e., split AI inference processing) to fulfill the application related KPIs.

-	Whether and how the 6G network (except RAN) can provide AI training as services to applications (AF or in UE). Following aspects are further considered:

-	Whether and how to generate and expose AI training data to AF or UE based on the AI model training requirement (e.g., requested training data structure, requested location and/or time of the data collected, requested location and/or time to expose the AI training data etc.)

-	Whether and how to train the AI model provisioned by AF or UE and expose the trained AI model based on the AI model training requirement (e.g., requested AI model training method, requested target accuracy, requested location and/or time to expose the trained AI model etc.)

NOTE 4: Coordination with SA3 and SA5 may be needed on AI inference and AI training.

NOTE 5: Coordination with WT#5 for data framework and WT#6 for computing may be needed.

NOTE 6: New service charging part may be coordinated with SA5 Charging group.

-  Whether and how the UE/AF can provide AI traffic (e.g., AI tokens) characteristics (e.g., error tolerance) to the 6G network so that the network can perform traffic handling and provide QoS considering the AI traffic characteristics.

NOTE X: Coordinated with SA1 on the AI traffic characteristics and coordinated with WT1.2 for QoS.

# * * * * End of changes * * * *