3GPP TSG-SA WG2#171	S2-2508746

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	[KI#2, New Solution] AIoT Device Registration Procedure and Data Transmission

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-2

Abstract of the contribution: Solution for Registration Procedure for the DO-A Capable AIoT device.

# 1.	Discussion

The solution proposes a simplified AIoT registration for initial registration and mobility registration based on the concept of an AIoT Registration Area.

Although, the general procedure on NAS signaling to perform initial registration or registration update due to mobility or a periodic timer expiration is well known in NR, there are differences for supporting registration-like procedure for A-IoT devices from NR due to the following aspects:​

- There is neither a concept of cell ID(s) nor periodic SIB1 like broadcast signals. Also, the AIoT device can’t determine any PCI-like identifier (which can be determined in NR via decoding PSS/SSS) which can determine the serving cell and hence the location​

- AIoT devices are meant to be low complexity and low-cost devices and might not be able to maintain/run long timers to decide when to perform periodic registration updates​

- AIoT doesn’t reuse the concept of “Tracking Areas” and R19 AIoT introduced the concept of “A-IoT areas”​ which are however not visible to the AIoT Device

- AMF need not be involved in registration procedures as it is the case for NR​

- AIOTF is the most appropriate node to be responsible for registration of an AIoT Device, and the AMF can be used only for routing of signalling

- AIoT NAS is supposed to be simpler compared to Uu NAS​

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-xxx:

# >>>>BEGINNING OF CHANGES<<<<

## 6.X	Solution #X: AIoT Device Registration Procedure

### 6.X.0	High-level solution Principles

The following principles apply:

-	A DO-A capable AIoT Device actively register to the network to inform the network of its presence autonomously, and has the ability to send device originating signalling and data. The AIoT Registration enables the network to track the AIoT Device within an AIoT Registration Area and allows the network to reach the device by paging it in a subset of the network (in the AIoT Registration Area) only. This is especially useful for outdoor scenarios but also for nomadic indoor scenarios (e.g., AIoT device is moved to a different warehouse)​.

-	AIoT NAS is assumed to be simpler compared to Uu NAS and therefore the AIoT Registration is simplified w.r.t. the traditional registration.

-	AIOTF performs the NAS termination from the AIoT Device, including AIoT device authentication and registration management for AIoT Devices.

-	AIoT devices are meant to be low complexity and low-cost devices and might not be able to maintain/run long timers to decide when to perform periodic registration updates​, therefore a periodic AIoT Registration is not proposed or may be made optional.

-	AMF need not be involved in registration procedures as it is the case for NR​.

-	The AIOTF supports termination of AIoT NAS protocol with an AIoT Device (TS 23.369[x] and TS 33.369 [y]), therefore in this solution the AIOTF is the NAS termination point. AIOTF is the most appropriate node to be responsible for registration of an AIoT Device, and the AMF can be used only for routing of signalling.

-	No PLMN selection is assumed and the AIoT device shall attempt registration only in the PLMN it belongs to.

### 6.X.1	Description

This solution addresses KI#2 on how to enable the device to inform the network of its presence autonomously for Topology 1.

In this solution, it is assumed that the AIoT Device actively initiates the registration process with the network. The  registration type can:

- initial registration: the AIoT Device performs an initial registration based on one or more triggers received in in an “AIoT system information” in an R2D message (depending on RAN2 decision) or based on some pre-specified criteria (e.g. including upon switching on, upon receiving an explicit trigger from gNB/AIOTF, upon satisfying a certain filter criteria, at a certain time) when the AIoT Device needs to inform the network of its presence, and prior to autonomously sending data.

- mobility registration update: the AIoT Device performs a mobility registration based on a trigger. This solution proposes a trigger based on an AIoT Registration Area provided to the AIoT Device when it registers with the network.

Editor’s Note: It is FFS whether other registration triggers are required and/or feasible in this release.

In this solution, the AIoT Registration signalling can include one or more of the following:​

- AIoT Registration Type: initial registration, mobility registration​

- AIoT Device ID​

- Security parameters for authentication​

- AIoT device’s capabilities (including security capabilities)

The solution assumes that RAN WGs would define features to support the AIoT Device to identify whether it has moved from the current AIoT Registration Area, which may e.g. include the gNB/UE reader  sending “A-IoT system information” in an R2D message containing one or more of the following:​ Reader ID(s) supported by the gNB or UE reader ID​, AIoT areas supported by the gNB or UE reader, mapping of Reader to AIoT area(s), a list of PLMN(s) or SNPN(s) supported by the gNB/reader​, triggers for A-IoT registration (e.g., all devices should perform initial registration, only devices matching a certain filter criteria needs to perform initial registration, perform registration only upon explicit request from UE reader/gNB, perform registration  only at a certain time, etc.)​.

Editor’s Note: the details of the information provided to the AIoT Device and by whom are dependent on the work in RAN WGs and will be aligned once RAN WGs progress.

### 6.X.2	Procedures

Figure 6.X.2.1: Registration Procedure

1.	If the AIoT Device is in the home PLMN, the AIoT Device sends a D2R message to NG-RAN, including D2R parameters (including at least a Temporary Registration ID, if available, and the selected PLMN ID to enable the NG-RAN to route to the appropriate AIOTF) and a AIoT Registration Request (AIoT Registration Type, device information, security parameters).

Editor’s Note: the D2R parameters can be defined based on the work in RAN WG.

The AIoT Registration Type indicates if the AIoT Device is perform an Initial Registration or a Mobility Registration Update. The AIoT Device performs an Initial Registration if it is not registered with the network and performs a Mobility Registration depending on a registration trigger (e.g. if the AIoT Device has moved outside the AIoT Registration Area).

2.	The NG-RAN selects the Serving AIOTF based on the D2R parameters and local configuration. The NG-RAN sends the selected AIOTF ID to AMF which routes the messages to the selected AIOTF.

3.	NG-RAN sends the AIoT Registration Request to the Serving AIOTF including the temporary ID and the selected PLMN ID. When the temporary ID identified a different AIOTF with which the AIoT Device registered previously, the Serving AIOTF fetches the AIoT Device context from the previous AIOTF.

4.	[Conditional] The Serving AIOTF determines the old AIOTF using the Temporary ID, and retrieves the AIoT Device context from the old AIOTF.

5.	The Serving AIOTF may perform authentication of the AIoT Device, and stores the security context.

Editor’s Note: Details of authentication procedures will be determined by SA3 WG.

6.	The Serving AIOTF selects the ADM for the AIoT Device based on the AIoT Device’s permanent ID.

7.	The Serving AIOTF indicates the ADM it is serving the AIoT Device. The Serving AIOTF may retrieve the subscription data for the AIoT Device.

9.	The AIOTF sends the Registration Accept to the AIoT Device, including a new Temporary Registration ID and AIoT Registration Area.

In order to support periodic AIoT Device registration updates without mandating the support of timers in the AIoT Device, a reader can periodically perform a “special” inventory and include the Reader’s Area Information. Upon receiving the “special” inventory request, e.g. containing the Reader’s AIoT Area Information, only the “new” AIoT Devices that have not seen this Reader’s area information before can respond to perform periodic registration area update.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support AIoT Registration, including retrieving context from another AIoT, supporting AIoT Device authentication, and providing AIoT Registration Area to the AIoT Device.

NG-RAN:

-	Supports AIOTF selection and transferring NAS messages to and from the AIoT Device.

AIoT Device:

-	Support the AIoT Registration procedure and processing the AIoT Registration for mobility.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<