3GPP TSG-SA WG2#171	S2-2508754

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	[WT#1.3, non-3GPP Access] Discussion paper on way forward for non-3GPP access

Document for:	Discussion

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this paper introduces some rational on the way forward for non-3GPP.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

WT#1.3 contains the following text:

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

5GS introduced N3IWF which enabled NAS as the common control plane between UE and network for both 3GPP and untrusted non-3GPP RATs. This was driven by the desire to achieve an access agnostic core and a common RAN-CN interface, which the N3IWF indeed enabled.

However, the introduction of the N3IWF led to unnecessary complexity on both the UE and the network side since many of the NAS concepts are specific to 3GPP access and are not applicable to non-3GPP accesses.​

​It has repeatedly been noted that the only realistic commercial use case so far for 3GPP-WiFi interworking is operator voice over Wi-Fi, which in commercial deployments is being addressed by the ePDG, and the support of selected operator services. It is worth noting that the ePDG solution does not suffer from the same complexity drawbacks as N3IWF.​

For these reasons, we argue that it is important for 6G to study a lean solution for non-3GPP access for operator voice service following the key principles of the ePDG solution, i.e., a secure entry point into the operator’s core network, without introducing unnecessary complexity for the UE and the network to support such use case.

We therefore propose that in WT#1.3 we do:

- not study generic non-3GPP access because there are only two specific access types in this area (WiFi and fixed) that require different solutions as we have seen in the past.

- focus on two specific use cases instead: (1) Operator voice over WiFi and (2) fixed access integration.

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

# >>>>BEGINNING OF CHANGES<<<<

Annex A:
Work Tasks

Editor's note:	This Annex will capture updates to WT scope based on the 6G SID in SP-250806. The plan is to update the Study Item Description with updated Work Task descriptions from this Annex.

This Annex contains clarifications of Work Task scope, as preparation for Key Issue drafting.

## A.X 	Work Task 1

Editor's note:	Description of Work Task X. Sub-clauses can be used if needed.

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

1.3.	Study whether and how to support and/or enhance non-3GPP access in 6G focusing on the following use cases:

- operator voice and selected operator services over Wi-Fi

- fixed access integration

NOTE: solutions for the various use cases may be different.

Editor’s Note: other use cases may be added on a contribution-basis.

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.

# >>>>END OF CHANGES<<<<