3GPP TSG-WG SA2 Meeting #171	S2-2508779

# Wuhan, China, 13 – 17 October 2025		(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0101 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to AIOTF and ADM to support temporary ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | Below is an excerpt of SA3 WG conclusions during the meeting in August 2025: ## 5.4 Protection of AIoT device identifier privacy ### 5.4.1 General This clause describes the mechanisms to protect AIoT device identifier privacy during the inventory procedure. The mechanism is based on the use of a Temporary ID (i.e., T-ID). The T-ID is generated based on the key (i.e., KAIoT_root) shared between AIoT device and ADM. Depending on the situation and deployment scenario, the network operator can choose which paging procedure to use. When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2. … Considering the SA3 conclusion, the functionality of AIOTF and ADM, as specified in TS 23.369, needs to be modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The descriptions for IOTF and ADM is update to intropduce the support of Temporary ID. Further, the Temporary ID is introduced in a new clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The temporary ID concept is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3, 4.5.9, 5.5, new 5.7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an NG-RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the NG-RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the NG-RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	NG-RAN selection and optionally a list of RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the NG-RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

-	During Inventory procedure, if the privacy of an AIoT Device identity is enabled in the AIOTF, the AIOTF sends a request to the ADM to retrieve temporary ID for the AIoT Device.

-	Perform aggregation of the AIoT service responses, determine and provide assistant for the AIoT aggregation in NG-RAN.

NOTE:	Authentication of AIoT Devices related to AIOTF is defined in TS 33.369 [9].

* * * * Start of 2nd Change * * * *

### 4.5.9	ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

-	Management of AF authorization data.

-	Temporary ID generation for AIoT Device.

The AIoT device profile data and AF authorization data used by the ADM may be stored in the UDR.

* * * * Start of 3rd Change * * * *

## 5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. AIoT Device Permanent ID or credentials) is stored external to the network.

The AIoT Device Permanent ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device Permanent ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in clause 5.2.3 of TS 23.502 [4], it is stored in the ADM network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

In case when the privacy protection for the AIoT Device Permanent ID during the inventory procedure is is enabled, the ADM may stored an AIoT Device Temporary identifier derived from previous interaction with the AIoT device as described in [9].

The AIoT device authentication is performed at the ADM. For this purpose, the ADM stores AIoT Device credentials.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |
| AIoT Device Temporary ID | A temporary ID may be stored as described in [9]. |
| Security credentials | Security credentials stored per AIoT device as described in [9]. |

NOTE:	Security materials and security mechanism involving ADM are specified in TS 33.369 [9].

* * * * Start of 4th Change * * * *

### 5.7.x	AIoT Device Temporary Identifier

The AIOTF may store a local configuration to enable the privacy protection for the AIoT Device Permanent ID during the inventory procedure. The following applies:

-	If the privacy protection is not enabled, the AIOTF shall use as AIoT Identification Information to the NG-RAN either the Filtering Information or the AIoT Device Permanent Identifier if provided in the request from the AF/NEF.

-	If the privacy protection is enabled, the AIOTF shall use as AIoT Identification Information to the NG-RAN either the Filtering Information or a AIoT Device Temporary ID. The AIoT Device Temporary ID is used when the AIoT Device Permanent Identifier is included in the request message from the AF/NEF.

If the AIOTF determines that an AIoT Device Temporary ID has to be used in the Inventory request message, the AIOTF sends a request to the ADM. The ADM generates the AIoT Device Temporary ID based on the credentials stored at the AIoT device and ADM as described in [9].

Depending on the situation and deployment scenario there can be different types of AIoT Device Temporary ID. The ADM determines which type of AIoT Device Temporary ID is used. The following types are possible:

-	AIoT Device Temporary ID of type stored. This T-ID is not concealed. It is derived and stored at the ADM from previous exchange with the AIoT device.

-	AIoT Device Temporary ID of type concealed stored. It is stored at the ADM from previous exchange with the AIoT Device is used as input to a pre-configured concealment mechanism.

-	AIoT Device Temporary ID of type concealed AIoT device permanent identifier. The AIoT Device Permanent Identifier stored in the ADM is used as input to a concealment mechanism and the output is the AIoT Device Temporary ID.

The AIOTF includes an indication about the type of AIoT Device Temporary ID sent within the Inventory request message to the NG-RAN. This indication is further sent to the AIoT Device in the RAN paging procedure. The AIoT Device uses the type of AIoT Device Temporary ID to compare the received AIoT Identification Information (i.e. paging ID) with a corresponding same type of AIoT Device Temporary ID generated or stored internally in the AIoT device.

* * * * End of Changes * * * *