3GPP TSG-WG SA2 Meeting #171	S2-2508813

Wuhan, China, October 13 - 17, 2025

Source: 	Interdigital

Title: 	Proposal for Interim Agreements on KI#3

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for interim agreements on KI#3.

# 1	Introduction

This contribution provides the interim agreements on KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.3	Agreed Principles for KI#3

- A Sensing Function registers its NF profile as a sensing function at NRF. The registered NF profile includes its capability as a sensing function, a sensing service area, and the supported sensing service types.

- It is assumed that, for each sensing function, the list of gNBs as authorized sensing entities at the sensing service area of the sensing NF and the capability of each gNB with its location as a sensing entity are preconfigured at the sensing function.

- When a sensing service request is received from SSC, a sensing function is discovered and selected for the sensing operation at the target sensing service area for the requested sensing service type based on its registered NF profile at NRF.

- A sensing function discovers and selects gNB(s) for the requested sensing service type at a target sensing service area from the preconfigured information within the sensing function.

# * * * * End of change* * * *