3GPP TSG-SA2 Meeting #171	S2-2508847

October 13-17, 2025, Wuhan, China	(revision of S2-2508121)

Source:	Ericsson, AT&T

Title:	[WT#1.2, UP] WT & KI for User plane architecture

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: Proposal for updates on the WT#1 item for User plane architecture taking S2-2508121 as baseline for discussion and updates and for Key Issues

# Introduction

This contribution includes clarifications and proposals on some of the aspects that were included as WT description in baseline document S2-2508121:

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic.
Editor’s Note: whether this issue belongs to UP or QoS is FFS.

- Whether and how to enhance CP-UP interaction for better interoperability.

- Whether and how to extend the service-based architecture for the user plane control interface to UPF.

- Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

- Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

This document also makes a proposal for Key Issues

# 2. Discussion

Regarding the item:

“Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic.
Editor’s Note: whether this issue belongs to UP or QoS is FFS.“

Expectations for network capabilities and application performance continue to rise with 6G, however:

- Application traffic is increasingly encrypted, and the execution of some pooicy rules (traffic detection and actions) is more and more often not possible. Traffic identifiers that networks have historically relied upon for traffic detection are becoming less stable or opaque. Server Name Indication (SNI) is increasingly encrypted, connection 5-tuples are volatile due to mobility; NAT rebinding, multipath, and QUIC connection migration cause 5-tuple change, and in IP, 3-tuples can not be treated as distinct flow identifiers in upcoming deployments.

- End-to-end protocol fields to determine performance is becoming impractical. The ability of networks to observe, measure, and demonstrate delivered performance is being limited by encryption.

- Applying a Maximum Bit Rate (MBR), and general throttling mechanisms interact poorly with modern media and interactive applications, create complications for adaptive bitrate algorithms and disrupt latency-sensitive transport behaviour, thus degrading user experience.

- Insertion/detection of protocol headers for sharing information along the path has become impossible, ineffective and inappropriate by encryption and integrity in modern protocol designs.

As a conclusion, as part of userplane evolution, on-path application-network collaboration should be assessed to determine whether and how on-path explicit information sharing between endpoints and the network userplane functions can enable that certain policies can be applied and assist when user plane functions cannot rely on end-to-end protocol fields. Actions that can benefit include differentiated QoS/Charging, information sharing, performance monitoring, etc

Regarding the item:

“Whether and how to enhance CP-UP interaction for better interoperability”.

This is a pure protocol issue that should be addressed by the CT WG4. CT4 has already performed a study on this topic and it has introduced enhancements. Any remaining gaps and how to address them should be discussed in that CT4. On the other hand, if finally SBI is introduced for PFCP over N4, interoperability (for example driving for simplicity avoiding alternatives) shall be a priniciple for the stage 2 and stage 3 normative work.

Regarding the item:

“Whether and how to extend the service-based architecture for the user plane control interface to UPF.”

This study sub-WT should answer the question of whether and how CP should use SBI to control the UP packet processing. Considering that,

-   Introducing SBI for N4 is likely to complicate interworking and drive cost during migration (two separate protocols for the same purpose co-existing for a long time), and

-   Enhancing PFCP to address the needs of 6G use cases may be an alternative.

It needs to be studied whether to use SBI, or to enhance PFCP to address the 6G requirements. The proposal is to study this topic in CT WG4 where the competence on these protocols resides. When the study is completed, SA2 can implement the stage 2 specifications based on the conclusions.

Regarding the item:

“Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.”

Along the 5G releases, mechanisms have been specified and evolved for user plane flexibility. Most recently, UPEAS phase 2 included a key issue with focus on user plane flexibility; from Rel-19 UPF (re)selection can also consider UP capabilities outside the 3GPP scope. Taking into account that the 6G connectivity will be used for even more services and use cases, 6G will also need a flexible user plane. Given previous and very recent efforts, the Study should focus on identifying whether there are any gaps and whether any additional mechanisms are needed on top of what has already been defined.

# 3. Proposal

Introduce the following changes in TR

**** First Change ****

# Annex A.X. WT#1.2 [User Plane] Scope

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS User plane framework as starting point:

1.	Whether and how to enhance application-network collaboration for 6G user plane traffic to enable that the application end-points and the network explicitly exchange information on-path that allows the network and the application to adapt to application traffic (e.g. when end-to-end protocol fields can not be relied for this due to encryption, etc).

NOTE x: "on-path" refers to exchanging the information on the same path as the application traffic.

NOTE y: this WT can be used, for example, to apply rules with Handling of Headers control (e.g. to detect or provide tokens) or rules for differentiated QoS and/or charging, etc. It could also be used to monitor performance (e.g. packet loss), etc.

NOTE z: This WT needs to be coordinated with the 6G work on the QoS framework, and the common exposure framework.

2.  Whether and how the user plane flexibility (i.e., UPF (re)selection) for different service requirements needs to be enhanced, considering path performance and capability between access network and data network.

3.  Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

**** Next Change  ****

# 5.X. Key Issue #X Enhanced User Plane Architecture for 6G

### 5.X.1 	General

The contents of the 5GS will be used as a starting point for the study.

### 5.X.2 	Key Issue #X.1: Enhanced Network-Application collaboration by on-path information exchange

Expectations for network capabilities and application performance continue to rise with 6G, however:

- 	Application traffic is increasingly encryted, and the execution of some policy rules (traffic detection and actions) is more and more often not possible. Traffic identifiers that networks have historically relied upon for traffic detection are becoming less stable or opaque. Server Name Indication (SNI) is increasingly encrypted, connection 5-tuples are volatile due to mobility (with NAT rebinding, multipath, and QUIC connection migration whichcause 5-tuple change), and IP 3-tuples can not be treated as distinct flow identifiers in upcoming deployments.

-  Using End-to-end protocol fields to determine performance is becoming impractical.The ability of networks to observe, measure, and demonstrate delivered performance is being limited by encryption and header integrity.

-  Applying a Maximum Bit Rate (MBR), and general throttling mechanisms interact poorly with modern media and interactive applications, create complications for adaptive bitrate algorithms and disrupt latency-sensitive transport behaviour, thus degrading user experience.

-  Insertion/detection of protocol headers for sharing information between network and end-point has become impossible, ineffective and inappropriate by encryption in modern protocol designs.

In cases where user plane functions cannot rely on end-to-end protocol fields, the execution of rules could evolve and work in a collaborative way with applications, for example sharing information on-path that the network and end-points can use.

This key issue will study whether and how to exchange on-path information between application and network to allow for better traffic handling, including

-  What and how is the information exchanged

-  What enhancements are needed if any in the UP packet processing control and UP event exposure.

NOTE x: "on-path" refers to exchanging the information on the same path as the application traffic.

NOTE y: On-path information exchange may be used to apply rules with Handling of Headers control (e.g. to detect or provide tokens) or rules for differentiated QoS and/or charging, etc. It may also be used to monitor performance (e.g. packet loss), etc.

NOTE w: the user plane actions are controlled by the policy framework, which guarantees that the on-path communication is used in an operator-controlled manner.

NOTE z: Other key issues may also identify information (e.g. QoS) to be shared on-path.

NOTE h: This KI is not to study an on-path alternative for the exposure/reporting of capabilities. If that issue is studied, it should be done under the common exposure framework WT. Coordination is needed between this WT and the common exposure framework WT.

### 5.X.3	Key Issue #X.2: Enhancements on User Plane Flexibility

Along the 5G releases, mechanisms have been specified and evolved for user plane flexibility. This KI should focus on identifying whether there are gaps and whether additional mechanisms are needed on top of what has been defined.

The following aspects should be studied under this Key Issue:

-  Whether and how the user plane flexibility (i.e., UPF (re)selection) for different service requirements needs to be enhanced, considering path performance and capability between access network and data network.

### 5.X.4	Key Issue #X.3: User Plane Enhancements related to mobility and session continuity

Along the 5G releases, mechanisms have been specified and evolved to handle user mobility and session continuity, including capabilities for most demanding services (e.g. URLLC). The KI should focus on enhancements addressing gaps and opportunities to optimize the procedures.

The following aspects should be studied under this Key Issue:

-  Whether and how the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services) needs to be enhanced.

*** End of Changes ****