3GPP TSG-SA2 Meeting #171	S2-2508862

Wuhan, China, October 13 – 17, 2025

Source:	Ericsson

Title:	Consideration of RAN activation and support of available bitrate monitoring

Document for:	Approval

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/Rel-19

Abstract: This contribution provides a proposal on signaling of the status on support for Available Bitrate Monitoring from NG-RAN to 5GC.

# 1. Introduction

SA2 received from RAN3 in S2-2506144 the LS to SA2 on reporting status of Exposure of available bitrate stating the following:

RAN3 has discussed whether gNB provides the status information (e.g. explicit activated/deactivated status information, or only explicit activated information) of Available Bitrate monitoring to SMF after gNB receives the request for Available Bitrate monitoring. RAN3 kindly asks SA2 if such status information is beneficial from CN’s perspective.

# 2. Discussion

Available bitrate monitoring was introduced in Rel-19 as a concept that works very similarly to ECN marking reporting: a configuration phase is first performed over NG-RAN CP interfaces followed with the reporting over UP towards UPF. The ECN has a reporting status indication (ECN Marking or Congestion Information Reporting Status IE) signalled back by the NG-RAN to SMF on a per QoS Flow per PDU session basis, prior to UP reporting, which is indicating the activation status at NG-RAN.

Therefore, a similar activation status indication for available bitrate monitoring should be made available to SMF by gNB, because otherwise the SMF will not know if the NG-RAN will monitor available bitrate for all or only some QoS flows.

Observation 1: Available bitrate monitoring works in a similar way as ECN or congestion monitoring, with a CP configuration and UP reporting. The gNB indicates the activation status of ECN marking / congestion monitoring per QoS flow to the SMF over CP.

Observation 2: Unlike for ECN/congestion monitoring, in case of available bitrate monitoring, the SMF does not know the activation status of available bitrate monitoring for a/all QoS flow at the gNB..

The knowledge about the status of available bitrate monitoring at NG-RAN can be used by SMF when configuring the UPF to perform monitoring of the reports/results provided by NG-RAN in the UP and thus consequently configure the UPF to monitor only on QoS Flows where NG-RAN indicated that available bitrate monitoring is active. This will ensure that UPF capacity is not impacted by monitoring on QoS Flows where NG-RAN has not activated available bitrate monitoring.

Observation 3: SMF awareness of the activation status of the available bitrate monitoring at NG-RAN can be used to avoid impacts to UPF capacity.

To enable visibility of the NG-RAN status for available bitrate monitoring in 5GC, SA2 should answer to RAN3 that it is useful that at the setup of the PDU Session the SMF is made aware of the available bitrate monitoring activation status in NG-RAN. Similarly during handover, the SMF should know the status of Available bitrate monitoring at the target NG-RAN node

The impacted NGAP messages will be the PDU SESSION REQUEST/MODIFY RESPONSE messages, Handover Request Acknowledge and Path Switch Request where the Available Bitrate Reporting activation status is reported.

# 3. Proposal

Based on the discussion in this contribution, the sourcing company of this paper makes the proposal below:

Proposal 1: SA2 inform RAN3 that it is beneficial to add an activation status information in the PDU SESSION REQUEST/MODIFY RESPONSE messages, Handover Request Acknowledge and Path Switch Request messages for the available bitrate monitoring.

To address this proposal, the sourcing companies submitted a CR5548 to TS 23.502.