3GPP TSG-WG SA2 Meeting #171

October 13 – 17, 2025, Wuhan, China	S2-2508891

Source:	Ericsson

Title:	PCF Applies an Energy Saving QoS Profile for Energy Saving

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_ph2/Rel-20

Abstract of the contribution: PCF applies an alternative QoS profile for energy saving.

# 1. Introduction/Discussion

The Solution #7: Enabling dynamic QoS handling for energy efficiency in 3GPP TR 23.700-67 proposes

The original high level proposal:

“AF provides Energy Saving Service Requirements along with legacy QoS requirements to PCF, the Energy Saving Service Requirements which indicates the service requirement(s) for the AF session if energy saving is applied by the 5GS.

-	PCF generates the Energy Saving QoS profile(s) and the priority list for the QoS flow based on AF's request and operator policy. The Energy Saving QoS profile(s) can be applied to both GBR and Non-GBR QoS flow.

-	PCF sends the Energy Saving QoS profile(s) and the priority list to SMF and then to NG-RAN along with the legacy QoS profile.

-	NG-RAN selects the Energy Saving QoS profile only if NG-RAN decides to downgrade the QoS parameters due to energy saving.

-	NG-RAN notifies the selected Energy Saving QoS profile to SMF, SMF will further inform the selected Energy Saving QoS profile to PCF and to UPF for charging purpose.

In Solution #7

- It proposes the Energy Saving QoS profiles can be applied for non-GBR QoS flow as well. However, the NG-RAN today does not enforce QoS control for non-GBR QoS flow and does not provide relevant notification.

- The NG-RAN can only take action to apply energy saving QoS profile based on the NG-RAN energy related information. The NG-RAN does not have holistic view like considering core network energy related information and fine-granular Energy information from EIF.

Therefore, this pCR proposes, to modify or enhance the solution #7 to let PCF in the 5GC to apply the Energy Saving QoS profile according to the Energy related information from either OAM (slice or 5GC level energy information) or EIF.

It is because

- PCF can access the energy related information from EIF and OAM including both RAN and 5G Core network. It is also possible to extend the EIF and OAM to provide energy related information considering RAN only if needed.

- PCF can instruct the UPF through SMF to perform rate limiting at SDF level while the RAN cannot. A QoS flow might contain several SDFs. SDF level enforcement gives more fine-granular control than QoS flow level control. A QoS flow level bit rates are just an aggregation of bit rates from all SDFs. See Figure 1.1.

- UPF can enforce the DL/UL SDF MBR in a non-GBR QoS flow

- UPF can enforce the DL/UL SDF MBR/GBR in a GBR QoS flow

Figure 1.1 QoS Enforcement and Rate Limitation in RAN and UPF

-  The existing Alternative Service Requirements (similar concept) sent from AF, see section 5.14, TS 29.122, is provided at SDF level. Energy Saving Service Requirement at SDF level makes it aligned with existing Alternative Service Requirements usage.

PCF can after applying the Energy Saving QoS profile for the SDF, also send the notification to the AF to indicate the applied Energy Saving Service Requirement.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-67:

First change

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |  |
| x |  | X |  |  |

Second change (all new)

## 6.X	Solution #XX: NG-RAN indicates alternative QoS profiles to the PCF for energy saving

## 6.X.0	High-level solution principles

The solution is based on following principles:

- AF provides Energy Saving Service Requirements for each SDF along with expected service requirements to PCF, the Energy Saving Service Requirements are allowed be applied by the 5G network if energy saving is needed.

- PCF generates the Energy Saving QoS profile(s) and the priority list for the QoS flow based on AF request and operator policy. The Energy Saving QoS profile(s) can be applied to SDFs in both a GBR flow and a non-GBR QoS flow.

- PCF collects the energy related information from the OAM/EIF.

- PCF applies service parameters in one of Energy Saving QoS profile when energy consumption related information goes above a threshold.

- PCF notifies the AF for the applied Energy Saving Service Requirement.

NOTE:	The energy-related information at UE or NF level used in KI#2 and KI#3 should be based on robust and accurate enough estimates to be fit for purpose.

### 6.X.1	Functional Description

This solution addresses KI#2.

AF provides a list of Energy Saving Service requirements per SDF such as described in section 5.14 AF session with required QoS, TS 29.122 along with the expected Service requirement to PCF through NEF.

PCF generates the Energy Saving QoS profiles for SDFs based on AF's request. This will be applied for SDFs in a GBR and non-GBR QoS flow. If the AF does not provide the Energy Saving Service Requirements, the PCF can generate the Energy Saving QoS profiles according to operator policy. The SDFs in GBR can have MBR. The SDFs in non-GBR have both MBR/GBR.

The PCF applies one of the Energy Saving QoS profile when decide to perform service adjustment by monitoring the consumption of energy related info in the network.

The PCF notifies the AF through NEF that an Energy Saving Service Requirement is currently applied.

### 6.X.3	Procedures

The picture below shows an example procedure where the Energy Saving Service Requirements are provided by AF to PCF. The PCF applies the Energy Saving requirements by monitoring consumption of energy related info of a network slice.

Figure 6.X.3.1 PCF applies Energy Saving QoS Profile by monitoring energy consumption related information of network slice

Prerequisite:

- PDU sessions from UEs in a network slice were established.

- UDR stores slice energy policy data.

1. PCF queries UDR for the slice policy data of energy savings.

2. UDR returns thresholds for energy related information monitoring for a slice.  See examples below:

| type | Threshold | Severity | Period |
| --- | --- | --- | --- |
| Energy consumption | 2000KWH | 1 | Hour |
| Energy consumption | 4000KWH | 2 | Hour |
| Non-renewable Energy consumption | 1000KWH | 1 | Hour |
| Non-renewable Energy consumption | 4000KWH | 2 | Hour |
| Non-Renewable energy ratio | 60% | 1 | 4 hours |
| Non-Renewable energy ratio | 80% | 2 | 4 hours |

NOTE:

- Bigger Severity value means a more aggressive service adjustment shall be applied such as picking a lower priority Energy Saving Service Requirement of the AF provided list.

- Hysteresis thresholding – hi/low value can be applied instead of just single value which is an example.

- Period is a measurement interval for a metric.

3. The PCF retrieves the slice level energy related information. For example:

- Energy consumption for a period.

- Non-renewable Energy consumption for a period.

- Renewable energy ratio for a period.

4. For each PDU session, AF may create an AF session, according to the procedures in 5.14 AF session with required QoS in TS 29.122, with SDF(s) containing Energy Saving Service requirements allowing them to be applied by the network for energy saving.

5.  PCF detects that a EnC related consumption of a slice goes above a threshold and decides to apply service adjustment. Note: The trigger from slice level is an example, the UE or SDF level threshold monitoring can also be used.

6. Optional: PCF may request the EIF/NWDAF to provides the X percent of the most energy consuming SDFs with PDU session ids.

7. EIF/NWDAF provides the list of SDFs with PDU session ids in the slice.

Note: PCF can alternatively apply policy decisions for SDFs in PDU-sessions (AF sessions) in the slice served by it without querying the most energy consuming SDFs in step 6.

For each of the returned SDF/PDU session associations

8. PCF checks if Energy Saving QoS profile is available for an SDF. If it is provided, the PCF selects a QoS Profile from the prioritized list (the first one has the highest priority thus lower energy saving) according to the monitoring threshold severity in the policy data, e.g. severity 1 will be mapped to the first QoS profile. Note. How to map the severity into QoS Profile Selection is up to PCF implementation.

PCF applies the selected QoS profile for the affected SDF in a PDU session using procedures in Section 4.16.5.2 PCF initiated SM Policy Association Modification, TS 23.502.

9. PCF adds notification to NEF to indicate that the QoS cannot be guaranteed due to energy saving and an Energy Saving Service Requirement is applied.

10. NEF sends the notification to the AF.

11. AF acknowledges the receipt of the notification.

12. NEF acknowledges the receipt of the notification.

### 6.X.4	Impacts on existing services, entities and interfaces

AF:

- Provide Energy Saving Service Requirements to NEF.

- Receive Nonfiction that an Energy Saving Service Requirement is applied.

NEF:

- Support Energy Saving Service Requirement provided from AF.

PCF:

- ApplyEnergy Saving QoS Profiles according to AF input or operatior policy.

- Send Notification to AF that an Energy Saving Service Requirement is applied to AF through NEF.

EIF/NWDAF:

- Provide the most energy saving SDFs with PDU session ids in a slice.

UDR:

- Store energy control policy data per slice.

# * * * * End of changes * * * *