3GPP TSG-SA WG2 Meeting #171	S2-2509079

13-17 October 2025, Wuhan, CN	(revision of S2-2507233)

Source:	Xiaomi

Title:	KI#3: New Sol: Supporting of IMS emergency call over NB-IoT GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#3.

1. Introduction/Discussion

This contribution is proposed to address KI#3 to support of IMS emergency call over NB-IoT NTN via GEO connecting to EPC.

2. Proposal

It is proposed to capture the following text in FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * Start of Change* * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| x |  |  | x |  |

* * * Second of Change (all new text)* * * *

6.X	Solution #x: support of IMS emergency call over NB-IoT via GEO

6.x.1	High level principles

The following principles are applied to this solution:

- Procedures for UE detectable and non-UE detectable IMS emergency session defined in TS 23.167 are used as baseline.

- Enhancements to the UE, NB-IoT(GEO) and EPC for supporting IMS emergency call shall be minimized.

- This solution only focuses on enhancement to EPS. The solution may combine with IMS optimization proposed by other solutions to support the IMS emergency call over NB-IoT(GEO).

6.x.2	Description

This solution addresses KI#3 to support of IMS emergency call over NB-IoT(GEO) connecting to EPC.

As specified in TS 23.167, the IMS emergency service can be a UE Detectable Emergency Session or a non-UE Detectable Emergency Session.

UE Detectable IMS emergency session

The UE shall perform emergency attach if not attached or request emergency PDN connection if already normal attached. Then, the UE performs an IMS emergency registration using the emergency PDN connection before initiating a UE detectable IMS emergency session.

As sepecified in TS 23.401, to enable the UE detected IMS emergency call over E-UTRAN, the emergency bearer services are provided to support IMS emergency session. The PLMN supporting emergency bearer services sends an Emergency Service Support indication in the Attach or TAU procedure. Based on the indication, the UE initiates emergency PDN connectivity procedure to establish the emergency bearer service. However, emergency bearer service is not supported when the UE is using NB-IoT, and an NB-IoT cell shall not indicate support for emergency service in any broadcast information.

The solution proposes to remove those restrictions mentioned above to achieve IMS emergency call over NB-IoT(GEO) connecting to EPC. The proposed enhancement on supporting multiple bearers for normal IMS voice service also applies to IMS emergency service.

Non-UE Detectable IMS emergency session

For non-UE detectable IMS emergency services, the UE performs normal attach, establishes normal PDN connection for IMS service and performs normal IMS registration. When the P-CSCF detects a UE initiated IMS session is an emergency session, the following handling can be applied for the non-UE detectable emergency service:

-	The P-CSCF rejects the session initation request to the UE with an indication that this is an emergency session, then the UE re-initiates a detectable emergency session; or

-	The P-CSCF allows the session intiation request to continue and the E-CSCF informs the UE that it is an IMS emergency session. Such an emergency session will use the normal IMS PDN connection.

Considering the rejection sending to UE and subsequently followed by a UE detectable emergency session initiation will cause larger delay when UE using NB-IoT(GEO), it is proposed to continue the non-UE detectable emergency session over the normal IMS PDN connection in the case of UE using the NB-IoT(GEO).

6.x.3	Procedures

6.x.3.1	UE detectable IMS emergency session

Figure 6.x.3.1-1: UE Detectable IMS Emergency session

- A GEO satellite of a NB-IoT access may broadcast its support of IMS emergency call over NB-IoT. This may help UE to determine whether camp on the cell. Considering the NTN cell may serve multiple PLMNs from different countries/areas, the support indication can be per PLMN granularity.

- The UE initiates the Attach procedure by transmission of an Attach Request to the MME.

- The MME sends an Attach Accept message to the UE including the Emergency Service Support indication. The indication informs the UE that Emergency bearer services are supported, i.e. the UE is allowed to request PDN connectivity for IMS emergency service when the UE is using NB-IoT (GEO).

- Based on indication, the UE initiates a PDN Connectivity Request (Request Type, APN) to the MME. The Request Type sets "Emergency" indicating the request PDN connenction is for emergency service. The APN is an Emergency APN.

- The MME allows the emergency PDN connection establishment according to the Emergency Configuration Data, and derives a PDN GW for the Emergency PDN connection.

- The MME sends Create Session request to S-GW/P-GW for the bearer establishment for the emergency PDN connection.

- MME sends PDN connectivity accept to the UE.

- The UE can initiate the IMS emergency registration and a detectable IMS emergency call via the emergency PDN connection.

6.x.3.2	non-UE detectable IMS emergency session

For non-UE detectable IMS emergency session, procedures of the UE normal attachment, PDN connectivity establishment for the IMS service, normal IMS registration can be reused. The non-UE detectable IMS emergency session establishment is the same as the procedure defined in clause 7.1.2 of TS 23.167 with the addition that:

In the case that the P-CSCF detects that this is a request to establish an emergency session, when the P-CSCF knows that the emergency session is over NB-IoT(GEO), the P-CSCF determines to continue the session over the normal IMS PDN connection instead of sending a rejection to the UE. This can be treated as an optimization for the non-UE detectable IMS emergency session over the NB-IoT(GEO) to avoid additional large delay caused by session rejection and reiniation.

6.x.4	Impacts on services, entities and interfaces

UE detectable IMS emergency session:

UE:

-	Support emergency attachment when using NB-IoT(GEO)

-	Request for emergency PDN connection for IMS ermgency session when using NB-IoT(GEO)

MME:

-	Determine whether to support IMS emergency service over NB-IoT(GEO) and provide Emergency Service Support indication to UE

-	Support emergency attachment when using NB-IoT(GEO)

-	Support emergency PDN connection for IMS ermgency session when using NB-IoT(GEO)

S-GW/P-GW:

-	Support emergency PDN connection for IMS ermgency session when using NB-IoT(GEO)

-

Non-UE detectable IMS emergency session

P-CSCF:

-	Determine to continue the IMS emergency session over the normal IMS PDN connection instead of sending a rejection to the UE

* * * End of Changes * * * *