SA WG2 Meeting #171	S2-2509082

13 – 17 October, 2025, Wuhan, CN

Source:	Xiaomi

Title:	Interim agreements for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC/Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for KI#1 of FS_5GSAT_Ph4-ARC.

- Discussion

Until SA2#170, 12 solutions were proposed and accepted for KI#1 in TR 23.700-19. This contribution tries to give summary and analysis to the solutions on KI#1, and interim conclusion is given based on the analysis.

Table 1: Solution summary

|  | CP | UP | CP+UP | IP | NIDD | IP+NIDD | One PDN connection |  | Two PDN connections |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 1 bearer | 2 bearers |  |
| Sol-1 |  | ● |  | ● |  |  | ● |  |  |
| Sol-2 |  | ● |  | ● |  |  |  | ● |  |
| Sol-3 |  | ● |  |  |  | ● |  | ● |  |
| Sol-4 |  | ● |  |  | ● |  |  |  | ● |
| Sol-5 | ● |  |  | ● | ● | ● |  | ● |  |
| Sol-6 | ● |  |  | ● |  |  |  | ● |  |
| Sol-7 |  |  | ● | ● |  |  | ● |  |  |
| Sol-8 |  |  | ● | ● |  |  |  | ● |  |
| Sol-9 |  | ● |  | ● |  |  |  | ● |  |
| Sol-10/11 |  |  | ● | ● | ● |  |  |  | ● |

- CP, UP or CP/UP combined solution:

Solution 5 and 6 propose CP based solution. IMS voice service has IMS signalling and IMS voice data flow. They have different QoS, reliability and delay requirements which need to be carried over two separate bearers. For CP based solution, establishing the two bearers needs two different SRBs which requires RAN to define a new SRB as per the existing mechanism. Another thing is the packet overhead of the CP based solution is larger than UP based solution based on the former analysis, which has less advantage of transmission delay under satellite access especially over the GEO satellite.

Solution 1,2,3,4 and 9 propose UP based solution. To meet the QoS requirement for the IMS voice service, two separate bearers are needed to carry the IMS signalling and IMS voice data respectively. For UP based solution, the procedures including the dedicated bearer establishment can be reused. However, there is a restriction that the maximum 2-DRB is allowed to be used in NB-IoT access per existing specification. If a DRB is occupied by other service before initiating the IMS voice service or if other service needs DRB while it is occupied by the IMS voice service, all these cases require to resolve the conflict (e.g. remove the restriction).

Solution 7,8 and 10/11 propose CP+UP based solution. Solution 7 proposes one default bearer using a SRB for both IMS signalling and IMS voice data, and can be switched to S1-U data path for voice data transmission if supported. The solution may not meet the IMS voice service requirement considering IMS signalling and voice data have different QoS requirements. Solution 8 proposes a default bearer over SRB is created for IMS signalling and a dedicated bearer over DRB is created for IMS voice data. The solution allows more than one service to be deployed under the maximum 2-DRBs restriction, however, supporting both CP and UP based bearers within an IMS PDN connection requires S-GW to maintain both S11-U path and S1-U path simultaneously during the IMS call and needs to identify the IMS signalling and IMS voice packet. Solution 10/11 proposes using two PDN connections for the IMS voices, this is an enhancement on the PDN connection for IMS voice service and requires both UE and network side to associate context of the two PDN connections.

Proposal 1: UP based solution is applied for the IMS voice service (including IMS signalling and voice packet) over NB-IoT(GEO)

- IP, non-IP, IP/NIDD combined solution:

6 solutions propose to use IP based solution, 1 solution proposes NIDD solution, 1 solution proposes IP+NIDD solution, solution 5 gives IP, NIDD and IP+NIDD based solutions, solution 10/11 give IP and NIDD based solutions.

Compared with NIDD(non-IP based)，IP based solution has larger packet overhead, but it can give a large optimization through RoHC for IP/UDP/RTP header. However, how to reduct RTP header in non-IP based solution is TBD.

Another thing is the IMS service relies on SIP for signalling exchange and the media streams are typically transmitted over IP/UDP/RTP. Both SIP signalling and voice media typically require IP address to interaction between two sides. If using the NIDD solution, some enhancements need to be supported for both UE and network side. For example, the UE should support NIDD encapsulate of voice packet and S-GW should transform the NIDD packet to the IP Packet. The supporting of SIP negotiation by UE without IP address should be enhanced.

Proposal 2: IP based solution is proposed for the IMS voice service over NB-IoT(GEO).

- One PDN connection or two PDN connections for IMS voice service:

2 solutions propose using one PDN connection with one default bearer for IMS voice service, 6 solutions propose using one PDN connection with one default bearer and one dedicated bearer for IMS voice service, 2 solutions propose using a pair of PDN connections for IMS voice service.

For one PDN connection with one default bearer to carry both IMS signalling and IMS voice data, it cannot provide QoS differentiation to the IMS service data flows. For supporting IMS service, the current mechanism is using a single PDN connection with an APN for IMS service. For using a pair of PDN connections, two APNs needs to be deployed for an IMS service, besides it requires both UE and network side to associate context of the two PDN connections.

Proposal 3:  A single PDN connection with two bearers is proposed for IMS voice service over NB-IoT(GEO)

2 	Proposal

It is proposed to capture the following interim agreements into TR 23.700-19 v1.0.0.

**** First Change  ****

7	Interim Agreements

7.1	Agreed Principles

7.1.1	Agreed principles for KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

To supports KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following principles are concluded:

Principle 1: The existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused to trigger UE-SAT-UE communication, select UPF(s) onboard satellite(s) and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

Principle 2: The following existing 5G VN features apply for KI#5:

-	A single SMF or a single SMF set is supported.

-	The UEs are in the same 5G VN group.

-	Local switch within UPF on-board satellite, N6 and N19 tunnel between UPFs on-board different satellites are supported.

-	Both IP PDU Session type and Ethernet PDU Session type are supported.

-	Only unicast UE-SAT-UE communication is supported.

Principle 3: The SMF determines whether to continue or to fall back the UE-SAT-UE communication due to serving satellite change, based on the information about the availability of ISL(s) between satellites serving the UEs as well as the 5G VN group supporting UE-SAT-UE communication. How SMF gets the information is per implementation, e.g. based on UPF report or from transport network via a proprietary interface.

Principle 4: PSA UPF on ground and UL CL/BP/local PSA UPF onboard satellite is supported as baseline. To support service continuity, the following apply:

-	For the UE-SAT-UE communication continuity case, the SMF reuses existing procedures for Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session as described in clause 4.3.5.7 of TS 23.502 [3] for the UE.

-	For the ground fallback case the SMF coordinates existing procedures at both UEs for Removal of additional PDU Session Anchor and Branching Point or UL CL as described in clause 4.3.5.5 of TS 23.502 [3]. The SMF firstly removes UL CL/L-UPF of peer UE and configure PDR&FAR and N6/N19 routing in ground PSA UPF(s) (if two UEs are using different PSA UPFs) during the satellite handover of local UE, then removes UL CL/L-UPF of local UE after satellite handover of local UE.

Principle 5: For PSA UPF on satellite, SSC mode 2 or 3 can be used to support service continuity. It is assumed that all the 5G VN group members are accessing NTN (no DN connectivity while UE-Sat-UE with PSA UPF).

7.1.1	Agreed principles for KI#1 of supporting of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Proposal 1: UP based solution is applied for the IMS voice service (including IMS signalling and voice packet) over NB-IoT(GEO)

Proposal 2: IP based solution is proposed for the IMS voice service over NB-IoT(GEO).

Proposal 3:  A single PDN connection with two bearers is proposed for IMS voice service over NB-IoT(GEO)

**** End of Changes ****