3GPP TSG-SA WG2 Meeting #171	S2-2509090

Wuhan, China, October 13 – 17, 2025

Source:	Apple

Title:	KI#2 conclusion principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates to interim agreements for KI#2.

# 1. Discussion

For KI#2 “Support of service adjustments for the UE”, interim agreements were discussed in SA2#170 and documented in S2-2507748. This contribution proposes change on top of the final version in SA2#170 specifically focussing on aspects of AF involvement and exposure of policy control decisions.

Solution #17 added a new IE “reason for policy change” that PCF provides an update to the PCC rule to SMF. SMF passes on this information to CHF. This is a useful information for CHF to be aware of the reason driving this PCF decision. In the similar way, any policy decisions due to energy saving actions in PCF is also important for an AF to know. This can help in coordinating with applications such that overall goal is aligned. Solution #26 proposed the AF subscription to be notified at the time or in advance of a policy change due to energy saving action.

Solutions #5 and #15 proposed the AF providing inputs for PCF to take policy decisions based on UE energy saving preferences. Similarly the solution #26 proposed that AF may intimate 5GC its wish to be temporarily excluded from any impending policy changes from energy saving actions. Such a request for exclusion may be sent irrespective of whether AF has a subscription to choose or provide preference for an energy saving policy that network may apply.

This contribution proposes updates to the agreed principles for KI#2. The changes proposed to the clauses 7.1.2 and 7.2.2 are highlighted in green.

# 2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

The interim agreements on principles for KI#2 are as follows:

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request and/or operator policy:

-	SM policy control.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	The PCF adds "reason for a policy change" in charging info of the associated PCC rule when service adjustment is applied for an SDF. SMF sends the reason for a policy change together with existing charging info to CHF.

-	The AF may indicate to the network the preferred or expected network energy saving behaviors per application. The PCF may perform energy saving SM policy control based on the AF provided service parameters (e.g. activate different UE subscribed energy saving service levels which lead to different service experiences).

-    The AF may subscribe to be notified for energy saving policy decisions made by PCF. The AF may also negotiate to receive an early notification when energy consumption exceeds a threshold lower than the value for triggering policy changes. The policy changes could be SM or AM policy changes. The PCF notification may include a time window for which the energy saving policy is applied.

-   The AF may indicate to 5GC a preference to be temporarily excluded from any energy saving policy change for a requested period of time.

* * * Next Change * * * *

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.2) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	The PCF may subscribe to notifications of the following energy related information from the EIF:

-	Energy Consumption information at UE, S-NSSAI (e.g. per UE per Network Slice), PDU Session and Service Data Flow (e.g. per UE per application) granularity.

-	renewable energy consumption ratio at UE, S-NSSAI (e.g. per UE per Network Slice), PDU Session and Service Data Flow (e.g. per UE per application) granularity.

-	The PCF may obtain the following energy related information from the OAM:

-	Energy Consumption information at Network Slice granularity.

-	renewable energy consumption ratio at Network Slice granularity.

Editor’s note:	Whether and how the renewable energy related information can be collected relies on SA WG5, so feedback from SA WG5 is needed to conclude renewable energy aspect.

-	The PCF may perform policy control taking into account energy related information from the EIF and/or OAM.

The following aspects are not pursued:

-	The PCF may generate PCC rules with energy related QoS parameters sets. The SMF may generate energy related QoS profile based on PCC rules with energy related QoS parameters sets or based on local policy, and provide the energy related QoS profile to the NG-RAN. NG-RAN selects the energy related QoS profile only if NG-RAN decides to downgrade the QoS due to energy saving. NG-RAN notifies the selected energy related QoS profile to SMF, SMF will further inform the selected energy related QoS profile to PCF and to UPF for charging purpose.

NOTE:	One or more energy related QoS Parameter Sets can be provided in a prioritized order.

Editor’s note:	Whether and how the energy related QoS profile can be used by NG-RAN for energy saving needs feedback from RAN WGs.

-	The PCF may make the following policy control decisions for network energy saving:

-	UE policy control, e.g. generating and updating URSP rules.

-

* * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

### 8.2 Conclusions for KI#2

Conclusions for this Key Issue are documented in clause 7.1.2

* * * End of Change * * * *