3GPP TSG-WG SA2 Meeting #171	S2-2509135

Wuhan, CN, 13th – 17th Oct, 2025

Source:	CEWiT

Title:	Interim Conclusion proposal for KI#2

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide a basis for future discussion, interim conclusions have been incorporated into Clause 7.

# 1.	Discussion

This contribution proposes the interim agreements for KI#2. The interim agreements are drafted based on the common points across all solutions proposed for KI#2 in 3GPP TR 23.700-14 V1.0.0.

The solutions #2, #3, #6, #8, #9, #10, #11, #12, #19, #27, #33, #36, #37 and #38 are proposed for KI#2. The following is a summary of all the solutions,

- Solution #2

The entities involved in the sensing procedure are UE and Sensing Support Function (SSF). The sensing service consumer is UE. The UE is authorized based on its subscription data which includes the dedicated S-NSSAI for Sensing Service. To revoke the authorization of UE's Sensing Service, the core network removes the dedicated S-NSSAI for Sensing Service from the UE subscription data.

- Solution #3

The entities involved in the sensing procedure authorization are AF, NEF/Gateway and Sensing Function (SF) (contains Control and Processing). The NEF authorizes the AF for usage of the Sensing Service. The Sensing Function may perform further authorization for the sensing request based on the parameters provided by the AF. If the Sensing Function performs authorization, it may indicate whether the request has been authorized or rejected in sensing service response.

- Solution #6

A Gateway sensing NF may be defined for sensing service authorization from sensing service. Upon receiving a request (via NEF), the Gateway sensing NF verifies whether the AF is authorized based on the AF's authorization information (e.g. based on service level agreement) and determines the target sensing service area(s) and the candidate sensing mode to support the requested sensing operation. If authorization is not granted, the Gateway sensing NF may send a response with reject cause to the consumer and if authorized and accepted, the sensing operation is handled by (distributed) sensing NF(s). When the authorized sensing service cannot be fulfilled by the sensing entities, the Gateway NF may revoke the authorization of the sensing service.

- Solution #8

In this solution, Sensing Control Function (SCF) will perform authorization for the AF requesting sensing service based on the sensing entities' capabilities and the sensing service context. For the request from untrusted AF, the NEF initially authorizes the AF request of the sensing service and forwards the request to SCF. For sensing service revocation, when the AF determines to stop the specific sensing service due to some reason, e.g. saving resource, the AF requests the SCF to cancel the relevant sensing service, which leads to SCF release the sensing resource, including the sensing entity.

- Solution #9

NEF performs authorization AF (i.e. in case of untrusted AF. A new NF called Sensing Service Function (SSF) authorizes the sensing service request based on the AF authorization data present in another new NF called Sensing Profile Function (SPF). The AF authorization data may contain AF ID, allowed area, allowed time, etc. The discovery of sensing entities and configuring sensing parameters to trigger sensing operations implicitly authorize the sensing entities. For revocation explicit request can be triggered by AF to terminate. Similarly, network can revoke when sensing operation is finished or some AF authorization is modified and notify the revocation status to AF.

- Solution #10

A new network function called Sensing Gateway is introduced to perform the authorization of sensing services requests based on the override information and sensing authorization profile for the sensing area. The authorization is supported via subscription and via sensing authorization profile handling. The authorization profile is stored in Sensing Authorization Information Function. The sensing authorization profile includes parameters such as the Sensing Authorization Indication (sensing allowed or not allowed), Sensing Notification Indication, valid time, allowed area and sensing accuracy. Authorization Override Indicator (AOI) is included in  request to determine whether the authorization profile can be overridden by sensing service request. If the AOI indicates "Override", the authorization step is skipped.

- Solution #11

In this solution, there are two types of Authorization involved in sensing service: i) Initial Authorization and ii) Optionally, Re-authorization. For the Initial Authorization, NEF authorizes the sensing service consumer. The SF performs the authorization for the sensing service request, by checking the conditions with NF (e.g. UDR, or CCF) configured with service profile. For re-authorization, the SF may re-select the Sensing Entity(ies) by checking the sensing service specific information.

SF may revoke the ongoing sensing service operation when the conditions are changed, and the authorization criteria are no longer met. Triggers for revocation of the on-going sensing service could be from 3rd party AF, or internal NF (e.g. SF, etc.), due to specific condition(s) are met, e.g. some regulation reasons, the Sensing Service consumer is not authorized to request the Sensing Service, or the sensing target area is not allowed to provide Sensing Service, etc.

- Solution #12

The NEF initially authorizes the AF request. The Sensing Control Function authorizes the AF request received from the NEF and stores the information in the Sensing Data Repository. SCF sends authorization requests to sensing entities to obtain the authorization information for the mapped entities. If the authorization is revoked, the service specific parameters present in SDR are deleted and NEF removes the service specific parameters from other participating NFs.

- Solution #19

In this solution, the Sensing Service Consumer (SSC) provides the sensing service request (SSR) to Sensing Management Function (SeMF) (via exposure function in case of 3rd party application as SSC). The SeMF forwards the SSR for validation and authorization to a Sensing Policy and Authorization Function (SPAF). The authorization functionality at SPAF checks if the SSR is allowed to be performed based on the request parameters, e.g. sensing mode, sensing location, sensing time, etc.

- Solution #27

For the untrusted AF case, the NEF may perform AF authorization. The SF performs Sensing service request authorization and may also perform necessary validation checks. The SF shall perform authorization validations depending on the type of the sensing service request, the target sensing area, the target objects sensing. The authorization and validation checks may be performed based on the provisioned profiles and policies, that indicate whether to allow or disallow a sensing service or what information can be exposed in sensing result.

- Solution #33

The gateway sensing NF may authorize the sensing service request from the AF.

- Solution #36

In this solution, the sensing service revocation is based on the triggers like i) Sensing in Non-allowed Area, ii) Sensing Entity request Sensing Function to pause/revoke the sensing task , iii) Sensing Function determines to pause/revoke the sensing service or reselect Sensing Entity. SF sends sensing revoke indication to the sensing RAN node and optionally sends revoke indication to AF/NEF.

- Solution #37

In this solution, Sensing Service authorization consists of two steps i) authorization of the AF performed by the NEF and ii) authorization of sensing service request performed by the Sensing Control Function based on the AF identity and request parameters along with the sensing authorization information. The sensing authorization information can contain allowed Sensing Service Area, allowed Sensing Service type etc. The Sensing Control Function notifies the AF that the authorization has been revoked and cancels all relevant sensing procedures related to the sensing service request whose authorization has been revoked.

- Solution #38

Authorization of the sensing service can be specifically supported by sensing authorization profile, which is managed by the SDM. The authorization profile of the sensing area specifies the authorization information applicable to the area. Based on the authorization information, a sensing request can be evaluated whether to be permitted or not.

Based on above provided summary of the solutions, we propose the following as interim agreements for KI#2:

- The authorization can be of two steps where NEF performing authorization of 3rd party AF and sensing NF authorizing the sensing service request

- The sensing service request authorization can be based on an authorization data which can be locally configured in sensing NF or retrieved from a storage entity.

- The authorization data can be based on sensing service area and includes parameters like allowed AF, allowed service types, etc.

- Based on authorization, sensing NF accepts or rejects the sensing request.

- Based on the triggers for revocation of an ongoing sensing request, sensing NF initiates revocation request to Sensing Entity(ies) to terminate the sensing service operations and notifies the same to NEF/AF.

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

## 7.1	Agreed Principles

7.1.A	KI#2

The following are the interim agreements for KI#2:

The authorization of sensing service involves two steps:

- NEF authorizing the 3rd party AF

NEF performs authorization for AF (i.e. in case of 3rd party AF) before accepting any sensing service request from AF. This is based on the existing procedure as per TS 23.502 [10].

- SeMF authorizing sensing service request

The SeMF receives the sensing service request from NEF/AF and performs the authorization of the sensing service request. Authorization is performed using the authorization data, which can be either pre-configured at SeMF or retrieved by SeMF from a storage entity e.g. UDR.

The authorization data can be based on sensing service area. The authorization data can include

- Allowed AF Ids

- Allowed service type(s),

- Allowed time duration

There can be other relevant parameters present in authorization data.

Depending on the type of service, such as regulatory services, the SeMF can decide to skip the authorization.

Based on the authorization outcome, whether successful or unsuccessful, the SeMF sends success or failure response to NEF/AF respectively.

The SeMF is responsible for revocation of the ongoing sensing service. On receiving a sensing revocation trigger, the SeMF initiates revocation request to Sensing Entity(ies) to terminate the sensing service operations and notifies the same to NEF/AF.

The triggers for revocation can be from AF/NEF, SeMF and gNB(s), for reasons such as non-allowed areas, load conditions, regulatory purposes, authorization profile update, etc.

# * * * * End of Changes * * * *