3GPP TSG-SA2 Meeting #172	S2-2509971

17 - 21 November, 2025, Dallas, USA                            (was S2-2508364)

Source:	Nokia, TurkTelekom, Lenovo, Turkcell

Title:	Proposed agreement of principles for Key Issues 5

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#5

# 1.	Introduction

This document discusses aspects related to several KIs and proposes principles for conclusions by considering the scope, architectural assumptions and the solutions

## 1.1 Discussion for KI#5:

Exposure of sensing results to the SC(AF) is provided by the SF/NEF based on the communication path the sensing request has reached the core network (with or without NEF) from the AF. Using the existing mechanisms of exposure framework, the core network provides the sensing results to the AF.

Proposal for KI#5:

Following content for KI#5 is proposed to be captured as principles for agreement.

- SF provides the sensing results to the NEF and then NEF using the exposure framework provides the sensing results to the AF.

- The trigger and preparation of sensing results are based on the agreed type of a sensing service request received from the AF, i.e., one time, event-based or subscription-based.

- Sensing request from the SC includes sensing service type, sensing target area, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS), sensing results configuration (one-time, event-based, subscription-based).

Note: The parameters that can be considered between the SC and the SF will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

## 1.2 Merge of documents from SA2#171 for KI#5:

This section merges the content of the proposals from various companies listed in the table below.

|  | Interim agreement for KI#5 |
| --- | --- |
| S2-2508364 | 23.700-14: Proposed agreement of principles for Key Issues 5. |
| S2-2508240 | 23.700-14: KI#5: Conclusion. |
| S2-2508281 | 23.700-14: Proposal on Interim agreements for KI#5. |
| S2-2508286 | 23.700-14: KI#5: Interim agreements. |
| S2-2508467 | 23.700-14: KI#5 Interim Agreements . |
| S2-2508604 | 23.700-14: Interim Agreement Proposals for FS_Sensing_ARC. |
| S2-2508768 | 23.700-14: 23.700-14: Interim Agreements for Key Issue#5: Sensing Result Exposure. |
| S2-2508816 | 23.700-14: Proposal for Interim Agreements on KI#5. |
| S2-2508832 | 23.700-14: KI#5 Sensing Result Exposure, Interim conclusion proposal. |
| S2-2508858 | 23.700-14: Interim agreement for KI#5. |
| S2-2509148 | 23.700-14: Evaluations and proposed Interim conclusion for KI#5. |
| S2-2509139 | 23.700-14: Interim Conclusion proposal for KI#5. |

Proposed agreeable principles for KI#5 from merged version from SA2#171 document

- SF generates, optionally stores and provides the processed sensing results, including aggregated, pre-processed, or AI/ML-assisted data to the NEF and then NEF using the exposure framework or data connection provides the sensing results (Object Presence Detection, Sensed Object count, Sensed Object definition, Sensing Contextual information including Sensing QoS feedback) to the AF and for trusted AF the SF communicates to the AF directly as currently.

NOTE X: It is assumed that a sensing service requester and a sensing service consumer are the same entity.

Note X: If the functional split is used, it is an option, that the SCF configures the SPF to deliver the result to the authorized sensing service consumer (AF) directly.

- The trigger and preparation of sensing results are based on the agreed type of a sensing service request received from the AF, i.e., one time, event-based or subscription-based. The event trigger may include object detection, object in/out of a defined area, velocity threshold, path deviation.

NOTE X: The conditions for event triggered sensing report are to be specified during normative phase.

- Sensing request from the SC includes AF Identifier (optionally including its address), sensing service type, sensing target area, object definition, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS), sensing results configuration (one-time, event-based, subscription-based), Sensing Result Detail Level, Notify result with associated information.

- The results exposed can permit for the Application Function to request a subsequent sensing service (e.g., tracking) associated to at least one object that is contained in a Sensing Result.

- The gateway sensing function exposes the sensing result to NEF/AF.

- When a sensing service is authorized, it is assigned a Sensing Operation ID.

NOTE X: The parameters that can be considered between the SC and the SF will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

NOTE X: Privacy and security aspects related to sensing result exposure shall be coordinated with SA3.

NOTE X: Charging and OAM implications of exposing sensing results shall be coordinated with SA5.

Topics for further consideration for KI#5

Topics for further consideration for KI#5 are as below:

-	How to define the accuracy level requirements of sensing results, including confidence levels and methods for processing.

-	Whether and what specific types of contextual information are included with sensing results.

-	Sensing result determination by NG-RAN nodes: Whether NG-RAN nodes can perform a level of processing required to determine a sensing result (e.g. to detect an object) is RAN WGs decision and requires further coordination.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.5.1	Agreed Principles for KI#5

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#5:

- SF generates, optionally stores and provides the sensing results to the NEF and then NEF using the exposure framework provides the sensing results to the AF, and for trusted AF the SF communicates to the AF directly as currently.

- The sensing result may contain the Object Presence Detection, Sensed Object count, Sensed Object definition, Sensing Object location, Sensing Object ID, Sensing Contextual information including Sensing confidence level, Sensing QoS information, Sensing timestamp.

NOTE X: It is assumed that a sensing service requester and a sensing service consumer is the same entity.

- The trigger and preparation of sensing results are based on the agreed type of a sensing service request received from the AF, i.e., one-time, event-based or subscription-based. (Event trigger may be configured based on conditions such as object detection, object in/out of a defined area, or exceeding a velocity threshold).

- Additionally, the trigger to pause or resume sensing results are based on the agreed type of sensing service request and the authorization data. Based on the events and sensing service request, sensing activities could be paused or resumed; for example, when an objected is detected (for detection request) or in not allowed area or not allowed time action could be taken to pause further activities and resume when the objected is in allowed or in allowed time.

NOTE X: The conditions for event triggered sensing report are to be specified during normative phase.

- Sensing request from the SC includes AF Identifier (optionally address), sensing service type, sensing target area, object definition, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS), sensing results configuration (one-time, event-based, subscription-based), Sensing Result Detail Level, Notify result with associated information.

NOTE X: The results exposed can permit for the Application Function to request a subsequent sensing service (e.g., tracking) associated to at least one object that is contained in a Sensing Result.

NOTE X: The parameters that can be considered between the SC and the SF related to sensing request and sensing data report will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

NOTE X: Privacy and security aspects related to sensing result exposure shall be coordinated with SA3.

NOTE X: Charging and OAM implications of exposing sensing results shall be coordinated with SA5.

# * * * * Next change (all text new) * * * *

## 7.2	Topics for further consideration

### 7.2.5	Topics for further consideration for KI#5

Topics for further consideration for KI#5 are as below:

-	How to define the accuracy level requirements of sensing results, including confidence levels and methods for processing.

-	Whether and what specific types of contextual information are included with sensing results.

-	Whether a data connection would be required for exposure between the SF and the AF is to be determined.

-	Sensing result determination by NG-RAN nodes: Whether NG-RAN nodes can perform a level of processing required to determine a sensing result (e.g. to detect an object) is RAN WGs decision and requires further coordination.

# * * * * End of change * * * *