3GPP TSG-SA2 Meeting #172	S2-2509973

17 - 21 November, 2025, Dallas, USA                            (was )

Source:	Nokia

Title:	Proposed agreement of principles for Key Issues 1

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#1

# 1.	Introduction

The Sensing Function (SF) implements receiving of the sensing request as configurations from the AF (Sensing Client - SC) with or without NEF and providing the corresponding translated configurations to the Sensing Entity (SE); SF is responsible for collecting the sensing data from the SE and provide the generated sensing result to the SC for the associated request.

Based on the agreed scope and architectural assumptions, the considered use case for Release 20 5GA is for detection and tracking of aerial objects (e.g., drone) which could be realized on re-using of the existing eco system (e.g., BS) and its interface with the Core Network. There is no motivation or a need for SA2 to bring in major architectural changes and requirements on gNB in this stage of 5GA (where a new generation work has started) for receiving the configurations from SF and communicating the sensing data to the SF. For the use case described, existing mechanisms which are proven already in features likes positioning are proposed with limited changes. Bringing major architectural changes would bring complexities and challenges to commercialize 5GA Sensing, which involves bringing additional requirement on NG-RAN to interface to the CN, increased interoperability testing requirements, additional resources.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: AMF acts as transport for sensing activities i.e., for relaying the sensing configurations to the SE from the SF. AMF does not play any active role apart from transporting configurations between gNB and SF.

# * * * * End of change * * * *