3GPP TSG-SA2 Meeting #172	S2-2509981

# Dallas, US, 17 – 21 November 2025		(was S2-xxxxxxx)

Source: 	Transsion Holdings

Title: 	Discussion on the necessity to store AI model training data in CN and update to KI#1 Interim agreements

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes update to interim agreements for KI#1: Transfer of data over UP for UE data collection.

1	Discussion

This paper addresses the nature of data which is used for UE-side model training, and discusses the principles on how the network and UE efficiently collect the data by taking advantage of the nature of data for model training.

1.1 Nature of data used for model training

The nature of AI model training is to extract patterns from huge amount volume of data over a certain probability distribution through mathematical algorithms such as neural networks and decision trees, forming generalized model(s). In order to get the right pattern(s) to form correct generalized model(s), the collected data for model training should have the following characteristics,

- Large-Scale (Huge amount volume)

A larger volume of data generally allows models to learn more complex patterns, finer nuances, and reduces overfitting, thereby improving generalization capabilities. Usually models, such as deep learning models possess a strong "data absorption capacity" – often, the more data, the better the performance.

- Diverse (Source domain, Time domain, etc.)

Biased data can lead to biased AI models. If the training data contains unfair or unrepresentative samples, the AI model may inherit these biases and make unfair decisions. Ensuring diverse and unbiased data is critical for having AI with high levels of performance.

Observation 1: The best way to collect diverse data by 5GC is to collect the data from as many as possible huge amount of UEs randomly and at different time.

1.2 How do UEs collect the data effectively and efficiently

Almost all of the UE-side model training data is generated by UE during the UE is in connected mode, i,e, there exists traffic on the air interface. Compared to UE collecting data for the only purpose of collecting data, it is more efficient that UE collects data while there exists traffic already. It would be better for UE to log the UE-side model training data during the traffic in advance or UE collects the data when UE has traffic after UE gets the request from the network with hours or days delay. If there is no traffic in due time or logged data, UE may reject the request from the network.

Actually, there is no latency requirement for all model training data collection.

Observation 2: It would be efficient for UE to log the UE-side model training data in advance or UE collects the data when UE has traffic after UE gets the request from the network with hours or days delay. If there is no traffic in due time or logged data, UE may reject the request from the network.

1.3 How to improve system efficiency of collecting/using the data

Different consumers may request the same data for model training, if network has the historical collected data, network could provide the data immediately and avoid collecting the data from UE repeatly. Considering the numbers of consumer, the efficiency of collecting data could increase dramatically by storing the historical collected data in the network for reusing.

- Observation 3: Storing the collected data in CN can avoid repeatedly collecting the same data since the same data may be requested by different consumers, in other words, by storing the collected data, data collecting efficiency can be improved significantly. Otherwise wasting network resources is foreseen.

Proposal 1: To introduce/reuse repository functionality on storing the collected data in the core network in order to reuse the collected data.

Based on the discussion above, some interim principles are proposed to update KI#1: Transfer of data over UP for UE data collection.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	Repository functionality is used to store the collected UE data in the core network.

End of Changes