3GPP TSG-SA WG2 Meeting #172	S2-2510151

Dallas, USA, 17 – 21 November 2025

Source:	ETRI

Title:	Interim agreements on KI#6

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes agreed principles for KI#6 by evaluating the relevant solutions.

# 1. Discussion

In TR 23.700-14 v1.1.0, the key issue #6 studies how parameter configuration for Sensing Entities is performed, in particular:

-	How and what parameters need to be exchanged between the Sensing Entities and the Core Network.

A. Common priciples

In the TR, there are 6 solutions which address KI#6: solution #3, #15, #20, #27, #34 and #35. By evaluating those solutions, the following common principles for KI#6 are identified:

- (SF-SE Interactions) Sensing Function (SF) interacts with one or more Sensing Entities (SEs) to provide configuration parameters.

- (SE operations) Based on the configuration parameters provided by SF, the SE performs sensing operations including sensing data measurements and report.

- (Configuration Parameters) The configuration parameters for SE may include the following categories: sensing type, sensing target, performance, report of measured sensing data, and assistance information

- (Direct vs. Indirect) The interaction between SF and SEs for configuration can be performed directly or indirectly with involvement of AMF. (NOTE: it may depend on the KI#1 decision)

B. Priciples for Further Considerations

The following priciples are also identified for further considerations:

- (Start w/Configuration) The interaction between SF and SE for configuration may collocate with a request for the SE to start its sensing operations.

- (Parameters Derived) Configuration parameters may be derived from the Sensing service request from the consumer (AF), or determined by the SF based on operator’s policies.

C. Configuration Parameter Details

Beyond the 6 solutions which explicitly indicate to address KI#6, there are additional solutions which specify detailed parameters for SE configurations. Some other solutions also provide detailed parameters for the service requirement/configuration provided by AF to SF or provided by SF to SE, from which the configuration parameters for SE may be derived.

The following table provides a non-exhaustive summary of the detailed parameters per solution for request/configuration to SF or SE. These parameters remain subject to further discussion during the normative work.

| Sol # | Requirement/Configuration to SF | Requirement/Configuration to SE |
| --- | --- | --- |
| 1 |  | how to report data (e.g. the periodicity, or the criteria for an event report) |
| 2 | sensing service type (object detection, object tracking, environment sensing, etc) and reporting mode (e.g. one time, periodic or event triggered). |  |
| 4 | sensing type (e.g. object detection, tracking, environment monitoring, etc.), target (e.g. object, area), required service-specific performance (e.g. accuracy, resolution, latency) and potentially assistance information (e.g. object/area details) |  |
| 6 | sensing service type (object detection, object tracking, environment sensing, etc.), sensing service requirements (e.g. accuracy, latency, resolution, etc.) and time information when the sensing service is needed (e.g. time for sensing measurement, time for sensing report) |  |
| 7 |  | type of sensing to perform (e.g. waveform or measurement type), duration or interval, area or target identifiers, involved UEs (if any) and reporting criteria |
| 8 | target sensing service area, sensing service requirement and sensing operation parameters. Sensing service requirement may include sensing accuracy, sensing resolution, confidence level, missed detection probability, false alarm probability, max sensing service latency, refreshing rate etc. Sensing operation parameters may include on-demand report/periodical report/event-triggered report. |  |
| 9 | Type of sensing service operation (e.g. objection detection and tracking, environment monitoring, motion monitoring, AF ID, target area, sensing duration, sensing KPI) |  |
| 12 | Service Description, Target Area, Tracked Object Characteristics, Expected Service Time, Subscription to events | response time, reporting frequency and duration, Quality of Service (QoS) Class details e.g. best-effort, sensing accuracy for e.g. horizontal or vertical accuracy, possibility of false detection to be in a certain range, etc |
| 13 | Target sensing area, Sensing QoS, Sensing service type (e.g. motion monitoring, object detection and tracking). |  |
| 16 |  | sensing service ID, sensing area, object description, sensing KPIs, target reporting information (e.g. CP or UP reporting and the target address) for the sensing data |
| 26 | Sensing service requirement parameters (e.g. Sensing Service Type, Target Sensing Service Area, one-time/periodical/event-triggered Sensing result report mode). | configuration information (e.g. the IP address of the SPF, the Sensing Task ID) |
| 27 | Sensing service type, Sensing target area, Sensing time parameter, Sensing contextual information, Requested Sensing QoS parameters, Sensing result configuration |  |
| 28 | AF Identifier, External Target Sensing Area information (e.g. geographical area information and, optionally, altitude information), Type of sensing to perform (e.g. SensingEventType = "Object Detection", "Object Tracking"), sensing result detail level (e.g. "notify result only", "notify result with associated information"), event reporting mode (e.g. one-time, event-triggered or periodic reporting), sensing result exposure parameters (e.g. time and/or spatial resolution of the determined sensing result). |  |
| 29 | sensing type, sensing area, sensing reporting frequency |  |
| 30 | Sensing type, Target, Sensing Quality, Reporting mode, Optional sensing assistance information |  |
| 32 | AF ID., Sensing Correlation ID, Target Sensing Area, Sensing Service Type(s), Sensing Time Window(s)., Object Definition, Type of Event Reporting, Quality Indication of Sensing Result, Sensing Result Response Time |  |
| 33 | sensing service type, sensing service requirements (e.g. accuracy, latency, resolution, etc), sensing report type, sensing report start time, sensing report ending time and sensing report triggering condition | the time for sensing operation, any configuration information for sensing operation and a sensing operation ID |
| 34 | Sensing service type, Sensing object type, Target Sensing service area, Sensing report condition, Sensing time condition, Sensing service QoS, data transport-related information | Service parameters from AF (e.g. Sensing object type, Target Sensing service area, Sensing service QoS). Parameters configured by SF (e.g. RCS (Radar Cross Section) range list, object velocity range.) |
| 35 | type of sensing service (e.g. objection detection and tracking, motion monitoring), target area, sensing duration, sensing KPI, SSF ID |  |
| 36 | sensing task ID, target sensing area, sensing requirement, sensing exposure requirement | sensing task ID, sensing mode, sensing requirement, Sensing Non-Allowed Area information |
| 37 | Sensing Service requirement(s) (e.g. Target Sensing Service Area, accuracy, resolution, refresh rate, Sensing Service time duration, one-time report/periodical report/event-triggered, etc), Sensing Service type and corresponding information to obtain required Sensing Result from network |  |
| 39 |  | Configurations related to the sensing request (e.g. target sensing area, required sensing accuracy, vertical sensing resolution, horizontal sensing resolution, choice of sensing mode, object characteristics e.g. size, range of velocity of target, etc), Frequency of collection of sensing measurements/sensing reports/data/parameters |

# 2. Proposal

Based on the discussion, it is proposed to include the following changes to TR 23.700-14.

* * * Start of Change * * * *

7.1.6	Agreed Principles for KI#6

The following principles are agreed to address KI#6:

- Sensing Function (SF) interacts with one or more Sensing Entities (SEs) to provide configuration parameters.

- Based on the configuration parameters provided by SF, the SE performs sensing operations including sensing data measurements and report.

- The configuration parameters for SE may include the following categories: sensing type, sensing target, performance, report of measured sensing data, and assistance information.

- The interaction between SF and SEs for configuration can be performed directly or indirectly with involvement of AMF.

NOTE:	(c) - The details of configuration parameters need to be captured from the solutions for further discussion during the normative phase.

Editor’s note:	(d) - This is an architectural aspect and will be aligned with KI#1 conclusions.

* * * Next Change * * * *

7.2.6	Topics for further consideration for KI#6

The following principles are for further considerations for KI#6:

- The interaction between SF and SE for configuration may collocate with a request for the SE to start its sensing operations.

- Configuration parameters may be derived from the Sensing service request from the consumer (AF), or determined by the SF based on operator’s policies.

* * * End of Change * * * *