SA WG2 Meeting #172	S2-2510233

17 – 21 November, 2025, Dallas, TX, USA

Source:	Ericsson

Title:	Interim Agreement and Conclusion on KI#2 IMS enhancement for GEO NB-IoT NTN access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution discusses the consideration of two Solution Evaluation Criteria, and Solution classification when assessing the proposed solutions for KI#2 – IMS enhancement in GEO NB-IoT NTN access. Furthermore, it proposes new text to be added to the interim agreement and conclusion section.

# Discussion

As of SA2#172, there are 12 proposed solutions that address KI#2 – IMS enhancement for GEO NB-IoT NTN access with focus on reducing the CST. These solutions encompass a wide range of SIP signaling combinations, which makes their assessment quite complex. To aid this evaluation, it is recommended to consider the following aspects:

Evaluation Criterion 1: Ensure the solutions under evaluation avoid the “ghost ringing” risks (see definition below).

Reason: When terrestrial users experience the ghost ringing, there is a high likelihood they will hang up, leading to a waste of GEO satellite resources and to negative impacts on specific operators’ KPIs. The use of preconditions was specifically added in SIP to avoid the “ghost ringing” risk. Some of the proposed solutions eliminate the use of precondition to reduce the number of SIP messages exchanged over the satellite UNI. Moreover, the long SIP Message UNI propagation delay amplifies the “ghost ringing” risks.

Ghost Ringing - Experienced by the call recipient, who, after answering the call (“Hello…”), does not receive any answer from the calling side and thinks there is no one at the other end of the call.

Evaluation Criterion 2: When evaluating solutions consider the trade-off between implementation/ deployment complexity and the relative reduction in call setup time.

Reason: The solutions providing the shortest CST, assuming they meet Criterion 1, may demand UE and network implementation / deployment impacts which may not be justified by the CST reduction improvement compared with other solutions having lesser complexity.  There are two key related aspects to consider:

-	Complexity of Implementing and deploying additional B2BUAs in the networks – Developing the B2BUA functionality may quickly become complex when considering all possible race conditions, forking, mid-call events and service interactions. Moreover, many commercial networks have already deployed B2BUAs for other purposes (e.g. provide various services or to handle network deployment aspects). Managing the interaction between multiple B2BUAs may require deployment specific adaptations, which will increase overall solution complexity in live networks.

-	Complexity of implementing new SIP states and SIP state transitions in the UE and P-CSCF: Preserving the 3GPP SIP states and SIP state transition which are already implemented in existing devices / P-CSCFs will simplify the UE/P-CSCF implementations by maximizing the re-use of existing code. Moreover, this should help reduce any interoperability problems between UE and P-CSCFs when using the new GEO Satellite access.

Proposed solutions classification: To help in the solution selection we propose to classify them in two categories:

-	Using text encoding of SIP – all solutions in this category require the use of a B2BUA to allow for reduction in the number of messages sent over the service link

-	Using binary encoding of SIP – in this category one solution does not require a B2BUA since the binary encoding alone ensures compliance with CST requirements. Other solutions, however, propose the addition of the B2BUA in other to get further CST reductions.

# Proposal

It is proposed to consider the Evaluation Criterion 1 aspects as input for the proposed statements in the Interim Agreement Section

Additionally, to achieve the optimal balance between CST reduction and solution/deployment complexity while considering the Evaluation Criterion 2 and the Proposed Solution classification aspects, it is proposed to avoid solutions combining binary encoding of SIP with a B2BUA.  In other words, we should select either a text-based encoding solution or a solution using binary encoding of SIP without a B2BUA.

As result we propose the addition of the following text in 23.700-19.

* * * Start of 1st change * * * *

# 7	Interim Agreements

## 7.1	Agreed Principles

-	The selected solutions must preserve the key terrestrial user telephony expectations that contribute to a successful call establishment (i.e. absence of ghost ringing).

* * * End of 1st change * * * *

* * * Start of 2nd  change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Interim conclusions for Key Issue #2

The following interim conclusions for KI#2 (IMS enhancement for GEO NB-IoT NTN access) are made:

-	to achieve the optimal balance between CST reduction and solution/deployment complexity, it is concluded to only select solutions that are either based on text-encoding of SIP with B2BUA, or on binary-encoding of SIP without a B2BUA.

-	only one solution shall be chosen for the normative work: either a solution based on text-encoding of SIP with B2BUA, or a solution using binary-encoding of SIP without a B2BUA.

* * * End of changes * * * *