3GPP TSG-SA2 Meeting #172	S2-2510290

Dallas, USA, November 17th -21st 2025

Source:	Intel

Title:	KI#4, Update interim agreement

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Interim conclusion for KI#4 – Sensing Data and the Associated Information Collection and Transport is proposed.

# 1. Introduction

This pCR proposes the interim conclusion agreement update for KI#4 – Sensing Data and the Associated Information Collection and Transport. It is proposed to include the following Agreement principles in TR 23.700-14.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all text new)* * * *

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-  The configuration parameters sent from the SCF to the selected SE(s) determine whether dedicated data tunnel will be established between the SE and the SPF for sensing data transfer.

- If the configuration parameters include the SPF endpoint information (e.g. SPF ID/ FQDN), the SE will initiate the data tunnel with the SPF to transfer the sensing data requested.

- If the configuration parameters does not include the SPF end point information, the SE will send the sensing data via the AMF to the SPF.

NOTE: The details on the protocol for data tunnel between SE and the new network function in the core network is to be determined in coordination with the RAN3 WGs.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

# * * * * End of changes * * * *