3GPP TSG-WG SA2 Meeting #172	S2-2510366

17-21 November, 2025, Dallas , US	was S2-2509407, 8880

Source:	China Mobile

Title:	[KI#2, New Solution] DO-A Capable AIoT Device Registration Procedure

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Solution for Registration Procedure for the DO-A Capable AIoT device.

1. Introduction/Discussion

In the WT in Study on Architecture support of FS_AmbientIoT_Ph2, the following is the description for the registration procedure for a DO-A capable AIoT device:

WT#2: Study the support of DO-A Capable AIoT Devices, including: - Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure). - Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF. - Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF. NOTE 4: topology 2 aspect of WT#2 has dependency on WT#1.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-30.

# * * * * First change (all new text) * * * *

6.X	Solution #X: DO-A capable AIoT Device Registration Procedure

6.X.0	High-level solution Principles

DO-A capable AIoT devices possess the capability to transmit MO signalling and data. Furthermore, the AIoT device actively registers with the core network to notify it of its presence autonomously.

6.X.1	Description

This solution addresses KI#2, specifically concerning the method by which the DO-A capable AIoT device can autonomously notify the network of its presence for Topology 1.

In this solution, it is assumed:

The AIoT device actively initiates the registration process with the network. The registration types include:

- initial registration.

- mobility registration update.

- Periodic registration.

The AIoT device initiates initial registration when the device power on or its condifuration changes. The AIoT device maintains a local timer configurred and registers to the network periodcally. Mobility registration is triggered when the AIotT device notices the AIoT Registration Area changes.

NOTE:	Mobility registration needs coordination with RAN WG.

ADM stores the DO-A capable AIoT device profile information and the status of the device.

AIOTF handles NAS termination and device authentication for DO-A capable AIoT devices and manages registration and mobility for AIoT Devices.

When a registraion request is received by NG-RAN, ir selectes the serving AIOTF as follows:

-	If the registration request is initial registration, the NG-RAN selecets an AIOTF based on local configuration;

-	If the request contains a tempeory AIoT device ID, the NG-RAN selects the AIOTF based on the AIoTF information in the temporary AIoT device ID. The tempeory AIoT device ID is allocated by AIOTF when the initial registration is successful, which is sent to the AIoT device for storage and further usage during the registration lifetime.

After successful registration the serving AIOTF notifies the ADM about the successful registration event of the AIoT device and its own information. The ADM changes the status of the AIoT device and stores the serving AIOTF information.

If the stored AIOTF changes, the ADM initiates a procedure to old serving AIoTF to release the contexts in the old AIoTF.

6.X.2	Procedures

11. ADM stores the serving AIOTF

12. ADM initiates context release procedure with the old serving AIOTF

8. AIoT device replies with registration complete

9.  AMF forwards the registration complete to AIOTF

10.  AIOTF notifies the ADM of registration status

7.  AIOTF responses the Registration accept to the AIoT Device

6.  AIoT Device Authentication and Authorization

4.  gNB forwards the registration to the serving AIOTF

5. AIOTF fetches context information from the previous serving AIOTF

0.AIoT Device profile provisioning in ADM

2. AIoT Device initiates registration procedure

1.AIoT Device meets the trigger conditions

AIoT Device （DO-A）

AMF

ADM

New AIOTF

gNB

Old AIOTF

3.gNB discovers the serving AIOTF

Figure 6.X.2.1: DO-A capable AIoT Device Registration Procedure

- The DO-A capable AIoT Device’s subscription data is provisioned in the ADM

Editor’s Note: The subscription data of a DO-A capable AIoT device is FFS.

- The DO-A capable AIoT Device meets the following trigger conditions, then it will send a NAS Registration Request to the core network.

The AIoT Device performs an Initial Registration if it is not registered with the network.

The AIoT Device performs Mobility Registration if it has moved outside of its AIoT Registration Area.

The AIoT Device performs periodic registration if it has registered on the network and remains in its AIoT Registration Area beyond the designated time period.

Editor’s Note: The details of the AIoT Registration Area need to align with the RAN WG

2.	AIoT Device sends a message to NG-RAN, including a NAS Registration Request (Registration Type, Temporary ID, device information, security parameters). Registration can be initial registration, mobility registration, or periodic registration. The AIoT device ID, a permanent AIoT device ID or a temporary AIoT device ID if available, is included in the request.Then, NG-RAN sends the Registration Request to the selected AMF.

3.  The gNB discovers the serving AIOTF.

Editor’s Note: The details of how the gNB discover the serving AIOTF are FFS.

4.  The gNB forwards the registration request to the serving AIOTF by either direct connection or indirect connection via the AMF.

5.	[Conditional] The serving AIOTF determines the old AIOTF using the temporary AIoT device ID and retrieves the AIoT Device context from the old AIOTF.

6.	The AIOTF performs the Authentication/Security procedure and stores the security context.

Editor’s Note: Details of authentication/security procedures will be determined by the SA3 WG.

7.	The AIOTF responds the Registration Accept to the AIoT device , including a new temporary AIoT device ID assigned by the serving AIOTF if it is not allocated, AIOT Registration Area, and periodical registration timer.

8. The AIoT device responds to the AIOTF with Registration Complete via NG RAN and AMF.

9. The AMF forwards the Registration complete to the AIOTF.

10. The AIOTF notifies the ADM of the registration complete event about the AIoT device. The information of the AIOTF is included in the notification, which is used for downlink data transmission.

11. The ADM stores the serving AIOTF information for the AIoT device.

12. [Conditional] If the ADM discovers that the serving AIOTF has changed, then it initiates the context release procedure to the old AIOTF to delete the context in it.

6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support Registration and Mobility Management.

NG-RAN:

-	Supports the AIOTF selection and transferring NAS messages to and from the AIoT Device.

AIoT Device:

-	Support performing the registration procedure.

-	Store and use temporary AIoT device ID from netwrok.

ADM:

-	Support AIoT device profile management.

-	Support AIoT context release procedure to old serving AIOTF.

-	Support new interface with AIOTF.

# * * * * End of changes * * * *