3GPP TSG-SA WG2 Meeting #172	S2-2510373

17 – 21 Nov, 2025, Dallas, U.S.                                                               (was S2-2509789)

Source:	China Telecom

Title:	[WT#1.2, Localized Service Access] Support of Localized Service Access in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the WT scope of supporting localized service access.

Introduction

In SA2#170 and SA2#171 meeting, companies' view over WT#1.2 Localized Service converges slowly.

It is general agreed that the support for access to Localized Services as specified in 5G is the starting point for the discussion on support of 6G use cases and requirements related to localized service access. And the following gaps between requirement and current SNPN/PNI-NPN are identified:

1. Service continuity is the requirement in 22.261:

Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support:

-    access to the hosting network and use home network services or selected localized services via the hosting network,

-    seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network.

2. Local control, authorization and authentication are mentioned in use cases in 22.870 and identified as requirement:

1.	[PR 5.6.2.6-1] Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period.

2.	[PR 5.6.2.6-2] Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator.

3.	[PR 11.10.6-1] Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party).

5.	[PR 11.10.6-2] Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator.

6.	 [PR 11.10.6-3] The 6G network shall support a mechanism to allow the operator or authorized 3rd party to perform local control of the localized network when the localized network loses connection with the PLMN network.

Thus, it is proposed to document the WT scope for WT#1.2 Localized service access based on postponed S2-2509789 from SA2#171 as the following.

**** First Change ****

(All text new, changes shown w.r.t S2-2509789 marked in grey)

Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1: Support for access to Localized Services as specified in 5G (refer to TS 23.501) is the starting point for the discussion on support of 6G use cases and requirements related to localized service access.

NOTE 1a: Network selection and access control are out of scope of this work task.

- [Agreeable?] Authorization and authentication of UE for localized service access including how to enable locally managing and using the part of subscription that is needed for UE to access to localized service offered as part of PLMN.

  - Void

[Agreeable] NOTE 2: Authorization, authentication aspects need to be coordinated with SA3.

NOTE 2a: Solutions for this WT are expected not to introduce additional UE functionality that is specific for accessing localized services to avoid dependency on specific UE feature support.

- Void

- [Agreeable] Whether and how to support UE mobility, service continuity between PLMN network and network providing localized service access, or between the networks providing localized service access, including interface and interaction procedure between PLMN network and networks providing localized service access, or between the networks providing localized service access.

[Agreeable?]NOTE 3a: Service continuity refers to accessing the same service (e.g. internet connectivity) that is provided by both the initial/source network and the latter/target network.

[Agreeable]NOTE 3b: Localized service access can be provided by a part of a PLMN as well.

- Void

- Void

- Void

- Void.

NOTE 4: Void

NOTE 5:	Void

**** End of Changes ****