3GPP TSG-WG SA2 Meeting #172	S2-2510391

Dallas, USA, 17 – 21 November 2025	(revision of S2-250abcd)

Source:	vivo

Title:	Conclusions on KI#6: Configuration of Parameters for Sensing Entities

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: this paper tries to provide the interim conclusion for KI6: Configuration of Parameters for Sensing Entities.

1. Introduction

The solutions for KI#6 are documented in Solution #3, #15, #20, #27, #34, and #35, which are summarized as the following aspects:

-	The SF is responsible for generating the configuration of parameters for Sensing Entities and providing it to Sensing Entity.

-	The SF generates the configuration of parameters for Sensing Entities according to the sensing service requirement and sensing capability of the Sensing Entity,

-	The sensing service requirement includes: Target sensing service area, Sensing result report condition (e.g., one time or periodic)

-	The sensing capability of the Sensing Entity includes: Supported sensing area, supported sensing mode.

-	The configuration of parameters for Sensing Entities includes:

-	The information for sensing measurement guidance, e.g., Sensing area, Sensing mode

-	The information for sensing data report:

-	Sensing data report condition: e.g., one time, periodic

-	Sensing data tunnel info: e.g. SF IP address, terminal endpoint info

Proposal: it is proposed to adopt the above summarized aspects as the conclusions for KI #6: Configuration of Parameters for Sensing Entities.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14 v1.1.0.

# * * * * First change (all new)* * * *

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

8.X Conclusions for KI#6 Configuration of Parameters for Sensing Entities

To resolve the KI#6 for Configuration of Parameters for Sensing Entities, the following conclusions are adopted for normative work.

-	The SF is responsible for generating the configuration of parameters for Sensing Entities and providing it to the Sensing Entity.

-	The SF generates the configuration of parameters for Sensing Entities according to the sensing service requirement and sensing capability of the Sensing Entity:

-	The sensing service requirement includes e.g., target sensing service area, sensing result report condition (e.g., one time or periodic).

-	The sensing capability of the Sensing Entity includes e.g., supported sensing service area, supported sensing mode (i.e., Tx, Rx).

-	The configuration of parameters for Sensing Entities includes:

-	The information for sensing measurement, e.g., target sensing service area, sensing mode (i.e., Tx, Rx).

-	The information for sensing data report:

-	Sensing data report condition: e.g., one time, periodic.

-	Sensing data tunnel info: e.g. SF IP address, SF endpoint info.

# * * * * End of changes * * * *