SA WG2 Meeting #172	S2-2510579

17 – 21 Nov, 2025, Dallas, U.S.A

Source:	NTT DOCOMO

Title:	[WT#07, all topics] Scope and key issue on NTN

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

This contribution proposes scope and key issue to Work task 7 of the 6G study on NTN.

# 1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #7 of the approved 6G SID

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

As the industry progresses toward 6G, Non-Terrestrial Networks (NTN) are expected to play a pivotal role in achieving ubiquitous, resilient, and high-capacity global connectivity. The growing demand for uninterrupted services in diverse environments—including maritime, aviation, and remote terrestrial regions—has accelerated interest in multi-orbit satellite systems. These systems integrate Low Earth Orbit (LEO), Medium Earth Orbit (MEO), Geostationary Orbit (GEO) satellites, and High-Altitude Platform Stations (HAPS) to complement each other's capabilities and ensure service continuity, especially in scenarios where terrestrial coverage is unavailable or unreliable.

Each orbit brings complementary strengths: LEO systems enable low-latency communication and are suitable for time-sensitive applications, while GEO and MEO satellites offer broader and more stable coverage for delay-tolerant services or bulk data transmission. This layered orbital diversity supports differentiated service delivery based on application requirements and user mobility. The resulting cross-hybrid architecture—blending terrestrial and multi-orbit NTN assets—represents a natural architectural evolution for delivering seamless global connectivity in 6G.

As 6G evolves to incorporate Non-Terrestrial Networks (NTN) as an integral component of global connectivity, ensuring service continuity across dynamic satellite environments becomes essential. While many aspects of orbit-specific mobility will be addressed within RAN studies, certain control-plane and architectural considerations remain within SA2’s purview.

In particular, the 6G core network may need to address gaps related to path selection, routing coordination, and constellation-level transitions that cannot be fully resolved by radio procedures alone. Existing mobility mechanisms are primarily designed for UE movement, whereas NTN introduces additional mobility due to satellite motion, especially in LEO and MEO systems, which may lead to frequent access changes and path re-establishment challenges. These aspects can affect service anchoring, policy enforcement, and continuity when transitioning between NTN and terrestrial access networks, or between different NTN orbits.

Therefore, as a pragmatic step, this work task proposes to perform a gap analysis from the core network perspective to identify where existing mobility and continuity mechanisms are sufficient and where enhancements may be required to ensure seamless operation in multi-orbit NTN scenarios.

Current 3GPP system specifications provide a foundational framework for NTN integration, primarily focusing on individual orbit-based connectivity and basic support for satellite mobility. However, several gaps remain when considering multi-orbit NTN scenarios: i) Service continuity mechanisms across orbit types are not explicitly addressed in existing mobility and session management procedures; ii) Orbit-aware communication path selection is not natively supported in control plane functions such as AMF and SMF, nor in policy control mechanisms; iii) Latency differentiation, orbit characteristics, and satellite visibility are not yet integrated into decision-making for access selection or mobility handling; iv) No unified architectural framework currently exists to handle orbit-level handover, path switching, or multi-orbit load balancing while maintaining service quality and session stability.

In the absence of standard mechanisms for such functionality, vendors and operators may implement proprietary solutions, leading to fragmentation and inefficiencies. To fully leverage the advantages of a multi-orbital architecture, the 3GPP system architecture must support dynamic communication path management across satellites in different orbits, without service interruption or degradation.

Technical Impact:

Addressing the above gaps within 3GPP architecture would require studying the implications of multi-orbit integration on several SA2-managed functions, including:

- Enhancements to mobility management, enabling service continuity across satellites with different orbital dynamics.

- Definition of orbit-related parameters (e.g., latency class, coverage stability, dwell time) that can be exposed through UE or network signalling for path selection and policy enforcement.

- Support for orbit-aware NTN-specific QoS control, ensuring that service requirements are met despite changing satellite paths.

- Clarification of the interaction between multi-orbit NTN elements and existing network slicing, PCF, NEF/SEF, and interworking mechanisms.

- Evaluation of session continuity models when the UE moves between satellites (or relay nodes) in different orbits—especially for stateful services or high-reliability applications.

Based on the justification, it is proposed to update the annex of TR 23.801-01 v0.0.0 to reflect the scope of WT#7.

**** First Change ****

## Annex A.X 	WT#7 Scope

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:  The detailed scope for WT#7 will be coordinated and aligned with RAN, including multi-orbit NTN-NTN mobility handling, in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:  This WT has IWK aspects that are scoped under WT#2.

NOTE 3:  This WT has dependency on mobility aspects that are scoped under WT#1.1.

**** Second Change ****

# 5.X. Key Issue #X: Support for NTN

This key issue for the support of NTN will study the following:

-	Study service continuity aspects for NTN, including TN–NTN and NTN–NTN mobility.

-	Perform a high-level gap analysis to determine whether existing mobility and continuity mechanisms in the 6G architecture are sufficient for multi-orbit and dynamic NTN cases.

-	Identify architectural dependencies and coordination points between RAN and CN to ensure coherent NTN support.

-	How to ensure session continuity without packet loss during orbit transitions in case of multi-orbit architecture, while satisfying those service-specific requirements.

-	Study how to support path selection and routing coordination for NTN, including scenarios where multiple satellite orbits are available, for maintaining service continuity.

-	How to ensure interworking between the multi-orbit NTN system and other accesses, including 3GPP accesses in the terrestrial network and non-3GPP satellite accesses.

**** End of Changes ****