SA WG2 Meeting #172	S2-2510641

17 - 21 November, 2025, Dallas, USA

Title:	On the issue of UE user change in NSSAA and DN secondary Authentication

Source:	Nokia

Document for:	Discussion

Agenda Item:	20.18

Abstract:	this paper highlights a potential issue with some 5G features that require an extra level of authentication based on user identity due to the fact the MT is not aware of different users of a phone. The MT is not handling user access to the device, the TE is. Proposes the issue may be fixed in the work on TEI20_NEPS for NSSAA aspects.

# 1.	Introduction and problem statement

The NSSAA and DN-Secondary Authentication and Authorization features provide a User ID extra-level of authentication and authorization on top of the primary SUPI authentication and authorization. In other words, these features take care of checking that the user of the device (subject to the subscriber of the device being authorized) is also authorized to use a S-NSSAI subject to NSSAA or a DNN subject to DN-Secondary Authentication and Authorization.

The features also take care of executing the extra level authorization only when strictly needed, i.e. the first use and then periodic re-authentications based on AAA server policies (which are not in 3GPP scope nor controllable by the operator). The re-authentications are not too frequent and are normally time driven (e.g. scheduled every few hours, if any at all). The trade-off is between security and user experience (it is not a good user experience to need to reauthenticate too frequently).

A device (UE) is composed of a TE (terminal equipment, i.e. the computing part the user interacts with) and a MT (the modem, which takes care of connections/registrations/mobility etc). The user-level management is normally a TE task, that does not involve the modem (MT). For instance, the TE can be a laptop, a tablet and the MT a wireless card (embedded or not), or a TE maybe the computing part of a smartphone. The user access control to such devices can allow multiple users to be able to log in and "unlock" the TE via OS-level PINs or biometric factors (via e.g. registration of different fingerprints (which may be from different persons), registration of two face IDs – possible e.g. in iOS, (which may be from different persons), or user ID/passwords, or different user profiles configured in the device (Android , which also includes a work profile which can be associated with a dedicated PIN). Guest-mode is possible on some TEs (Windows, Android devices).

Because the user is handled at TE level, the MT is expected to rely on the TE on the handling of the EAP interaction in NSSAA and DN Secondary Authentication and Authorization.

The reader may have by now grasped where this is leading to: it can happen that the UE has successfully passed NSSAA or DN-Secondary Authentication and Authorization for USER1. But then USER2 is admitted to using the TE based on the presentation of a valid OS-level credential (e.g. by log in based on user ID/password, pin or biometric factors), but unfortunately the 3GPP system is unaware of this change of user of the UE. This may breach the "security" that is built into the NSSAA or DN-Secondary Authentication and Authorization features and therefore we propose to address this in order to have a completely deployable feature that meets high security standards which are expected from 3GPP specifications.

# 2.	Proposed solution

It is proposed that when there is a change of user which the TE can detect, the TE notifies the MT via unspecified interface (or AT-Commands for split TE/MT) of user change event. Then, the MT sends to the network a message that notifies the AMF (for NSSAA) or the SMF(+PGWC) (for DN-Secondary authentication and Authorization or NSSAAoEPS) that the UE has detected a user change. This then triggers necessary re-authentications initiated by AMF and respectively SMF(+PGW-C). it is proposed the interaction with the AMF is via a new IE in MRU, and towards the SMF+PGW-C via PDU session / PDN connections update procedures including in PCO the indication of new user detection.

# 3. 	Conclusion

It is proposed this issue is discussed and if agreeable the CRs in S2-2510674 and S2-2510650 are approved to cover the NSSAA impact. 5GS_Ph1 CRs would be needed for DN-Secondary Authentication and Authorization feature, and Nokia would be happy to provide those at the next meeting when maintenance is back on the agenda.