3GPP TSG-SA WG2 Meeting #172	S2-2510689

# October 17-21, Dallas, US

Source:	Xiaomi

Title:	Conclusion for KI#2 Authorization and Revocation to Support Sensing Service

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes the conclusion for KI#2 Authorization and Revocation to Support Sensing Service.

1. Discussion

TS 23.700-14 v1.1.0 has documented the interim agreements for KI#2 Authorization and Revocation to Support Sensing Service:

7.1.2 Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service The following principles are agreed for KI#2: a) The Sensing Service authorization consists of two steps: 1) Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9]; 2) Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality; NOTE: In this clause, Sensing authorization functionality may refer to Sensing Function, Sensing Control Function or Sensing Gateway and the wording may need alignment according to the conclusion of key issue 1. b) Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase. Table 7.1.2-1: Sensing Authorization information for Sensing Service AF Authorization Data; Description. AF ID; Identifier used to identify the AF.. Allowed/Not allowed area for sensing; Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1).. Allowed/Not allowed time period for sensing; Indicate the allowed time period within which AF can trigger a particular sensing service operation.. (Allowed) sensing service type; Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations.. NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed.. c) The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing authorization functionality for authorisation of the sensing service request. d) Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met. e) The AF can cancel the on-going Sensing Service Request when needed. f) There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

Proposal 1: copy all the interim agreements for KI#2 in clause 7.1.2 into clause 8.2 conclusion for KI#2.

Observation 2: when AF is in the trusted domain, e.g., there would be some kind of service level agreement between AF and operator for the sensing service. It could be assumed that AF is authorized for Sensing service, or can be authorized by implementation way.

Proposal 2: add one NOTE1 as “it is implementation for AF and operator to perform the Authorisation of the AF for Sensing Service request if the AF is inside the trusted domain.”

Observation 3: in last SA2#171 meeting, one important consensus to introduce one new Sensing Function was agreed. And there is no issue for Sensing Function to perform the Authorization of the AF’s Sensing Service Request. Don’t see the need to introduce another new network function (e.g., Gateway function) to perform the Authorization of the AF’s Sensing Service Request. and additionally new interface between Gateway Function and Sensing Function should be defined. For 5G-A Sensing, it is better to avoid the unnecessary impacts.

Proposal 3: Sensing authorization functionality is performed by Sensing Function. And replace “Sensing authorization functionality” with “Sensing Function”

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * First Change * * * *

8.2 conclusion for KI#2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

NOTE1: it is implementation for AF and operator to perform the Authorisation of the AF for Sensing Service request if the AF is inside the trusted domain.

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing Function

b)	Sensing Function determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 8.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 2: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing Function  for authorisation of the sensing service request.

d)	Sensing Function   can revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

* * * End of Changes * * * *