3GPP TSG-SA2 Meeting #172	S2-2510713

# Dallas, Texas, US, November 17 - November 21

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6482 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling and Disabling LP-WUS per UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, InterDigital Inc., Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S2-2509926/R2-2506261, RAN2 indicaes the RAN2 made the following 4 agreements: 1. RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE. 2. RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS per UE in IDLE and INACTIVE. Detail signalling is up to SA2 and CT1. 3. RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1 and RAN3. 4. RAN2 assumes that without such NAS signalling, UE is allowed to use LP-WUS in IDLE and INACTIVE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add "LP-WUS Disabled" or "LP-WUS Enabled" indication in the "RRC Inactive Assistance Information". Add “Support of LP-WUS Disabling” in the the UE 5GMM Core Network Capability. Adds functionality for enabling/disabling LP-WUS in NAS for UE and AMF. Adds functionality for enabling/disabling LP-WUSPS in NAS for UE and AMF, this is already supported in stage-3. Adds functionality AMF to provide LP-WUS Disabled indication in paging message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support of LP-WUS enabling/disabling via NAS signaling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2.5, 5.4.4a, 5.4.12a, 5.4.12a.1, 5.4.12a.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************** First Change ****************************

##### 5.3.3.2.5	CM-CONNECTED with RRC_INACTIVE state

RRC_INACTIVE state applies to NG-RAN. UE support for RRC_INACTIVE state is defined in TS 38.306 [69] for NR and TS 36.306 [70] for E-UTRA connected to 5GC. RRC_INACTIVE is not supported by NB-IoT connected to 5GC.

The AMF shall provide assistance information to the NG-RAN, to assist the NG-RAN's decision whether the UE can be sent to RRC_INACTIVE state except due to some exceptional cases such as:

-	PLMN (or AMF set) does not support RRC_INACTIVE;

-	The UE needs to be kept in CM-CONNECTED State (e.g. for tracking).

The "RRC Inactive Assistance Information" includes:

-	UE specific DRX values;

-	UE specific extended idle mode DRX values (cycle length and Paging Time Window length);

-	The Registration Area provided to the UE;

-	Periodic Registration Update timer;

-	If the AMF has enabled MICO mode for the UE, an indication that the UE is in MICO mode;

-	Information from the UE identifier, as defined in TS 38.304 [50] for NR and TS 36.304 [52] for E-UTRA connected to 5GC, that allows the RAN to calculate the UE's RAN paging occasions;

-	An indication that Paging Cause Indication for Voice Service is supported;

-	AMF PEIPS Assistance Information (see clause 5.4.12.2) for paging a UE in CM-CONNECTED with RRC_INACTIVE state over NR as defined in TS 38.300 [27];

-	AMF LP-WUSPS Assistance Information (see clause 5.4.12A) for paging a UE in CM-CONNECTED with RRC_INACTIVE state over NR as defined in TS 38.300 [27];

-	AMF provided "LP-WUS Disabled" indication (see clause 5.4.12a.1);

-	CN based MT communication handling support indication for RRC_INACTIVE state (see clause 5.31.7.2.1).

The RRC Inactive Assistance Information mentioned above is provided by the AMF during N2 activation with the (new) serving NG-RAN node (i.e. during Registration, Service Request, Handover) to assist the NG RAN's decision whether the UE can be sent to RRC_INACTIVE state. If the AMF allocates a new Registration Area to the UE, the AMF should update the NG-RAN with the new Registration Area by sending the RRC Inactive Assistance Information accordingly. The Paging Cause Indication for Voice Service is used to assist NG RAN to perform RAN based paging.

RRC_INACTIVE state is part of RRC state machine and it is up to the RAN to determine the conditions to enter RRC_INACTIVE state. If any of the parameters included in the RRC Inactive Assistance Information changes as the result of NAS procedure, the AMF shall update the RRC Inactive Assistance Information to the NG-RAN node.

When the UE is in CM-CONNECTED state, if the AMF has provided RRC Inactive assistance information, the RAN node may decide to move a UE to CM-CONNECTED with RRC_INACTIVE state.

The state and "endpoints" (in the case of Dual Connectivity configuration) of the N2 and N3 reference points are not changed by the UE entering CM-CONNECTED with RRC_INACTIVE state. A UE in RRC_INACTIVE state is aware of the RAN Notification area and periodic RAN Notification Area Update timer.

The 5GC network is not aware of the UE transitions between CM-CONNECTED with RRC_CONNECTED and CM-CONNECTED with RRC_INACTIVE state, unless the 5GC network is notified via N2 notification procedure in clause 4.8.3 of TS 23.502 [3].

At transition into CM-CONNECTED with RRC_INACTIVE state, the NG-RAN configures the UE with a periodic RAN Notification Area Update timer taking into account the value of the Periodic Registration Update timer value indicated in the RRC Inactive Assistance Information and uses a guard timer with a value longer than the RAN Notification Area Update timer value provided to the UE.

If the periodic RAN Notification Area Update guard timer expires in NG-RAN, the NG-RAN shall initiate AN Release procedure as specified in clause 4.2.6 of TS 23.502 [3].

When the UE is in CM-CONNECTED with RRC_INACTIVE state, the UE performs PLMN selection procedures as defined in TS 23.122 [17] and TS 24.501 [47].

When the UE is CM-CONNECTED with RRC_INACTIVE state, the UE may resume the RRC Connection due to:

-	Uplink data pending;

-	Mobile initiated NAS signalling procedure;

-	As a response to RAN paging;

-	Notifying the network that it has left the RAN Notification Area;

-	Upon periodic RAN Notification Area Update timer expiration.

If the UE resumes the connection in a different NG-RAN node within the same PLMN or equivalent PLMN or within the same SNPN or equivalent SNPN, the UE AS context is retrieved from the old NG-RAN node and a procedure is triggered towards the CN (see clause 4.8.2 of TS 23.502 [3]).

NOTE 1:	With Dual Connectivity configuration if the UE resumes the RRC connection in the Master RAN node, the Secondary RAN node configuration is defined in TS 38.300 [27].

If the RAN paging procedure applying DRX or eDRX value no longer than 10.24s, as defined in TS 38.300 [27], is not successful in establishing contact with the UE the procedure shall be handled by the network as follows:

-	If NG-RAN has at least one pending NAS PDU for transmission, the RAN node shall initiate the AN Release procedure (see clause 4.2.6 of TS 23.502 [3]) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.

-	If NG RAN has only pending user plane data for transmission, the NG-RAN node may keep the N2 connection active or initiate the AN Release procedure (see clause 4.2.6 of TS 23.502 [3]) based on local configuration in NG-RAN.

NOTE 2:	The user plane data which triggers the RAN paging can be lost, e.g. in the case of RAN paging failure.

If the RAN paging procedure applying eDRX value longer than 10.24s, as defined in TS 38.300 [27], has not requested the CN based mobile terminated (MT) communication handling as described in clause 5.31.7.2.1 and is not successful in establishing contact with the UE after paging the UE, the procedure shall be handled by network as follows:

-	If NG-RAN has at least one pending NAS PDU for transmission, the RAN node shall initiate the AN Release procedure (see clause 4.2.6 of TS 23.502 [3]) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.

-	If NG-RAN has only pending user plane data for transmission, the NG-RAN node may keep the N2 connection active and based on implementation send indication to the CN requesting the CN based mobile terminated (MT) communication handling as described in clause 5.31.7.2.1, or initiate the AN Release procedure (see clause 4.2.6 of TS 23.502 [3]) based on local configuration in NG-RAN.

NOTE 3:	The user plane data which triggers the RAN paging can be lost, e.g. in the case of RAN paging failure.

If a UE in CM-CONNECTED with RRC_INACTIVE state performs cell selection to GERAN/UTRAN/E-UTRAN, it shall follow idle mode procedures of the selected RAT as specified in clause 5.17.

In addition, a UE in CM-CONNECTED state with RRC_INACTIVE state shall enter CM-IDLE state and initiates the NAS signalling recovery (see TS 24.501 [47]) in the following cases:

-	If RRC resume procedure fails,

If the UE receives Core Network paging,

-	If the periodic RAN Notification Area Update timer expires and the UE cannot successfully resume the RRC Connection,

-	In any other failure scenario that cannot be resolved in RRC_INACTIVE state and requires the UE to move to CM-IDLE state.

When a UE is in CM-CONNECTED with RRC_INACTIVE state and a trigger to change the UE's NG-RAN or E-UTRAN UE Radio Capability information happens, the UE shall move to CM-IDLE state and initiate the procedure for updating UE Radio Capability defined in clause 5.4.4.1. (For specific requirements for a UE operating in dual-registration mode see clause 5.17.2.1)

When UE is in CM-CONNECTED with RRC_INACTIVE state, if RAN has received Location Reporting Control message from AMF with the Reporting Type indicating single stand-alone report or continuously reporting whenever the UE changes the cell, the RAN shall perform location reporting as specified in clause 4.10 of TS 23.502 [3].

When the UE is CM-CONNECTED with RRC_INACTIVE state. If the AMF receives Nudm_UECM_DeregistrationNotification from UDM, the AMF shall initiate AN Release procedure as specified in clause 4.2.6 of TS 23.502 [3].

When UE is in CM-CONNECTED with RRC_INACTIVE state, if RAN has received Location Reporting Control message from AMF with the Reporting Type of the Area Of Interest based reporting, the RAN shall send a Location Report message to AMF including UE presence in the Area Of Interest (i.e. IN, OUT, or UNKNOWN) and the UE's last known location with time stamp.

When the UE is in CM-CONNECTED with RRC_INACTIVE state, if the old NG-RAN node that sends the UE into RRC_INACTIVE state receives the downlink N2 signalling, it initiates the RAN paging as defined in TS 38.300 [27]. If the UE resumes the RRC Connection towards a different NG-RAN node, the old NG-RAN node includes the "UE Context Transfer" indication into a response container to the NF (e.g. AMF or SMF) that generates such N2 downlink signalling. Then the NF shall reattempt the same procedure when the path switch from the old NG-RAN node to the new NG-RAN node is complete.

**************************** Next Change ****************************

### 5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms, etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (clause 5.3.2.1 of TS 23.401 [26]).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Network Slice Replacement as described in clause 5.15.19.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information (E-UTRA) see clause 5.4.9.

-	Paging Subgrouping Support Indication (NR) see clause 5.4.12.

-	Support of LP-WUS Disabling (see clause 5.4.12a).

-	LP-WUS Paging Subgrouping Support Indication, see clause 5.4.12a.

-	CAG, see clause 5.30.3.3.

-	CAG with validity information (if UE supports CAG), see clause 5.30.3.3.

-	Subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12).

-	Support of NSAG (see clause 5.15.14).

-	Partial Network Slice support in a RA (see clause 5.15.17).

-	Minimization of Service Interruption (MINT), as described in clause 5.40.

-	Equivalent SNPNs (see clause 5.30.2.11).

-	Unavailability Period Support, as described in clause 5.4.1.4.

-	Support for network reconnection due to RAN timing synchronization status change, see clauses 5.27.1.12 and 5.3.4.4.

-	UE Configuration of network-controlled Slice Usage Policy (see clause 5.15.15.2).

-	Temporarily available network slices (see clause 5.15.16).

-	Support of S-NSSAI location availability information, as described in clause 5.15.18.2.

-	Support of network verified UE location over NR NTN (see clause 5.4.11.4).

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications:

-	Connection Release Supported.

-	Paging Cause Indication for Voice Service Supported.

-	Reject Paging Request Supported.

-	Paging Restriction Supported.

Otherwise, the UE with the capabilities of Multi-USIM features but does not intend to use them shall not indicate support of these one or more Multi-USIM features.

A UE not operating two or more USIMs shall indicate the Multi-USIM features are not supported.

NOTE:	It is not necessary for a UE operating two or more USIMs to use Multi-USIM features with all USIMs.

**************************** Next Change ****************************

### 5.4.12a	LP-WUS

#### 5.4.12a.1	General

The RAN and UE may use Low Power Wake UP Signal (LP-WUS) with Paging Subgrouping (LP-WUSPS) to reduce the power consumption of the UE in RRC_IDLE and RRC_INACTIVE over NR (see TS 38.304 [50]).

The LP-WUSPS can be based on a LP-WUS Subgroup ID allocated by the AMF (see clause 5.4.12a.2 for further details) or is determined by the RAN on its own as defined in TS 38.304 [50] and TS 38.331 [28] if the LP-WUS is not disabled.

The AMF, when determining its paging strategy (see clause 5.4.3), may decide to disable or enable LP-WUS per UE and should take into consideration whether a gNB is using LP-WUSPS determined by the RAN.

If LP-WUS is supported, the UE shall indicate the support of LP-WUS in the Registration Request message via the UE MM Core Network Capability. The AMF may use local configuration information (e.g. when not all the gNBs in the Tracking Area support LP-WUS or when it wants to avoid LP-WUS overload) to determine whether to disable LP-WUS per UE using NAS signalling (including both subgroup ID determined by AMF and by RAN). If the UE supports disabling LP-WUS and the AMF has determined to disable LP-WUS for the UE, the AMF stores the indication "LP-WUS Disabled" in the UE context and provides it to the UE in every Registration Accept message. If there is no “LP-WUS Disabled” indication, e.g., in the Registration Accept message to the UE and in RRC Inactive Assistance Information to NG-RAN, then the LP-WUS feature is enabled in RRC_IDLE and RRC_INACATIVE.

If the UE receives "LP-WUS Disabled" indication in a Registration Accept or UE Configuration Update message, it shall ignore the “per cell” LP-WUS indication defined in TS 38.304 [50] and TS 38.331 [28] and expect to be paged without LP-WUS.

The AMF may use the Registration procedure (as described in clause 4.2.2 of TS 23.502 [3]) or UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]), and N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to update the LP-WUS disabling/enabling and AMF LP-WUSPS Assistance Information in the UE and NG-RAN.

If the UE supports AMF disabling LP-WUS and did not receive the "LP-WUS Disabled" indication in Registration Accept or UE Configuration Update message, then the UE shall assume that LP-WUS is enabled, and it shall determine per cell which LP-WUSPS method to use as defined in TS 38.304 [50] and TS 38.331 [28].

If LP-WUS has been disabled for a UE, the AMF shall indicate in the Paging message to the gNB that LP-WUS is disabled and shall not provide LP-WUSPS Assistance Information in the Paging message to NG-RAN when paging the UE in RRC_IDLE. In addition, for UEs in RRC_INACTIVE mode, the AMF shall indicate that LP-WUS is disabled and not provide the AMF LP-WUSPS Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

If paging subgroups for LP-WUS are being allocated by the AMF, then all AMFs connected to one gNB (including the AMFs in different PLMNs using 5G MOCN network sharing) shall use a consistent policy in allocating UEs to the paging subgroups. The AMF may configure up to 32 different LP-WUS Subgroup IDs.

NOTE:	Because the UE uses the AMF allocated LP-WUSPS across all cells in its TAI-list, and, different, overlapping TAI lists can be allocated to different UEs, then to avoid UE power consumption increasing, it is likely to be necessary that all AMFs with an overlapping coverage area use a consistent policy in allocating UEs to the LP-WUSPS.

The NG-RAN node and the UE determine per cell which LP-WUSPS method to use as defined in TS 38.304 [50] and TS 38.331 [28].

**************************** Next Change ****************************

#### 5.4.12a.2	Core Network Assistance for LP-WUSPS

To support the LP-WUSPS, the LP-WUS Paging Subgrouping Support Indication and the LP-WUSPS Assistance Information are used by the AMF and NG-RAN to help determine whether LP-WUSPS applies to the UE and which paging subgroup is used when paging the UE (see TS 38.300 [27]).

In the Registration Request message, the UE includes the LP-WUS Paging Subgrouping Support Indication if the UE supports AMF LP-WUSPS Assistance Information for LP-WUS Paging Subgrouping. If the UE includes the LP-WUS Paging Subgrouping Support Indication, the UE may also include the paging probability information to assist the AMF. An AMF supporting LP-WUSPS stores the UE provided LP-WUS Paging Subgrouping Support Indication and the AMF determined AMF LP-WUSPS Assistance Information in the UE context. The AMF may use local configuration, the UE's paging probability information if provided, information provided by the RAN (e.g. any of the Information On Recommended Cells And RAN nodes For Paging), and/or previous statistical information for the UE to determine AMF LP-WUSPS Assistance Information. The AMF LP-WUSPS Assistance Information includes the Paging Subgroup ID.

NOTE 1:	To minimise MT voice call setup latency, the AMF could allocate Paging Subgroup IDs for LP-WUS taking into account whether or not the UE is likely to receive IMS voice over PS session calls.

NOTE 2:	To avoid MT traffic for more mobile UEs causing more stationary UEs to be woken up, the AMF could allocate Paging Subgroup IDs for LP-WUS taking into account the UE's mobility pattern.

If the AMF has determined AMF LP-WUSPS Assistance Information for the UE and has not disabled LP-WUSPS for the UE, the AMF stores it in the UE context in AMF and provides it to the UE in every Registration Accept message.

If the AMF has determined AMF LP-WUSPS Assistance Information and has not disabled LP-WUSPS for the UE, the AMF shall provide it to NG RAN when paging the UE. In addition, in order to support LP-WUSPS for UEs in RRC_INACTIVE mode, the AMF shall provide the AMF LP-WUSPS Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

When the NG-RAN is using AMF allocated subgroups for LP-WUSPS, both the UE and NG-RAN use the AMF LP-WUSPS Assistance Information to determine the paging subgroup to apply as defined in TS 38.300 [27].

The AMF may use the Registration procedure (as described in clause 4.2.2 of TS 23.502 [3]) or UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]), and the N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to update or remove the AMF LP-WUSPS Assistance Information in the UE and NG-RAN.