3GPP SA WG2 Meeting #172	 S2-2510757

# Dallas, USA,17-21 November 2025					             			  			  (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0567 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of multi-hop relay operation in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ProSe_NPN |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 indicated in the LS (S2-2509923/ C1-255515) that stage 3 has introduced the support of multi-hop UE-to-Network Rleay and Multi-hop UE-to-UE Relay operation in SNPN based on TS 23.304 text, and requested SA2 to clarify the support. The existing stage 2 text in TS 23.304 in clause 4.2.9.1 does mention multi-hop relay operation explicitly. Therefore, clarifications should be added to properly support the multi-hop relay operation in SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification in 4.2.9.1 to explicitly mention the support of multi-hop relay operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch of specification of stage 2 and stage 3 on the support of multi-hop relay operations in SNPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 4.2.9.1	Support for 5G ProSe in SNPN

The proximity based services defined in this specification are applicable to SNPN(s) with the following adaptations:

-	The non-roaming architecture in clause 4.2.1 is applicable also to SNPN, whereby the term PLMN is replaced by subscribed SNPN.

-	The roaming architecture in clause 4.2.2 is not supported.

-	Inter-SNPN operation in clause 4.2.3 is not supported.

-	For 5G ProSe Direct Discovery, the procedures in clauses 5.2 and 6, the network function services in clause 7 and the identifiers as specified in clause 5.8.1 are used with the following adaptations:

-	The concept of Local PLMN for direct discovery is not supported.

-	In the ProSe Application ID in clause 5.8.1.1 the value of PLMN ID within the ProSe Application ID shall be combined with a Network Identifier (NID) to identify an SNPN.

-	The ProSe Application Code in clause 5.8.1.4 is extended to allow the value of PLMN ID within the ProSe Application Code to be combined with a Network Identifier (NID) to identify an SNPN.

-	The Announcing PLMN ID in clause 5.8.1.11 is not used in the case of SNPNs.

-	The Monitored PLMN ID in clause 5.8.1.17 is ignored in the case of SNPNs.

-	In the 5G ProSe Direct Discovery with 5G DDNMF procedures as specified in clause 6.3.1, the term PLMN is to be replaced by SNPN, whereby PLMN IDs shall be combined with a Network Identifier (NID) to identify an SNPN, except if it pertains to PLMN IDs of VPLMNs or Local PLMNs or Announcing PLMN IDs. The term HPLMN is to be replaced by subscribed SNPN.

-	The 5G ProSe UE-to-Network Relay reference architectures for 5G ProSe (Multi-hop) Layer-2 UE-to-Network relay, 5G ProSe (Multi-hop) Layer-3 UE-to-Network relay and (Multi-hop) UE-to-UE relay in clauses 4.2.7 4.2.8, and 4.2.8A are supported, where only the non-roaming architecture as specified for 5G ProSe Layer-3 UE-to-Network relay in clause 4.2.7.1 is supported for SNPNs and the PLMN A, PLMN B and PLMN C in clauses 4.2.7 are the same in case of SNPNs and are replaced by the SNPN. The term PLMN is replaced by SNPN and the term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs.

-	For Authorization and Provisioning and for the subscription information of the ProSe service in NPNs, the procedures and functionality as specified in clauses 5.1 and 5.7 and 6.2 are used with the following adaptations:

-	The term PLMN is replaced by SNPN. The term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs. Except the subscribed SNPN, no other SNPNs are authorized to perform the 5G ProSe service (i.e. 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe (Multi-hop) UE-to-Network Relay discovery, 5G ProSe (Multi-hop) UE-to-Network Relay communication, 5G ProSe (Multi-hop) UE-to-UE Relay discovery and 5G ProSe (Multi-hop) UE-to-UE Relay communication) over PC5 reference point.

-	For 5G ProSe (Multi-hop) Layer-2 UE-to-Network relay operation, the procedures as specified in clauses 5.4.2 ,5.4.4.2 and 5.4.5 and the identifiers as specified in clause 5.8.3 are used with the following adaptations:

-	The term PLMN is replaced by SNPN.

-	The 5G ProSe Layer-2 Remote UE has to operate in SNPN access mode, if it intends to access SNPN for ProSe via 5G ProSe Layer-2 UE-to-Network relay.

-	The 5G ProSe UE-to-Network Relay or 5G ProSe Intermediate UE-to-Network Relay has to operate in SNPN access mode, if it intends to access SNPN for ProSe via 5G ProSe Layer-2 UE-to-Network relay.

-	The 5G ProSe Layer-2 UE-to-Network relay may participate in the 5G ProSe UE-to-Network Relay Discovery procedure for emergency service if the serving NG-RAN broadcasts ims-EmergencySupport or imsEmergencySupportForSNPN in system information as specified in TS 38.300 [12] and TS 38.331 [16].

-	The NCGI in clause 5.8.3.3 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

-	The 5G ProSe Layer-2 Remote UE only selects the 5G ProSe Layer-2 UE-to-Network Relay f or 5G ProSe Intermediate UE-to-Network Relay or which the NCGI includes the subscribed SNPN of 5G ProSe Layer-2 Remote UE for non-emergency service.

NOTE 1:	To prevent that unauthorized 5G ProSe Remote UEs can gain access to an NPN via 5G ProSe UE-to-Network Relay, the existing User Plane and/or Control Plane authorization verification procedures as specified in clause 6.3.4 of TS 33.503 [29] and restricted 5G ProSe Direct Discovery procedures as specified in clause 6.1.3.2 of TS 33.503 [29] are reused.

-	For 5G ProSe (Multi-hop) Layer-3 UE-to-Network relay operation, the procedures as specified in clauses 5.4.1, 5.4.4.3, 5.4.5 and 6.5 and the identifiers as specified in clause 5.8.3 apply with the following adaptations:

-	The term PLMN is replaced by SNPN. The term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs.

-	The 5G ProSe Layer-3 UE-to-Network relay has to operate in SNPN access mode in order to relay traffic to/from an SNPN for a 5G ProSe Layer-3 Remote UE.

-	The 5G ProSe Layer-3 Remote UE has to operate in SNPN access mode in order to exchange traffic to/from an SNPN via a 5G ProSe Layer-3 UE-to-Network relay.

-	The NCGI in clause 5.8.3.2 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

-	The TAI in clause 5.8.3.2 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

NOTE 2:	To prevent that unauthorized 5G ProSe Remote UEs can gain access to an NPN via 5G ProSe UE-to-Network Relay, the existing User Plane and/or Control Plane authorization verification procedures as specified in clause 6.3.3 of TS 33.503 [29] and restricted 5G ProSe Direct Discovery procedures as specified in clause 6.1.3.2 of TS 33.503 [29] are reused.

-	The support for 5G ProSe for UEs in limited service state as specified in clause 5.9 applies whereby the term PLMN is replaced by an SNPN.

The SNPN onboarding procedure defined in clause 5.30.2.10 of TS 23.501 [4] via 5G ProSe UE-to-Network Relay is not supported.

# >>>>END OF CHANGES<<<<