3GPP SA WG2 Meeting #171	S2-2508214

# 13 - 17 October, 2025, Wuhan, China																(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1510 | rev |  | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on timestamp of input data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some input data definitions do not include an explicit timestamp or time-related information, which is essential for correlating multiple input data sources when training ML models in the NWDAF. In case time information is missing or unable to retrieve, a NOTE is added to clarify that the data reception time may be used as the corresponding information. Regarding the UE’s direction in clause 6.7.2.3, it should be considered as part of the UE location information: - Combining UE direction with UE location is more reasonable, since direction alone is not meaningful. - This also ensures alignment with clause 6.21.2 of TS 23.288, where the location estimate and velocity estimate (including speed and direction information) are defined as part of the UE location information. For the timestamp in clause 6.7.5.2, there may multiple Exceptions, so the tiemstamp for each exception is significant. In clause 6.17.2, the timestamp is nesscary to correlate the UE location from LCS and the UE location from AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing timestamp in input data. Add NOTE to clarify that the data reception time may be used as the corresponding information. Clarify that the UE’s direction should be part of the UE location information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to accurately correlate data from different sources, leading to reduced effectiveness or accuracy of ML model training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4, 6.7.2.3, 6.7.5.2, 6.17.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change* * * *

### 6.2.4	Correlation between network data and service data

The Correlation information in input data which helps NWDAF correlate data from different NFs, OAM and UE application(s) is defined in Table 6.2.4-1, which is subject to all the network data analytics.

NOTE 1:	For simplicity, the correlation information is not listed in the input data per network data analytics.

NOTE 2:	For the data without corresponding time information, using data reception time as the corresponding timestamp.

Table 6.2.4-1: Correlation Information

| Correlation Information | Description |
| --- | --- |
| Timestamp, IP address 5-tuple | To correlate the data from AF and from UPF. |
| Timestamp, AN Tunnel Info (Clause 9.3.2.2 of TS 38.413 [16]) | To correlate the UPF data and OAM data which are reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |
| Timestamp, UE IP address | To correlate the data from UPF and SMF. |
| Timestamp, SUPI | To correlate data from SMF and AMF. |
| Timestamp, SUPI, DNN, S-NSSAI or UE IP address | To correlate data from SMF and PCF. |
| Timestamp, RAN UE NGAP ID (Clause 9.3.3.2 of TS 38.413 [16]) and Global RAN Node ID | To correlate the AMF data and OAM data reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |
| Timestamp, Application ID, IP filter information | To correlate data from SMF and AF. |
| Timestamp, UE ID or UE IP address, Application ID, DNN, S-NSSAI | To correlate data from 5GC NF (e.g. SMF, UPF) and UE Application (via the AF). |

# * * * * Next change * * * *

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the statistic applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2] (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Observed location statistics (see NOTE 2). |
| >> UE location (NOTE 5) | TAs or cells which the UE stays or geographical location (longitude and latitude level) (see NOTE 3). |
| >> Ratio (NOTE 5) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 5) | The geographical distribution of the UEs among the TAs or cells or location coordinates. |
| >> Requested Linear Distance Threshold (NOTE 4) | The linear distance threshold used for UE location reporting. |
| >> Geographical Identifier (NOTE 5) | Geographical Identifier as specified in TS 23.228 [47] (see NOTE 6). |
| >> UE's direction (NOTE 5, NOTE 7) | The direction of the UEs in the Area of Interest. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: If Visited AOI(s) was provided in the analytics request/subscription, the UE location provides information on the observed location(s) that the UE or group of UEs had been residing during the Analytics Target Period. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 4: The requested Linear Distance Threshold is provided only when in the analytic filter information of the analytics request there are multiple linear distance thresholds and the target is a single UE. NOTE 5: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 6: It depends on the implementation how the NWDAF collects the geographic identifier. NOTE 7: It depends on the implementation how the NWDAF generates the statistics of UE’s direction. |  |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the prediction applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2] (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Predicted location prediction during the Analytics target period. |
| >> UE location (NOTE 3) | TAs or cells where the UE or UE group may move into or geographical location (longitude and latitude level) (see NOTE 2). |
| >> Confidence | Confidence of this prediction. |
| >> Ratio (NOTE 3) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 3) | The geographical distribution of the UEs among the TAs or cells or location coordinates. |
| >> Geographical Identifier (NOTE 3) | Geographical Identifier as specified in TS 23.228 [47] (see NOTE 4). |
| >> UE's direction (NOTE 3, NOTE 5) | The direction of the UEs in the Area of Interest. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 4: It depends on the implementation how the NWDAF collects the geographic identifier. NOTE 5: It depends on the implementation how the NWDAF generates the predictions of UE’s direction. |  |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

If a UE Location order indicator is included in the Analytics Reporting information, the NWDAF does not aggregate the UE locations in a long duration but provides the UE locations one by one in their own time period, i.e. the "UE location (1..max)" in the UE Mobility analytics has only one UE location (TA, Cell or a finer granularity UE Location smaller than cell) which indicates the UE is located in this UE location in the duration from the time slot start (i.e. time stamp when the UE enters this location as described in clause 6.7.2.2).

# * * * * Next change * * * *

#### 6.7.5.2	Input Data

The Exceptions information from AF is as specified in Table 6.7.5.2-1.

On request of the service consumer, the NWDAF shall collect and analyse UE behavioural information from the 5GC NFs (SMF, AMF, AF), or OAM as specified in clauses 6.7.2.2 and 6.7.3.2 and/or expected UE behavioural parameters from UDM as defined in clause 4.15.6.3, TS 23.502 [3], depending on Exception IDs.

NOTE:	Care needs to be taken with regards to load by avoiding to cause major extra signalling when collecting data for any UE.

Table 6.7.5.2-1: Exceptions information from AF

| Information | Description |
| --- | --- |
| IP address 5-tuple | To identify a data flow of a UE via the AF (such as the Firewall or a Threat Intelligence Sharing platform) |
| Exceptions (1..max) (NOTE 1) |  |
| > Exception ID | Indicating the Exception ID (such as Unexpected long-live/large rate flows and Suspicion of DDoS attack as defined in Table 6.7.5.3-2) of the data flow. |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour. |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > Timestamp | A time stamp of time that the abnormal behaviour occurred. |
| NOTE 1: The Exceptions information and the UE behavioural information as defined in clauses 6.7.2.2 and 6.7.3.2 could help NWDAF to train an Abnormal classifier, which could be used to classify a UE behaviour data into Normal behaviour or Exception. |  |

# * * * * Next change * * * *

### 6.17.2	Input Data

In Input data for the analytics ID is listed in Table 6.17.2-1. A training should be done by performing a specific LCS positioning method and comparing the position result with the ground truth UE location obtained by another method.

NOTE:	There are at least two conceivable options for the ground truth data used for the ML Model training of this analytics ID. One option is to use the known location of Positioning Reference Units (PRUs), another option is to use the positioning result of one positioning method (preferably Global Navigation Satellite System (GNSS) information such as GPS information). Collecting PRU related information by NWDAF is not supported in this release. There could be other options for ground truth data which can be determined in the future releases.

Table 6.17.2-1: Data Collected by NWDAF for Location Accuracy Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE IDs | AMF | (List of) SUPI(s) |
| Finer granularity UE location | LCS NOTE 1 | UE location information. |
| >UE location |  | Geographical location or location in local coordinate (as defined in TS 23.032 [34]) |
| >positioning methods |  | The positioning methods used by the LMF when the location measurement was made |
| >LCS QoS |  | LCS QoS accuracy as defined in clause 4.1b in TS 23.273 [39] |
| >Indoor/outdoor indication |  | Indication of indoor/outdoor when LMF can distinguish indoor/outdoor environment |
| >NLOS/LOS indication |  | Indication whether the UE measurements were based on LOS/NLOS measurements |
| >Timestamp |  | A time stamp of time associated with the UE location. |
| UE location (ground truth) | AF | Timestamped UE positions |
| >UE location | NOTE 2, | GNSS location information |
| >Timestamp | NOTE 3 | Time stamp for the UE location |
| >Accuracy |  | The accuracy of the GNSS location information |
| NOTE 1: The procedure to collect location data using LCS is described in clause 6.2.12. NOTE 2: How the AF obtains timestamped GNSS location information of a UE ID is out of 3GPP scope. Whether to trust the UE location from AF is up to the implementation. NOTE 3: When AF is deployed in the untrusted domain, NEF is employed to mediate the interactions between NWDAF and AF via the Naf_EventExposure_Subscribe service specified in clause 5.2.19.2 of TS 23.502 [3], as described in clause 6.2.8.2.3. |  |  |

# * * * * End of changes * * * *