SA WG2 Meeting #171	S2- 2508285

13 – 17 October 2025, Wuhan, China

Source:	LG Electronics

Title:	KI#4: Interim agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#4.

Discussion

For KI#4 "Sensing Data and the Associated Information Collection and Transport", 11 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Sensing Data Collection Method | Sensing Result Generation Function | Associated Information Usage for Sensing Result Generation |
| --- | --- | --- | --- |
| #1 | Direct connection between SF and SE(s); supports large data transport; QoS guaranteed | SF generates sensing result; SE may provide sensing results | Required; no details |
| #3 | SEs report sensing data to SF (via path established by SF-SE) | SF processes sensing data and determines Sensing Result; SF can be split into SCF and SPF | Required; no details |
| #5 | SEs-AMF-SF flow; SEs perform sensing measurement; sensing data first sent to AMF; AMF forwards data to SF | SF processes sensing data and determines sensing result | Not mentioned |
| #18 | Data connection between SE and SF (direct data connection for gNB) | SF processes sensing data via data connection | Required; no details |
| #21 | Direct Nx interface between NG-RAN and SF | SF generates sensing result based on sensing report from NG-RAN | Required; Correlation ID and sensing report |
| #22 | Direct data tunnel between gNB and SF via sensing control signalling; | SF generates sensing result based on sensing data and associated information | Required; Sensing Service ID for binding sensing data; config and service requirement info. |
| #23 | SEs provide sensing data to SePF(s) | Multiple SePFs process data and generate sensing results; | Not mentioned; |
| #24 | SE(s) sends sensing data to SDCF via data connection | SSMF may process data to produce sensing result | Required; SDCF notifies SSMF with managed data |
| #25 | SCF decides to establish direct gNB-SPF connection | SPF processes sensing data received from gNB and generates sensing result; SCF manages session context and SPF selection | Required; no details |
| #26 | gNBs report data via data tunnel to SPF; SCF provides configuration and task parameters | SPF aggregates sensing data and generates sensing result; SCF receives and exposes sensing result; SDMF stores results | Required; Sensing Task ID used for correlation; SCF checks SDMF for existing results |
| #34 | Direct connection between RAN and SF (or SCF/SPF if split applies) | SF (or SPF if split applies) processes sensing data and generates result | Required; no details |

Summary

- 9 solutions (Sol#1, Sol#3, Sol#18, Sol#21, Sol#22, Sol#24, Sol#25, Sol#26, Sol#34) support direct connection between SF and SE as sensing data collection method. - All of solutions agree with new sensing function which generate sensing results. - Most of solutions (Sol#1, Sol#3, Sol#18, Sol#21, Sol#22, Sol#24, Sol#25, Sol#26, Sol#34) need to be able to provide associated information for sensing result generation. - The associated information for sensing result generation is FFS and can be determined based on coordination with RAN WG.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#4

The interim agreements on principles for KI#4 are as follows:

-	The Sensing Function is responsible for generating the sensing result.

-	Sensing data and/or the associated information are collected from Sensing Entity(ies) to Sensing Function(s) via a direct connection for supporting sensing result generation.

7.2	Topics for further consideration

7.2.Y	Topics for further consideration for KI#4

Topics for further consideration for KI#4 are as below:

-	How to transmit sensing data between Sensing Function(s) and Sensing Entity(ies).

-	What sensing data and associated information are transferred via a direct connection.

-	How to support the transport of large volumes of sensing data without affecting other services, and how to ensure QoS.

-	Where and how sensing data is aggregated and processed (e.g., at Sensing Entity(ies), Sensing Function(s)).

* * * * End of Changes * * * *