SA WG2 Meeting #171	S2-2508416

Wuhan, China, October 13 - 17, 2025

Source: 	vivo

Title: 	Interim agreements on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides interim agreements on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection.

1	Discussion

According to the TR 23.700-14 V1.0.0, there are 10 solutions (#1, #3, #6, #13-#17, #35, #36) related to the KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection.

-	Sol#1, Sol#3, Sol#4 mentions that the Sensing Entity selection is performed by Sensing function and states the detailed handling refers to other solutions of KI#3. The Sol#4 emphases the SE discovery and selection based on the target sensing service area.

-	Sol#13 and #17 is under the AMF involved architecture, the interaction between AF and SE is via the AMF for control signalling. This solution involves the SF, AMF and SE discovery and selection. Among of them, the NEF is responsible for the SF discovery and selection based on Target sensing service area by local configuration or querying the NRF. The SF is responsible for the AMF based on the Target sensing service area by local configuration or querying the NRF. SF is responsible for the SE discovery and selection based on the Target sensing service area by querying the AMF, AMF returns the matched SE to the SF. Before that the SE provides it sensing info (e.g., ID, capability) to the AMF during the NG-setup procedure for subsequent SE discovery and selection. The NF profile of the SF in the NRF can include Supported sensing area, supported sensing QoS, Supported sensing service type.

-	Sol#14 is related to the SF discovery and selection from trusted AF and un-trusted AF perspective. The SF registers its NF profile (NF Type, supported sensing service areas, supported sensing modes, Supported sensing types) into the NRF for subsequent discovery and selection from AF/NEF.

-	Sol#15 proposes the SF keeps the inventory of all available Sensing Entities and their capabilities, when receives the request, the SF selects the SE to be activated. In this solution, the SF registers its supported capability (sensing area) and service type (e.g. motion monitoring, object detection and tracking) in the NF profile to the NRF. SE registers its sensing capability (Supported sensing area, Sensing Transmitter and/or Sensing Receiver.) to the SF.

-	Sol#16 proposes 2 options for SE discovery and selection. In option#1, the SF may select a primary Sensing Entity and delegates further decisions like sensing mode, sensing transmitter(s) and sensing receiver(s) selection to the primary Sensing Entity. In Option 2: the SF may select one or more Sensing Entities to act as sensing transmitter(s) and sensing receiver(s) and the sensing mode. Before that the SE initiates a association procedure and provides its SE ID, sensing capability information, supported sensing service area, supported sensing service type to the SF for subsequent SE discovery and selection.

-	Sol#35 involves direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF). The SE info (capability of providing sensing service, sensing service as Tx or Rx or both Tx and Rx, supported Area) is configured in the SF by OAM. The SF selects the SE based on the sensing service request and the SE info configured by the OAM. In case of the indirect architecture, the AMF reports the SE (NG-RAN) ID which has connection to the SF, after the SE (NG-RAN) selection, the SF selects the SE based on the connection between SE and AMF.

-	Sol#36 proposes a Non-Allowed Area for sensing service, this Non-Allowed Area info is provided by the SF to the SE, when sensing object is approaching Sensing Non-Allowed Area, or the sensing object is out of coverage, the SE notifies the SF for sensing task pausing or revocation.

Based on above solution summarize, the following observations are obtained:

Observation 1: A new function (e.g., Sensing Function, SF) is introduced, and it is the SF responsible for SE discovery and selection.

Oberservation2: NEF is responsible for the SF discovery and selection based on local configuration or by querying the NRF according to the sensing service requirement from the AF.

- The sensing service requirement includes:

-	the Target sensing service area,

-	required sensing QoS, which indicates the required QoS for sensing result, e.g. the required accuracy of sensing result

-	sensing service type (e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2])

- The NF profile of the SF includes:

-	Supported sensing area

-	Supported sensing service type (e.g. motion monitoring, object detection and tracking)

-	Supported sensing QoS, e.g. which indicates the accuracy for the sensing result generation which the SEMF supports

- By querying the NRF for SF discovery and selection, the NEF provides the sensing service requirement as filter to the NRF, the NRF returns the matched SF entity info to the NEF as in NF discovery procedure of TS23.502.

Observation 3: Two connection architecture exist including direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF).

-	In direct connection architecture (SE-SF):

-	for the SE discovery and selection:

-	the SE provides/registers its sensing related info to the SF or the sensing related info of the SE is configured in the SF by OAM, the sensing related info includes:

-	SE ID

-	SE capability (e.g., supported sensing area)

-	SF discovers and selects the SE based on Target sensing service area indicated by the AF and the SE supported sensing area.

-	In indirect connection architecture (SE-AMF-SF):

-	for the SE discovery and selection:

-	Option1: AMF assisted SE discovery and selection (as in Sol#13, #17)

-	the SE registers/provides its sensing related info via to the AMF, the SF requests the AMF returns the SE which is in the Target sensing area

-	Option2: SE registers/provides its sensing related info via the AMF to SF in advance, SF discovers and select the SE based on SE registered sensing related info and the Target sensing area. (as in Sol#16). This option needs the AMF to configure the SF info in advance for transferring the SE info.

-	For AMF discovery and selection:

-	Option1: SF discover and selects the AMF by querying the NRF based on the Target sensing area (as in Sol#13)

-	Option2: SF discover and selects the SE first and then selects the AMF which has the NG connection to the selected SE. (As in solution #35). This option needs the AMF to configure the SF info and reports the SE info which has NG connection to the AMF in advance.

Based on above observation, it proposes the following principles for SE and sensing related function discovery and selection in normative work:

-	Both direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF) is considered in normative work for SE and sensing related function discovery and selection.

-	The following info (i.e., sensing service requirement) indicated by AF in the sensing service request is used for the SE and sensing related function discovery and selection.

-	Target sensing service area

-	Required sensing QoS, which indicates the required QoS for sensing result, e.g. the required accuracy of sensing result

-	Sensing service type, indicates the required sensing service type, e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2]

-	The NF profile of the SF includes the following info:

-	Supported sensing area;

-	Supported sensing service type, e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2]

-	Supported Sensing QoS: which indicates the accuracy for the sensing result generation which the SF supports

-	In direct connection architecture (SE-SF):

-	For SE discovery and selection:

-	The SF is responsible for the SE discovery and selection.

-	The SE registers/provides the following info to the SF when e.g., the NG setup between the SE and SF

-	SE ID (i.e., the gNB ID)

-	Sensing capability, including the supported sensing area

-	The SF discovers and selects the SE based on the SE provided Sensing capability and sensing requirement indicated by AF, including, e.g.,:

-	The SF discovers and selects the SE which supported sensing area matches the Target sensing service area

-	For SF discovery and selection:

-	The NEF/AF discovers and selects the SF based on local configuration or by querying the NRF according to the AF indicated sensing service requirement as above.

-	The SF may determine/updates its NF profile based on the Sensing capability of the SE which has NG connection with the SF, including, e.g.,

-	the Supported sensing area of the SF is the sum of/constructed by the Supported sensing area of the SE which has NG connection with the SF

-	In indirect connection architecture (SE-AMF-SF):

-	For SE discovery and selection:

-	The SF is responsible for the SE discovery and selection.

-	The SE registers/provides the following info to the AMF when e.g., the NG setup between the SE and AMF

-	SE ID (i.e., the gNB ID)

-	Sensing capability, including the supported sensing area

-	The SF discovers and selects the SE by querying the AMF by providing the e.g., Target sensing service area

-	The AMF returns the matched SE info (i.g., the gNB ID, supported sensing capability) to the SF and the SF make the SE selection based on the SE supported sensing capability.

-	For AMF discovery and selection:

-	The SF is responsible for the AMF discovery and selection.

-	The SF discovers and selects the AMF based on local configuration or by querying the NRF according to the AF indicated Target sensing service area as above.

-	For SF discovery and selection:

-	The NEF/AF is responsible for the SF discovery and selection;

-	The NEF discovers and selects the SF based on local configuration or by querying the NRF according to the AF indicated sensing service requirement as above.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

7	Interim agreements

7.1	Agreed Principles

7.1.3	Agreed Principles for KI#3

The following interim agreements on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection, are made:

General Principles:

-	Both direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF) is considered in normative work for SE and sensing related function discovery and selection.

NOTE:	Architecture decision depends on the agreement of KI#1.

-	The following info (i.e., sensing service requirement) indicated by AF in the sensing service request is used for the SE and sensing related function (e.g., SF) discovery and selection.

-	Target sensing service area

-	Required sensing QoS, which indicates the required QoS for sensing result, e.g. the required accuracy of sensing result

-	Sensing service type, indicates the required sensing service type, e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2]

-	The SF registers itself in the NRF with its NF profile, this is to enable the discovery of SF instances e.g. by an NEF. 	The NF profile of the SF includes the following info:

-	SF ID/address, NF type

-	Supported sensing area;

-	Supported sensing service type, e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2]

-	Supported Sensing QoS: which indicates the accuracy for the sensing result generation which the SF supports

In direct connection architecture (SE-SF):

-	For SE discovery and selection:

-	The SF is responsible for the SE discovery and selection.

-	The SE registers/provides the following info to the SF when e.g., the NG setup between the SE and SF

-	SE ID (i.e., the gNB ID)

-	Sensing capability, including the supported sensing area

-	The SF discovers and selects the SE based on the SE supported Sensing capability and sensing requirement indicated by AF, including, e.g.,

-	The SF discovers and selects the SE which supported sensing area matches the Target sensing service area

-	For SF discovery and selection:

-	The NEF/AF discovers and selects the SF based on local configuration or by querying the NRF according to the AF indicated sensing service requirement as above.

-	The SF may determine/updates its NF profile based on the Sensing capability of the SE which has NG connection with the SF, including, e.g.,

-	the Supported sensing area of the SF is the sum of/constructed by the Supported sensing area of the SE which has NG connection with the SF

In indirect connection architecture (SE-AMF-SF):

-	For SE discovery and selection:

-	The SF is responsible for the SE discovery and selection.

-	The SE registers/provides the following info to the AMF when e.g., the NG setup between the SE and AMF

-	SE ID (i.e., the gNB ID)

-	Sensing capability, including the supported sensing area

-	The SF discovers and selects the SE by querying the AMF by providing the e.g., Target sensing service area

-	The AMF returns the matched SE info (i.g., the gNB ID, supported sensing capability) to the SF and the SF make the SE selection based on the SE supported sensing capability.

-	For AMF discovery and selection:

-	The SF is responsible for the AMF discovery and selection.

-	The SF discovers and selects the AMF based on local configuration or by querying the NRF according to the AF indicated Target sensing service area as above.

-	For SF discovery and selection:

-	The NEF/AF discovers and selects the SF based on local configuration or by querying the NRF according to the AF indicated sensing service requirement as above.

# * * * * End of change* * * *