SA WG2 Meeting #171	S2-2508431

13 - 17 October, 2025, Wuhan, China                  	(revision of S2-2506944)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0028 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AMF AIoT Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 group has defined the following AIoT Message Types in TS 38.413: INVENTORY REQUEST, INVENTORY RESPONSE, INVENTORY FAILURE, INVENTORY REPORT, COMMAND REQUEST, COMMAND RESPONSE, COMMAND FAILURE, AIOT SESSION RELEASE COMMAND, AIOT SESSION RELEASE COMPLETE, and AIOT SESSION RELEASE REQUEST. It is proposed to clarify the Message Type in AMF AIoT service definition to align with RAN specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the Message Type in AMF AIoT service definition to align with RAN specification and modify the corresponding AIOTF and NG-RAN interaction procedure for indirect connectivity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Message Types in AMF AIoT service definition are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data  towards NG-RAN or AIoT devices.

Inputs, Required:

1)	NGAP AIoT Information to deliver to NG-RAN.

2)	NG-RAN ID.

3)	AIoT NGAP Message Type ("Inventory Request", "Command Request", or "Session Release Command").

4)	AIOTF Identifier and Correlation Identifier, this is to allow identifying the association between NG-RAN and AMF.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

SECOND CHANGE

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	NGAP AIoT Information received from NG-RAN.

2) 	AIoT NGAP Message Type ("Inventory Response", "Inventory Report", "Inventory Failure", "Command Response", "Command Failure", "Session Release Response" or "Session Release Request").

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

END OF CHANGES