SA WG2 Meeting #S2-171	S2-2508648

13 - 17 October, 2025, Wuhan, China	(was S2-250xxxx)

Source:	China mobile

Title:	Updates on solution 15

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes the updates on solution 15 for Sensing_ARC / Rel-20.

1.	Discussion

This solution 15 mainly aims to address key issue#3 on how to perform SF (Sensing Function) and Sensing Entities (sensing transmitters and receivers) discovery and selection. Generally, two phases need to be considered during the whole sensing service procedure, one is how NEF selects a SF and the other one is how SF selects Sensing Entitie(s).

For the first phase, it is the NEF to perform SF discovery and selection based on interaction with NRF or local configuration. The NEF should determine which SF to serve a sensing request from AF based on serving area of the SF and Target Sensing Service Area requested by the AF and ensure that the selected SF can serve the target area of the required sensing service.

-  Before this, Sensing Entities should associate themselves and report their capability, location or serving area information to a SF. SF may aggregate the information from the connected Sensing Entities and generate the SF serving area and capability information, e.g., Supported Sensing Service area, supported sensing service type/scenario (UAV detection or transport traffic monitoring, etc.,), service quality level, etc., and may register itself to the NRF.

-  After the NEF authorizes the sensing service request from the AF, the NEF performs the discovery of SF as defined in TS 23.502 [x] clause 4.17.4. The NEF determines the SF instance(s) based on sensing service requirement (e.g., Target Sensing Service Area, service type, service quality requirement, etc.,) from AF and the information received from NRF like supported sensing service area and sensing capability, etc.

For the second phase, it is the selected SF to selects Sensing Entitie(s).

- The SF should select Sensing Entities based on their sensing service area, sensing capability, etc., and sensing service requirement (e.g., Target Sensing Service Area, service type, service quality requirement, etc.,) from AF, which ensure selected Sensing Entities can serve the sensing service.​

In general, the factors for SF Selection (performed by NEF) and Sensing Entity (performed by SF) selection are listed as follows:

-	Supported Sensing Service area.

- 	Sensing Capabilities (e.g., supported service type).

-	Sensing service requirement (e.g., target Sensing Service Area, service type, service quality requirement, etc.,) from AF.

Observation: There are some detail information referring to the sensing entities register and NEF interacting with NRF for SF discovering need to be updated in this solution.

Proposal : It is proposed to add some detail description in sol 15

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-14.

*** Start of the change ***

6.15	Solution #15: Sensing Function selection and Sensing Entity selection based on Sensing Entity Capabilities and Conditions

6.15.0	High-level solution principles

In this solution the SF (Sensing Function) keeps inventory of all available Sensing Entities and their capabilities. The SF also performs selection of SE (Sensing Entities) for every sensing service request. The NRF is used for discovery of the SF.

6.15.1	Description

This solution addresses Key Issue #3 and Key Issue #6.

This solution proposes a new network node Sensing Function (SF).  The SF register/deregister/update itself to the NRF as defined in clause 4.17.1.1, 4.17.2, and 4.17.3 of TS 23.502 [10], respectively. In case of registration or update the Sensing Function include its supported sensing capabilities to the NRF to be stored in the SF profile at the NRF. The NRF is used for the discovery of SF's and may be used by any NF including Sensing Entities, trusted AF or by the NEF for any AF's. The NEF or trusted AF performs SF discovery and selection based on interaction with NRF or local configuration. The NEF should determine which SF to serve a sensing request from AF based on serving area of the SF and Target Sensing Service Area requested by the AF and ensure that the selected SF can serve the target area of the required sensing service.

The capabilities in the SF profile may include:

-	Supported area

Sensing service type (e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2]).The Sensing Entities register/deregister to the SF and the SF keeps an inventory of available Sensing Entities and their capabilities. The Sensing Entities discover the SF via local configuration or use the NRF for that.Sensing Entities should associate themselves and report/register their capability, location or serving area information to a SF. SF may aggregate the information from the connected Sensing Entities and generate the SF serving area and capability information, e.g., Supported Sensing Service area, supported sensing service type/scenario (UAV detection or transport traffic monitoring, etc.,), service quality level, etc.

Editor's note:	Weather and how a Sensing Entity can register with multiple SF or not is FFS.

For every new or changed sensing service request and after authorization and revocation that will be defined in Key Issue#2 the SF decides if any activation, deactivation or configuration changes is needed to any Sensing Entities. The decision logic is up to SF implementation but a set of capability parameters from the sensing entities will be specified and how the SF activate, deactivate or reconfigure the Sensing Entity.

The profile for each Sensing Entity may include the following capabilities:

-	Sensing Transmitter and/or Sensing Receiver.

-	Supported sensing area.

NOTE:	Additional capabilities will be decided by RAN.

In general, the factors for SF Selection (performed by NEF), Sensing Entity (performed by SF) selection are listed as follows:

-	Supported Sensing Service area.

- 	Sensing Capabilities (e.g., supported service type).

-	Sensing service requirement (e.g., target Sensing Service Area, service type, service quality requirement, etc.,) from AF.

6.15.2	Procedures

6.15.2.1	Sensing entity selection Procedure

The procedure for the discover and selection mechanism:

Figure 6.15.2.1-1: Procedure to support sensing service selection

1.	NEF sends the Nnrf_NFDiscovery_Request to the NRF.

2.	NRF respond with supported SF(s) and their capabilities.

3.	NEF may select a SF based on local configuration or supported SF capabilities and the consumer's request

4.	NEF sends Nsf_Activate_Subscribe to the SF to initiate a sensing session with the based on the sensing consumer's request.

5.	SF verify that the AF is authorized for the requested sensing service request as part of solutions of KI#2

6.	SF decide from its inventory of available Sensing Entities which one to activate/deactivate or reconfigure based on the decided sensing service request.

7.	SF sends Activate to activate all needed Sensing Entities that will be included for the sensing service. This may also include deactivation or reconfiguration of already active Sensing Entities.

8.	Sensing Entity perform the requested configuration and starts the requested sensing service

9.	Sensing Entity deliver the requested sensing data according to solutions for KI#4 and KI#5.

6.15.2.2	Sensing Function registration procedure

The procedure for Sensing Function Registration:

Figure 6.15.2.2-1: Procedure to support Sensing Entity registration

1.	SF sends Nnrf_NFManagement_NFRegister and include the supported capabilities as defined in clause 6.15.1 that will be stored in the SF profile at the NRF.

6.15.2.3	Sensing Entity registration procedure

The procedure for Sensing Entity Registration:

Figure 6.15.2.3-1: Procedure to support Sensing Entity registration

1.	Sensing Entities sends Register and include supported capabilities to the SF as defined in clause 6.15.1.

Editor's note:	It is FFS if additional configuration or local conditions at the Sensing Entity is included with the capabilities.

6.15.3	Impacts on Services, Entities and Interfaces

SF:

-	Inventory of available Sensing entities and corresponding capabilities.

-	Selection logic to select appropriate Sensing Entities and methods based on consumer request.

-	Authorization of sensing service request.

-	Activation/deactivation of Sensing Entities including configuration of Sensing entities.

NRF:

-	Support for storing a profile of the SF capabilities.

NEF:

-	Discovery and selection of SF based on SF profile.

Sensing Entities:

-	Support association/registration with SF.

*** End of the change ***