SA WG2 Meeting #171	S2-2508764

Wuhan, China, October 13 – 17, 2025	(revision of 7663)

Source:	[LG Electronics, Toyota, NEC] NTT DOCOMO

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1.

# Discussion

This paper summarizes the interim agreements for KI#1.

# Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

### 7.1.Y	Agreed Principles for KI#1

### 7.2.Y	Topics for further consideration for KI#1

Topics for further consideration for KI#1 are as below:

Agreed principles for KI#1 are as follows:

-	The following new/additional energy related information is exposed by EIF:

-	In the case of deployment of a single EIF in the network, the PLMN-wide UE categorization data e.g. percentile of the consumption of the UE as proposed by Solution#1 described in clause 6.1.

-	Per UE Application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session as proposed by Solution#12 described in clause 6.12.

NOTE 1:	Solution#1  is applicable if only one EIF is deployed in the network.

-	New formulas aiming at minimizing the impact of idle energy consumption on the calculation as proposed by Solution#10 described in clause 6.10.

NOTE 2:	Solution#10 needs to clarify the accuracy aspect due to the measurement error of the node-level energy consumption.

### 7.2.Y	Topics for further consideration for KI#1

Topics for further consideration for KI#1 are as below:

-	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF, as proposed by Solution#8 and Solution#9 described in clause 6.8 and clause 6.9, respectively.

NOTE 3:	The renewable energy information has dependency on SA5, so needs SA5 feedback to conclude renewable energy aspect.

-	New formulas taking idle energy consumption into account as proposed by Solution#11 described in clause 6.11 as another alternative EC estimation formula in the Annex T of TS 23.501 [2] and leave it to implementation and operator policy to use the appropriate estimation formula by EIF.

-	New formulas aiming at minimizing the impact of idle energy consumption on the calculation as proposed by Solution#10 described in clause 6.10.

NOTE 4:	Solution#10 needs to clarify the accuracy aspect due to the measurement error of the node-level energy consumption.

* * * * End of Changes * * * *