3GPP TSG-WG SA2 Meeting #171	S2-2509139

Wuhan, CN, 13th – 17th Oct, 2025

Source:	CEWiT

Title:	Interim Conclusion proposal for KI#5

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide a basis for future discussion, interim conclusions have been incorporated into Clause 7.

# 1.	Discussion

This contribution proposes the interim agreements for KI#5. The interim agreements are drafted based on the common points across all solutions proposed for KI#5 in 3GPP TR 23.700-14 V1.0.0.

The solutions #1, #2, #3, #19, #23, #27, #28, #29, #30, #31, #32, #33 and #37 are proposed for KI#5. The following is a summary of all the solutions,

- Solution #1s

This solution provides a framework containing Sensing Service consumer, Gateway Function and Sensing Function. The sensing service initiator can be NF or a UE and maybe initiated via a gateway function. The Sensing Function provides the sensing result to sensing service consumer based on sensing service request.

- Solution #2

The entities involved in the sensing result exposure procedure are UE and Sensing Support Function (SSF). The sensing service consumer is UE. A new NF called SSF, supports the interaction with the UE to receive the sensing service request and provide sensing service results. The sensing service request includes sensing service type, and reporting mode (One Time, Periodic, Event-Triggered).

- Solution #3

The entities involved in the sensing result exposure procedure are AF, NEF/Gateway and Sensing Function (SF) (contains Control and Processing). The sensing service consumer is AF/NEF. The AF initiates the procedure for sensing by sending sensing service subscription request to Core network. The solution supports One Time, Periodic and Event-Triggered reporting types. The SF (SCF) sends the sensing result to NEF and NEF notifies the sensing result to the AF.

- Solution #19

The entities involved in the sensing result exposure procedure are Sensing Service Consumer (SSC), Exposure Function, Sensing Management Function (SeMF). The SSC is 3rd party AF or UE. If the SSC is 3rd party AF, then the sensing service request (SSR) is sent via exposure function to SeMF. The sensing result is provided from SPF to SSC UE or 3rd party SSC via Exposure Function.

- Solution #23

The entities involved in the sensing result exposure procedure are Sensing Service Consumer (SSC), Sensing Management Function (SeMF), Sensing Processing Function(s) (SePF). The SSR is triggered by the SSC to SeMF. The sensing results are delivered to SSC by SePF.

- Solution #27

The entities involved in the sensing result exposure procedure are Untrusted AF, NEF, Other NF/Trusted AF and Sensing Function (SF). The sensing service consumer can be 3rd party AF, trusted AF or NF. The SF can receive multiple heterogeneous sensing requests from the sensing service consumers that indicate the type of requested sensing service, sensing area, QoS and optionally, the information about objects to be sensed. The sensing result may contain the object map, which includes object ID, object location information, confidence level of object detection etc. Additionally, the sensing QoS information is also included in the sensing result. The sensing request may be one-time or a subscription for periodic or event-based result notification.

- Solution #28

The entities involved in the sensing result exposure procedure are AF, NEF and Sensing Core Function (SnCF). The sensing request includes sensing request type such as Object Detection, Object Detection along with Tracking within a requested area. The sensing result may include Sensing Result detail level parameter such as notify “result only” or “notify result with associated information”. The NEF forwards the sensing service request to SnCF. The event reporting modes are one time, periodic or event-triggered. The SnCF provides the determined sensing result to NEF and NEF exposes the sensing result to AF.

- Solution #29

The entities involved in the sensing result exposure are AF, NEF, Sensing Control Function (SCF), Sensing Processing Function (SPF). The AF requests the NEF to obtain the sensing result from the network and NEF forwards the request to SCF. The SPF exposes the sensing result to AF directly.

- Solution #30

The entities involved in the sensing result exposure are trusted and untrusted AF, NEF and Sensing Function (SF). The sensing service consumer can subscribe for sensing results via NEF for untrusted AF(s) or directly to SF for trusted AF. The SF is responsible for exposing sensing results to authorized consumers. The reporting modes are one-time delivery, periodic reporting and event-triggered reporting. The sensing service response is provided to AF acknowledging the subscription request. The SF service operations are subscription and notification basis. The sensing service request consists of sensing type, target, sensing quality, reporting mode, and optionally, the sensing assistance information. Each result notification includes sensing result payload, and contextual information.

- Solution #31

The entities involved in the sensing result exposure are trusted and untrusted AF, NEF, Gateway Sensing Management Function (GSMF), Sensing Service Management Function (SSMF) and Sensing Result Storage NF. The GSMF receives sensing service request from trusted AF/NEF. The sensing service request contains consumer ID, sensing service type, target sensing area, expected sensing result, and capability of AF for transferring result over data connection.

If the sensing result is being stored in Sensing Result Storage NF, then the sensing result is exposed to AF directly via data connection or if exposure is based on on-demand generated sensing result, then the SeMF sends the sensing result notification over data connection to the AF.

- Solution #32

The entities involved in the sensing result exposure are AF(s) authorized by NEF, AF(s) in Core Network, NEF, and SeMF. The sensing service consumer can be AF(s) authorized by NEF and AF(s) in Core Network. The sensing service consumer subscribes to the sensing service to obtain the sensing result. The request may contain AF ID, target sensing area, sensing service type, type of event reporting, QoS of sensing result etc. The type of event reporting can be one-time, periodic and event-triggered. The SeMF reports the sensing result to the requesting entity. The sensing service response contains Object(s) Presence Detection i.e. Yes or No, Sensed Object(s) count, Sensed Object(s) definition (e.g. Sensing Object(s) Location, Object ID(s)), Quality Indication of Sensing Result etc.

- Solution #33

The entities involved in the sensing result exposure are trusted and untrusted AF, Gateway Sensing NF and Sensing NF. The AF is allowed as sensing service consumer. The sensing service consumer request can be for one-time, periodic or event-triggered based on Target Sensing Area or for a location of target UE. The gateway sensing NF accepts or rejects the sensing service request and responds to AF (via NEF). The gateway sensing NF sends the sensing result to AF.

- Solution #37

The entities involved in the sensing result exposure are trusted and untrusted AF, NEF and Sensing Control Function (SCF). The AF can be the sensing service consumer. The AF request may contain Target Sensing Area, accuracy and resolution etc. The reporting mode can be one-time, periodic or event-triggered. The sensing service types can be Object detection, Border Crossing Detection, Path Monitoring etc. The sensing result is provided by SCF to AF via NEF.

Based on the above given summary of the solutions, we propose the following interim agreements on KI#5:

- Sensing service exposure involves the entities such as sensing service consumer, i.e 3rd party AF or AF in trusted domain, Sensing NF, and NEF in case of 3rd party AF.

- The sensing service consumer subscribes for the sensing service and gets notified with the sensing service result.

- The sensing service request can contain target sensing area, AF ID, sensing service type, Mode of reporting etc.

- The sensing service result can contain Sensed object related parameters and other contextual information like time stamp, Quality of sensing etc.

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

## 7.1	Agreed Principles

### 7.1.A	KI#5

The following are the interim agreements for KI#5:

The sensing service exposure involves the following entities and procedures:

- Sensing service consumer:

- The sensing service consumers can be,

- AF(s) in the trusted domain within the Core network.

  - The AF(s) subscribes for a sensing service directly to the SeMF

- 3rd party AF(s) authorized by NEF.

  - The AF(s) subscribes for a sensing service to the SeMF via NEF

- NEF:

- The NEF forwards the sensing service subscription request to the selected SeMF

- NEF exposes the sensing result obtained from SeMF to 3rd party AF

- Sensing Management Function (SeMF):

- The SeMF receives the sensing subscription request and initiates sensing procedure towards SE to generate sensing result

- The SeMF sends the sensing results,

- to NEF via notification and NEF exposes the sensing results to AF

- directly to AF(s) in the trusted domain via notification

The sensing service request can contain:

- Target Sensing Area

- AF ID

- Sensing Service Type (e.g. Object Detection, Object Tracking etc.)

- Type of Event Reporting (One Time, Periodic, Event-Triggered)

- Quality requirements of sensing result

The sensing results can contain:

- Object Presence Detection i.e. Yes or No

- Sensed Object count

- Sensed Object definition

- Sensing Contextual information (e.g. Quality of Sensing Result – Confidence, Accuracy and Time Stamp)

# * * * * End of Changes * * * *