SA WG2 Meeting #172	S2-2509908

17 – 21 November 2025, Dallas, US

Source:	LG Electronics

Title:	KI#1: Conclusions and Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusions and interim agreements for KI#1.

Discussion

There were two major open issues (i.e., direct and/or indirect connection, and whether to define Gateway Sensing Function) for KI#1. These issues affect conclusions for other KIs. So, KI#1 needs to be concluded.

2. Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain logical functionalities to perform Sensing Control and Sensing Processing.

NOTE 1:	How to capture the deployment options of the two logical functionalities of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE 2: Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 3:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: Transmission of Sensing data and sensing control signalling between the SF and the SE are supported via direct connection. The SF requests the sensing service operations for the SE(s) and obtains sensing data (and/or results) over the direct connection.

* * * * Start of 2nd Change * * * *

!! All New Texts !!

7.2	Topics for further consideration

7.2.1	Topics for further consideration for KI#1

Topics for further consideration for KI#1 are as below:

-	How to establish the direct connection and which protocol is used for the connection between the SF and the SE.

* * * * Start of 3rd Change * * * *

!! All New Texts !!

8.1	Conclusion on KI#1

The key issue #1 covers “System Architecture to Support Sensing”.

In this Release, normative work will take place for 3GPP access-based Sensing Service, which supports gNB-based sensing operations for aerial object detection and tracking use cases.

The following aspects common to the Sensing Service are concluded as principles for normative work:

-	A new network function, referred to as the Sensing Function (SF) is defined to support Sensing Service.

-	The SF performs the following functionalities:

-	Discovery and (re-)selection of Sensing Entity (SE).

-	Interaction with the SE for sensing-related operations.

-	Exposure of sensing result to the AF directly or via the NEF.

-	The SF may include logical functionalities that play the roles of Sensing Control and Sensing Processing.

NOTE 1:	The details of SF discovery and selection and sensing result exposure to AF are addressed in KI#3, KI#5, respectively.

-	The gNB is the only entity that acts as the SE.

-	The SE performs the following functionalities:

-	Transfer sensing data to the SF.

-	Execute sensing measurements according to configuration from the SF.

NOTE 2:	The details of configuration parameter are addressed in KI#6.

-	Transmission of Sensing data and sensing control signaling between the SF and the SE are supported via direct connection.

NOTE 3:	The details of the sensing data collection and transfer are addressed in KI#4.

-	The SF requests and controls sensing operations for the SE(s) and obtains sensing data (and/or results) over the direct connection.

-	The SF provides the sensing result to the sensing service consumer (e.g., AF) according to the sensing service request.

-	NO dedicated storage NF is needed to store the sensing data and sensing result.

Figure 8.1-1 shows Sensing Service Architecture in the reference point representation.

Figure 8.1-1: Sensing System Architecture via Direct Connection

The Nsf interface is used between the SF and other 5GC NFs to handle sensing service-related requests from the sensing service consumer (e.g., AF) according to the sensing service request.

The Nx interface is defined between SF and SE to support direct connection. The SF transfers sensing data and sensing control signalling via the Nx interface.

* * * * End of Changes * * * *