SA WG2 Meeting #172	S2-2509912

17 – 21 November 2025, Dallas, US

Source:	LG Electronics

Title:	KI#5: Conclusions and Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusions and interim agreements for KI#5.

Discussion

[Summary] KI#5: Sensing Result Exposure

For KI#5 "Sensing Result Exposure", 13 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Exposed Sensing Result Type (w/ or w/o contextual info) | Sensing Result Exposure Method (e.g., one-time, periodic, event-trigger) |
| --- | --- | --- |
| #1 | Sensing result generated by SF (based on sensing data + associated info from SE); contextual info: Not mentioned | Configured in Sensing Service Request (e.g., periodicity, event report criteria) |
| #2 | Sensing result determined by SSF (based on sensing data + associated info collected by SSF); contextual info: Not mentioned | Reporting mode requested by UE (e.g., one-time, periodic, event-trigger) |
| #3 | Sensing result determined by SF or SPF (from 3GPP sensing data); contextual info: Not mentioned | Exposure via NEF to AF (one-time, periodic, or event-trigger, depending on sensing service request from AF) |
| #19 | Sensing result produced either by SPF or SE; contextual info: Not mentioned | Sensing result exposure alternatives (from SPF to SSC UE, from exposure function to 3rd Party SSC AF); Sensing result exposure method is FFS |
| #23 | Sensing result(s) generated by one or multiple SePFs; contextual info: Not mentioned | Sensing result delivered to SSC; Detailed result delivery mechanism: No details (FFS) |
| #27 | Sensing result generated by SF (SCF+SPF); contextual info: included (e.g., target object type, target object mobility profile, environment of sensing) | Sensing result exposure to sensing service consumer (i.e., trusted/untrusted AF or NF); (one-time, periodic, or event-based result notification) |
| #28 | Sensing result determined by SnCF; (Notify sensing result only, notify sensing result with associated information); contextual info: Not mentioned | AF specifies optional event reporting mode (one-time, periodic, event-triggered) |
| #29 | Sensing result generated by SPF; Sensing result types: Real-time sensing results; Subscription/Notification sensing results; contextual info: Not mentioned | Real-time sensing result via data connection; Event-triggered sensing result via CP or data connection |
| #30 | Sensing result generated by SF; Sensing result payload (e.g., location, speed); contextual info: included (e.g., timestamp, accuracy estimation, confidence level, serving area) | Direct exposure for trusted AFs or indirect exposure for untrusted AFs via NEF; (one-time, periodic, event-trigger reporting) |
| #31 | Sensing result generated by SSMF or retrieved from Sensing result storage NF; Sensing result types: sensing result format; contextual info: Not mentioned | Sensing result transferring mode: CP or data connection; |
| #32 | Sensing result determined by SeMF; Sensing service response: object presence, object count, object definition, quality indicators, velocity, time stamp; contextual info: Not mentioned | Sensing result request of the sensing service consumer; Event reporting type: one time, periodic, event triggered, maximum number of event reports |
| #33 | (distributed) Sensing NF calculate sensing result (based on sensing data with sensing operation ID + associated info); contextual info: operation ID, location info | SSC may request sensing service (for a one-time, periodic, and event-triggered report); sensing result exposure use signalling or data connection depending on configuration. |
| #37 | Sensing result generated by SPF/SCF (based on 3GPP sensing data + associated sensing info); contextual info: supported (No details) | Configured in Sensing Service Request (supports one-time, periodic, and event-triggered reporting) |
| Summary |  |  |
| - All solutions agree that (existing or new) sensing function(s) generate sensing results based on sensing data and/or associated information collected from sensing entity(ies). - 9 solutions (Sol#1, Sol#2, Sol#3, Sol#27, Sol#28, Sol#30, Sol#32, Sol#33, Sol#37) explicitly define sensing result exposure methods such as one-time, periodic, and event triggered reporting. - Most solutions (Sol#1, Sol#2, Sol#3, Sol#19, Sol#23, Sol#28, Sol#29, Sol#31, Sol#32, Sol#37) mostly left the use of contextual information as FFS or not mentioned, requiring further study. |  |  |

[Additional Issue] Sensing Service Request Message

The parameters related to the sensing service request need to be identified to support sensing result exposure. The detailed definition of these parameters and their mapping to sensing service procedures will be further discussed during the normative phase.

| Message | Parameters |
| --- | --- |
| Sensing Service Request (AF→SF) | - AF Identifier, Target Sensing Service Area, Sensing Service Type. - (Optional) Sensing Correlation ID. - (Optional) Required Sensing QoS (e.g., accuracy level of sensing result, sensing result format, confidence level, maximum response time, sensing time window). - (Optional) Sensing Reporting Mode (i.e., one-time, periodic, and event-triggered). - (Optional) Event-trigger Conditions. - (Optional) Sensing Object Information (e.g., object status). - (Optional) Required Sensing Contextual Information. |

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.5	Agreed Principles for KI#5: Sensing Result Exposure

The following principles are agreed for KI#5:

-	The Sensing Function (SF) is responsible for generating sensing results based on sensing data and sensing associated information collected from Sensing Entity(ies) (SEs).

-	The SF provides the sensing results either directly or via the NEF to the AF.

NOTE 1:	It is assumed that a sensing service requester and a sensing service consumer are the same entity in this Release.

-	The exposure of sensing result supports sensing reporting modes (i.e., one-time, periodic, and event-triggered) according to the sensing service request from Sensing Service Consumer(s).

NOTE 2:	The conditions for event-triggered sensing report will be specified during the normative phase.

-	The sensing service request from the AF to the SF includes the following parameters:

-	AF Identifier.

-	Target Sensing Service Area.

-	Sensing Service Type.

-	(Optional) Sensing Correlation ID.

-	(Optional) Required Sensing QoS (e.g., accuracy level of sensing result, sensing result format, confidence level, maximum response time, sensing time window).

-	(Optional) Sensing Reporting Mode (i.e., one-time, periodic, and event-triggered).

-	(Optional) Event-trigger Conditions.

-	(Optional) Sensing Object Information (e.g., object status).

-	(Optional) Required Sensing Contextual Information.

NOTE 3:	The details of parameters included in the sensing service request will be further discussed during the normative phase.

* * * * Start of 2nd Change * * * *

!! All New Texts !!

7.2	Topics for further consideration

7.2.5	Topics for further consideration for KI#5

Topics for further consideration for KI#5 are as below:

-	How to define the accuracy level requirements of sensing results, including confidence levels and methods for processing.

-	Whether and what specific types of contextual information are included with sensing results.

-	How to determine the appropriate sensing result exposure method (e.g., via control plane, via user plane), considering factors such as data volume, latency, QoS, and Sensing Service Consumer capabilities.

-	Whether and how sensing results are stored, and how stored results can be reused for new sensing service requests.

* * * * Start of 3rd Change * * * *

!! All New Texts !!

8.5	Conclusion on KI#5

The key issue #5 covers “Sensing Result Exposure”.

The following aspects common for sensing service are concluded as principles for normative work:

-	The Sensing Function (SF) is responsible for generating sensing results based on sensing data and sensing associated information collected from Sensing Entity(ies) (SEs).

-	The SF provides the sensing results either directly or via the NEF to the AF.

NOTE 1:	It is assumed that a sensing service requester and a sensing service consumer are the same entity in this Release.

-	The exposure of sensing result supports various reporting modes (i.e., one-time, periodic, and event-triggered) according to the sensing service request from Sensing Service Consumer(s).

NOTE 2: The conditions for event-triggered sensing report will be specified during the normative phase.

-	The sensing service request from the AF to the SF includes the following parameters:

-	AF Identifier.

-	Target Sensing Service Area.

-	Sensing Service Type.

-	(Optional) Sensing Correlation ID.

-	(Optional) Required Sensing QoS (e.g., accuracy level of sensing result, sensing result format, confidence level, maximum response time, sensing time window).

-	(Optional) Sensing Reporting Mode (i.e., one-time, periodic, and event-triggered).

-	(Optional) Event-trigger Conditions.

-	(Optional) Sensing Object Information (e.g., object status).

-	(Optional) Required Sensing Contextual Information.

NOTE 3:	The details of parameters included in the sensing service request will be further discussed during the normative phase.

NOTE 4:	The information related in sensing results (e.g., accuracy level of sensing result, confidence value) will be discussed during the normative phase.

NOTE 5:	The specific types of contextual information to be included with sensing results will be discussed during the normative phase.

NOTE 6:	The appropriate sensing exposure method (e.g., via control plane, via user plane), considering factors such as data volume, latency, QoS, and Sensing Service Consumer capabilities will be discussed during the normative phase.

NOTE 7:	Whether and how sensing results are stored, and how stored results can be reused for new sensing service requests will be discussed during the normative phase.

* * * * End of Changes * * * *