3GPP TSG SA WG2 Meeting #172	S2-2510011

Dallas, USA, 17 – 21 November 2025	(revision of S2-250xxxx)

Source:	Lenovo

Title:	KI#1: Conclusions

Document for:	Agreement

Agenda Item:	20.3.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Conclusions are proposed for the study for Key Issue 1 taking into account feedback received from RAN2.

# 1. Discussion

## 1.1	 Analysis of RAN2 LS response

On Paging the UEs in IDLE mode for providing them with data reporting configuration/triggering data collection procedure (Question 1.1)

Question 1.1

Some solutions in SA2 TR propose to page UEs in IDLE mode to provide them with data reporting configuration or to trigger data collection procedure.

SA2 would like to ask RAN2 whether paging UEs in IDLE mode, either to configure them for data collection reporting or to trigger the data collection procedure, is required or is up to SA2 to decide.

RAN2 Answer to Question 1.1

RAN2 concluded that there is no need to support paging of UEs in IDLE or INACTIVE mode to provide them with data reporting configuration or to trigger data collection procedure.

The LS response implies that there should be no impacts on the paging procedure defined in 3GPP TS 38.413, that is, no additional paging causes are needed to be supported, to page such UEs. It is also understood from the LS response that paging UEs for the purpose of providing only data reporting configuration should be avoided.

On data collection upon UE connected mode mobility (Question 1.2)

Question 1.2

Many solutions assume that UE data collection will be needed for some specific geographical area. Note this does not necessarily imply that gNB will receive an area in any message from CN.

- Does RAN2 confirm this assumption?

- Can RAN2 confirm that data collection continuation is required upon UE connected mode mobility, as long as the UE stays within the initial geographical area?

RAN2 Answer to Question 1.2

In Rel-19 RAN2 did not study whether data collection on some specific geographical area is needed and thus RAN2 did not conclude that continuation of measurements for data collection is required upon UE connected mode mobility as long as the UE stays within the initial geographical area.

However, RAN2 agreed that the data transfer of the collected data over the user plane should be continued during connected mode mobility.

As part of the Key Issue 1 conclusions, it is proposed that the data transfer request sent from the UE-side server/Data Collection Function may include requirements (e.g. geographical area) where data should be collected. Based on such requirements the UE can trigger the RAN for measurement configuration or start/stop data collection when the UE is in the applicable area. Such approach is inline with the RAN2 assumptions where the UE can indicate preferred data configuration option (as indicated in the response of Question 2.1). Any further dependency can be discussed after RAN2 continues to develop their solution and provide further feedback.

Regarding the RAN2 agreement on the data transfer of the collected data over the user plane during during connected mode mobility, all current SA2 solutions assume IP connectivity between the UE and the Data Collection Function hence data transfer will not be affected during connected mode mobility

On the end-2-end data collection procedure and dependency between data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and RAN (Question 2.1)

Question 2.1

The different end-to-end solutions in the SA2 TR consider different entities capable of triggering UE data collection procedure: some solutions assume that the UE side model training server (OTT server) triggers the overall data collection procedure (including data collection configuration and data transfer) via the core network, while other solutions consider that the UE triggers data collection procedure.

SA2 would like to ask RAN2 whether the trigger for UE data collection is possible and required from UE side model training server (OTT server), from the UE, or from both.

In addition, SA2 would like to clarify whether there are any dependencies between a data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and NG-RAN.

RAN2 Answer to Question 2.1

In Rel-19 RAN2 agreed that for UE-side data collection for UE-side model training, the RAN can provide a list of candidate data configuration options, and the UE can indicate the preferred data configuration options from this list. The UE can also indicate its preference to start or stop the data collection, after being enabled by the gNB to do send start/stop indications. However, the RAN can also provide UE with data collection configuration or release the data collection configuration at any point in time, with or without UE request.

RAN2 did not discuss how the RAN creates the list of candidate data configuration options, and how RAN decides when to configure a UE to start or stop the data collection. RAN2 assumes that the UE-side server via the CN or the OAM may provide some information that is used by the RAN to create the list of candidate data configuration options and when to start or stop the data collection for a UE.

RAN2's view is that if the RAN receives some information from the UE-side server via the CN or OAM there may be some dependencies between a data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and RAN. RAN2 have not agreed how this information is provided to the RAN and the details on the dependencies.

The RAN2 LS response confirms that it is within RAN2 merit how the UE and RAN node are configured with a list of candidate data configuration options and how the UEs are configured to start or stop data collection. It is also clarified by RAN2 in the LS response that, optionally, the RAN may use information provided by the CN as input to create candidate data configuration options. However, RAN2 did not indicate that information provided by the CN is a prerequisite for RAN to create candidate data configuration options. In addition, RAN2 did not specify what information is necessary and has not agreed how the RAN node receives such information.

It is understood from the RAN2 LS response that RAN2 considers the data configuration between UE and RAN as an independent procedure which can be enhanced if additional assistance information is received from UE-side server. As such, as part of the key issue 1 conclusions, it is proposed to conclude that the CN is not aware of candidate data configuration options available to the UE and the RAN and also that the CN cannot trigger UE or RAN to start or stop data collection. Any further dependency can be discussed after RAN2 continues to develop their solution and provide further feedback.

On selection of UEs (Questions 3.1, 3.2)

Question 3.1

In order to decide how to select UEs for UE data collection, SA2 would like to know if the UE data collection will be triggered by the UE model training server on individual UEs or if it will be triggered on a set of UEs, for example on UEs for a specific chipset. Note this does not necessarily imply that gNB will receive e.g. request for data collection of a specific chipset.

RAN2 Answer to Question 3.1

The measurement configuration for UE-side data collection is provided per UE, but RAN2 have not discussed how the UEs for data collection are selected.

Question 3.2

SA2 would like to ask RAN2 for any other information that could be relevant for selection of UEs.

RAN2 Answer to Question 3.2

RAN2 concluded that in some use-cases additional information such as (e.g., UE data collection and feature related radio capabilities, and user consent may be relevant for selection of UEs, but RAN2 have not discussed where (i.e., gNB, CN, UE server) this information would be considered.

The RAN2 LS response confirms that selection of UEs should be within SA2 merit. The answer to Question 3.2 is mostly related for selecting candidate data configuration options between UE and RAN.

## 1.2	 Proposed conclusions for Key Issue 1

The following main points were taken into account for the suggested conclusions:

-	Minimise impacts on existing core NFs, especially the AMF allowing fast deployment of the Data Collection feature. Lenovo prefers to avoid the data collection feature to rely on AMF supporting additional capability to enable the feature (e.g. NAS enhancements for enabling provisioning of configuration information to the UE, the UE indicating via NAS a data collection capability)

-	Keep the data collection procedures between UE and RAN and the Data Transfer request from UE-side server/Data Collection Function, independent.

For the following topics which are listed for further consideration as highlighted in clause 7.2.1 of 3GPP TR 23.700-04, the following conclusions are proposed:

How the 5GC NF controls UE data transfer.

From CN perspective the Data Collection Function relays a request for standardised data to a UE based on a request by  a UE-side server. As such, the role of the Data Collection Function is to indicate to, selected UE(s), the requirements on the data that needs to be transferred by the UE. The Data Collection NF can provide requirements for the UE on when to report data, i.e., when to transfer the data via a session with the data collection function (e.g. periodic, once UE buffer is full, time window etc).

In addition, the UE should be configured to be aware how the collected data are transferred to the Data Collection via an IP connection via a user plane connection over the 3GPP network (i.e. indicate to the UE how to map the transfer of data to a PDU session). This is supported using existing mechanism by the PCF providing appropriate URSP rules to the UE that helps the UE to determine the PDU session to use to transfer the data. The Traffic Descriptor of the URSP rule can correspond to well known 5-tuple (e.g. destination DCF address and port number).

Finally, the UE should be configured how to prioritise the transfer of standardised data against other type of active IP connections the UE may have. This is supported using existing mechanisms by the PCF (SM) providing appropriate QoS rule to help the UE determine how to prioritise the transfer of data against other active application sessions.

Whether and how the UE controls UE data transfer.

Given that the UE may have other active processes or user activities it should be possible for the UE to reject any transfer request.

Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

Propose to have a dedicated 5GC NF for controlling data transfer (Data Collection Function)

How the configuration information for UE data transfer is sent to the UE.

Propose the configuration information for UE data transfer to be sent to the Data Collection Function over UP as described, for example, in Solution 3. The UE establishes a session with the Data Collection Function and the Data Collection Function provides configuration information via the active data collection session.

How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

Preference is to allow the UE to discover the IP address of the Data Collection Function, e.g., by constructing an FQDN. Similar, functionality has been supported in V2X where the UE discovers a V2X control function by self-constructing an FQDN. As such there is no need for the 5GC NF to send UP configuration information to the UE.

How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

It is proposed to conclude that the visibility requirement on UE transferring standardized data is to be supported by the NF responsible for data collection. How the visibility requirements is fulfilled (i.e. how the NF checks that the UE transfers only standardized data) is internal implementation at the NF out of scope of 3GPP.

Whether and how to select UEs for UE data collection.

It is proposed to allow the UE-side server to indicate type of UEs (identified by PEI) that should be used to transfer standardised data. The Data Collection Function selects UEs by interfacing with the AMF to obtain a list of UEs supporting the PEI

NOTE: Interfacing with the AMF to obtain a list of UEs based on PEI is an existing capability of the AMF.

Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

As the data transfer is carried out over IP between the UE and the Data Collection Function, the data transfer can be identified by a 5-tuple (e.g. destination address and port number). Hence the UE PCF can provide appropriate Traffic Descriptors to allow the UE to identify how to route traffic to a PDU session and the SM PCF can provide appropriate QoS rule to allow the UE to identify the QoS requirements of the data transfer.

As such there is also no need of a dedicated DNN/S-NSSAI for differentiating traffic of of collected UE data from the UE regular traffic per PDU Session level. It is also proposed to indicate in the conclusion that having a dedicated DNN/S-NSSAI for differentiating collected UE data from regular UE traffic as an option.

Whether the UE needs to be authorized for UE data collection.

There is no requirement from RAN2 indicating that the RAN needs to be notified of UE authorised for UE data collection. It is proposed to wait for further feedback from RAN whether such requirement would be useful.

Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

Currently, there is no requirement from RAN2 indicating alignment of UE data measurements with a data transfer request from the 5GC. Any further dependency can be discussed after RAN2 continues to develop their solution and provide further feedback.

# 2. Proposal

The following conclusions are proposed.

-	A new 5GC NF, Data Collection Function, is used to control the transfer of standardised data from the UE.

-	The Data Collection Function provide to, selected UE(s), configuration information indicating the standardised data to transfer based on standardised data requested by a UE-side server. The configuration information is provided by the Data Collection Function to selected UEs via a user plane connection. Details of the user plane connection between the UE and the Data Collection Function are to be specified by stage 3.

-	A UE may reject a data transfer request from Data Collection Function providing appropriate response (e.g. low battery, too many active resources).

-	The UE-side server may indicate to Data Collection Function (via an NEF), type of UEs (identified by PEI) where standardised data needs to be retrieved. The Data Collection Function selects UEs by interfacing with the AMF to obtain a list of UEs supporting the PEI.

-	The Data Collection Function does not directly trigger UE to start/stop data collection procedure with a RAN node. The Data Collection Function may provide requirements on when data should be collected (e.g. area of interest, time of data) but the decision on when data collection should start is up to UE and RAN negotiations.

-	A UP path is used between the UE and Data Collection Function, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session. Details of the user plane connection between the UE and the Data Collection Function are to be specified by stage 3.

-	The UE discovers the Data Collection Function by self-constructing an FQDN (e.g. DataCollection.mnc.mcc.pub.3gppnetwork.org)

-	To differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level, a dedicated S-NSSAI/DNN may be used or collected UE data may be identified based on 5-tuple (e.g. destination address and port number of data collection function).

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	The visibility requirement on UE transferring standardized data is to be supported by Data Collection Function. How the visibility requirement is fulfilled (i.e. how the NF checks that the UE transfers only standardized data) is internal implementation at the NF out of scope of 3GPP.

**** First Change****

### 7.1.1	Void

**** Second Change****

### 7.2.1	Void

**** Third Change****

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.x	Conclusions for KI#1

-	A new 5GC NF, Data Collection Function, is used to control the transfer of standardised data from the UE.

-	The Data Collection Function provide to, selected UE(s), configuration information indicating the standardised data to transfer based on standardised data requested by a UE-side server. The configuration information is provided by the Data Collection Function to selected UEs via a user plane connection. Details of the user plane connection between the UE and the Data Collection Function are to be specified by stage 3.

-	A UE may reject a data transfer request from Data Collection Function providing appropriate response (e.g. low battery, too many active resources).

-	The UE-side server may indicate to Data Collection Function (via an NEF), type of UEs (identified by PEI) where standardised data needs to be retrieved. The Data Collection Function selects UEs by interfacing with the AMF to obtain a list of UEs supporting the PEI.

-	The Data Collection Function does not directly trigger UE to start/stop data collection procedure with a RAN node. The Data Collection Function may provide requirements on when data should be collected (e.g. area of interest, time of data) but the decision on when data collection should start is up to UE and RAN negotiations.

-	A UP path is used between the UE and Data Collection Function, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session. Details of the user plane connection between the UE and the Data Collection Function are to be specified by stage 3.

-	The UE discovers the Data Collection Function by self-constructing an FQDN (e.g. DataCollection.mnc.mcc.pub.3gppnetwork.org)

-	To differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level, a dedicated S-NSSAI/DNN may be used or collected UE data may be identified based on 5-tuple (e.g. destination address and port number of data collection function).

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	The visibility requirement on UE transferring standardized data is to be supported by Data Collection Function. How the visibility requirement is fulfilled (i.e. how the NF checks that the UE transfers only standardized data) is internal implementation at the NF out of scope of 3GPP.

**** End of Change ****