3GPP TSG SA WG2 Meeting #172	S2-2510053

17- 21 November, 2025, Dallas, US(revision of S2-220xxxx)()          (revision of S2-250xxxx)

Source:	China Mobile, AT&T, Boost Mobile Network, ZTE

Title:	[WT#5&KI] WT and KI of Data framework design for 6G network

Document for:	Agreement

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This paper proposes the WT and KI about the WT5 Data framework design for 6G network.

1. Justifications

In this paper, it is proposed to combine bullet 1 into bullet 2 based on last meeting postponed version, and to study the use cases during the study phase. It is not feasible to study only use cases without providing solutions for architecture design for 6G data framework. When we discuss about one scenario, we should have a whole picture of the impact of the scenario, e.g., functionalities, data type, data source and data exposure requirement.

**** First Change, postponed version ****

A.X 	Work Task X: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA5 is required. Common functionalities for data handling may be identified between SA2 and SA5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The WT includes the following aspects:

NOTE 2: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

- Identify high level use cases (e.g., AI/ML in the core, UE data collection for UE-sided model training, Sensing, Positioning) potentially subject to the data framework and study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution) )

- data labelling/metadata handling

- data processing (e.g., data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 2: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

NOTE 4:	 Coordination with SA5 may be required for charging aspects.

NOTE 5:  Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 6:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

- Study system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system in order to satisfy national or regional regulatory.

NOTE 7:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** Second Change (same with WT description) ****

5.X	Key Issue #X: Supporting 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA5 is required. Common functionalities for data handling may be identified between SA2 and SA5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The KI includes the following aspects:

1. Identify high level use cases (e.g., AI/ML in the core, UE data collection for UE-sided model training, Sensing, Positioning) potentially subject to the data framework and study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution) )

- data labelling/metadata handling

- data processing (e.g., data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 2: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

NOTE 4:	 Coordination with SA5 may be required for charging aspects.

NOTE 5:  Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 6:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

2. Study system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system in order to satisfy national or regional regulatory.

NOTE 7:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** End of Changes ****