3GPP TSG-SA2 Meeting #172	S2-2510081

# Dallas, USA, November 17 – 21, 2025		(revision of S2-2508654)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0090 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NG-RAN Configuration Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.369, the NG-RAN configuration information is specified as below: the NG-RAN configuration information (NG-RAN node ID, AIoT Area ID list, optionally RAN Reader(s) associated with each AIoT area and, optionally, the location served by each RAN Reader) In the definition above, the elements of the NG-RAN configuration information is not clearly structured, which causes risks for misinterpretation of TS 23.369. For example, the AIoT area information can be configured with both the NG-RAN node and the RAN readers at the same time. If both are present, they are redundant and may conflict if not aligned. In SA2, the RAN Reader selection is understood as optional in the AIOTF: - if RAN Reader selection is not needed, the RAN Reader(s) should not be configured with the NG-RAN node. In this case, the AIoT Area ID list should be configured with the NG-RAN node. - If RAN Reader selection is needed, the RAN Reader(s) should be configured with the NG-RAN node. In this case, the AIoT Areas should be configured with the RAN Readers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the structure of NG-RAN configuration information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear definition which causes risks for misinterpretation which may lead to interoperability issues. - Redundant or potentially conflicting AIoT area information in NG-RAN configuration information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Start of Changes * * *

### 5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN Readers based on the configured NG-RAN configuration information and the received AIoT service operation request. The AIOTF may also select a RAN Reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN configuration information for the selection. For each NG-RAN node, either served AIoT Area ID list or RAN Reader list is included. For each RAN Reader, served AIoT area ID list and optional location information is included. The provisioning of the NG-RAN configuration information can be performed via OAM or local configuration.

When the AIOTF receives an AIoT service operation request, based on the received Target Area information in the AIoT service operation request and the NG-RAN configuration information, the AIOTF selects the NG-RAN node(s) and optionally RAN Reader(s) to handle the request and the AIOTF determines a Requested Service Area Information for each selected NG-RAN node. The Requested Service Area Information provided by the AIOTF to each selected NG-RAN node can be empty, or includes a list of AIoT Area IDs, a list of RAN Reader IDs, or both a list of AIoT Area IDs and a list of RAN Reader IDs. How the NG-RAN node determines RAN Readers based on the Requested Service Area Information is specified in TS 38.300 [5].

NOTE 1:	The Target Area information in a received AIoT service operation request can span multiple NG-RAN node(s) or can be a subset of the supported AIoT areas of a single NG-RAN node.

The AIOTF sends the AIoT service operation request to each selected NG-RAN node, either directly or through the selected AMF, including the Requested Service Area Information for the NG-RAN node.

If an AIoT service operation request received by the AIOTF includes individual target AIoT Device Perminant ID(s), the AIOTF may consider the last known serving RAN Reader, if available from the AIoT Device context to determine the NG-RAN node and RAN Reader(s) for the request. In this case the last known serving RAN Reader ID is sent as the Requested Service Area to the selected NG-RAN node.

NOTE 2:	The RAN Reader ID is not exposed to the AF.

NOTE 3:	An AIOTF receives the Inventory Report from an NG-RAN node includes a RAN Reader ID that represents the AIoT Device’s location at Reader granularity.

NOTE 4:	The AIOTF uses the RAN Reader ID and the NG-RAN node to update the last known serving RAN Reader information in the local AIoT Device context.

# * * * End of Changes* * *