3GPP TSG-WG SA2 Meeting #172	S2-2510095

Dallas, United States, 17th Nov – 21st Nov, 2025

Source:	ETRI

Title:	KI#2: Interim Agreement Update and Conclusion

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Proposal to include the interim agreement update and conclusions for KI#2 in TR 23.700-04.

1. Introduction/Discussion

During the KI#2 study, several topics remained open for further consideration, including whether the UPF can act as a consumer of analytics, whether new UPF or SMF events are required for NWDAF data collection, and how to reduce the reporting load and signalling overhead by appropriate filtering or pre-processing.

This contribution proposes how to the existing KI#2 framework can also apply when user-plane traffic is encrypted. As most traffic in modern deployments is encrypted (e.g. TLS, QUIC), payload inspection is not feasible.

It is therefore proposed that the NWDAF derive analytics from observable metadata, such as packet-size distribution, inter-arrival time, and packet rate, obtained from UPF counters or OAM performance data.

This approach enables traffic pattern analysis and abnormal behaviour detection without compromising data confidentiality or requiring payload access.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change * * * *

7.1.2	Agreed Principles for KI#2

7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	Volume, rate or burst size of abnormal traffic.

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s).

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

-	From UPF/SMF:

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.).

-	Traffic flow filters.

-	Identification of corresponding UEs.

-	Information of traffic characteristics.

NOTE: For encrypted traffic flows (e.g. TLS, QUIC), the NWDAF may derive analytics based on observable metadata such as packet size distribution, inter-arrival time, and packet rate, which can be collected from UPF counters or OAM performance data, without inspecting any payload content.

-	The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501 [2]).

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations.

-	UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations).

-	adjust packet processing resources.

NOTE 1:	UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level.

-	The PCF as consumer of NWDAF analytics may for instance take the following actions:

-	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

-	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2:	This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF. The PCF may take the analytics outputs into account to derive the QoS for the target application(s).

-	The NWDAF may provide the output analytics on traffic pattern information, including:

-	Maximum Burst Size/ Maximum Data Burst Volume, Maximum Bitrate, Guaranteed Bitrate, 5GS delay, Packet Error Rate, Periodicity.

-	Corresponding information of the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s).

-	Identifiers of affected UEs, e.g. UE ID(s).

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow(s), e.g. packet filter, application ID of the traffic flow.

-	Information identifying the associated UEs.

-	Information related to the data flow characteristics, including, N6 delay, congestion information, indication that UPF drops packets and packet drop rate, data volume, packet delay, data rate, periodicity, and packet interval.

NOTE 1:	The indication of UPF dropping packets and packet drop rate is only applied to the abnormal scenarios and the scenarios when UPF dropped buffered data.

NOTE 2:	Whether the "User Data Usage Measures" event or another existing event is extended or new event(s) are defined will be determined in the normative phase.

NOTE Y: For encrypted traffic flows (e.g. TLS, QUIC), the NWDAF may derive the above analytics based on observable metadata such as packet size distribution, inter arrival time and packet rate, without inspecting any payload content. This enables the QoS related analytics to be applied also to encrypted flows.

-	The consumer actions of the PCF may take the output Analytics into account, e.g. to generate or update PCC rules by adjusting the QoS.

-	The analytics consumer may include the Analytics Filter Information in the request, such as application description, UE ID/address, DNN and S-NSSAI, etc.

# * * * * Second change * * * *

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-  Whether and how the UPF can be the consumer of the user plane traffic pattern and behaviour analysis for Use Case 1 or not.

-  How to handle analytics for encrypted traffic flows (e.g. TLS, QUIC) by using observable metadata such as packet size distribution, inter arrival time and packet rate, without inspecting payload content.

# * * * * Third change * * * *

8	Conclusions

It is agreed to use the following principles as baseline for normative work.

- NWDAF provides analytics to support user plane traffic pattern and behaviour analysis for network optimization, QoS adjustment and abnormal traffic mitigation.

- Two representative use cases were studied: abnormal traffic detection and mitigation, and traffic pattern analytics for QoS optimization.

- No impact on RAN or UE operation was identified.

- Analytics consumers may include SMF, PCF, OAM, AF, and UPF, depending on operator configuration.

- To reduce reporting load and control plane signaling overhead, Analytics Filter Information may be used to pre-filter or aggregate input data at the UPF or SMF before reporting to NWDAF.

- Input data from UPF and SMF include traffic identifiers, source filters, traffic characteristics (e.g. data rate, burst size, delay, loss rate, congestion indication) and UE identifiers.

- For encrypted user plane traffic (e.g. TLS, QUIC), NWDAF may perform analytics using observable metadata such as packet size distribution, inter-arrival time and packet rate, without inspecting payload content.

- This enables traffic pattern analysis and abnormal behaviour detection even when payloads are encrypted.

- Output analytics may include statistics and predictions on traffic type, rate, volume, burst characteristics, and identifiers of affected UEs, UPFs or PDU sessions.

- The analytics results may be used by the consumer (e.g. SMF, PCF or UPF) to perform actions such as traffic shaping, bandwidth limitation, or QoS parameter adjustment.

- Analytics filter information may include identifiers such as DNN, S-NSSAI, UE address, application ID, or packet filter set.

NOTE 1: Further study is needed in the normative phase regarding the definition of observable metadata, collection granularity and event exposure mechanisms.

NOTE 2: It is also to be discussed whether new UPF or SMF events are required for NWDAF data collection, or existing QoS Monitoring and User Data Usage Measures events can be reused.

NOTE 3: Privacy and confidentiality aspects of observable metadata shall be considered during normative work.

NOTE X: For encrypted user-plane traffic (e.g. TLS, QUIC), it is recognised that the NWDAF may derive analytics based on observable metadata such as packet-size distribution, inter-arrival time, and packet rate, without inspecting payload content. This enables traffic-pattern analysis and abnormal-behaviour detection while maintaining data confidentiality.

# * * * * End of changes * * * *