3GPP TSG-SA WG2 Meeting #172	S2-2510118

Dallas, USA, November 17 – November 21, 2025

Source:	vivo

Title:	[WT#1.2, Localized Service Access] Support of Localized Service Access

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to update the WT scope of supporting localized service based on the postponed version (S2-2509789) from pen holder.

.

Discussion

In Non-public network related features defined in 5G, the UE needs to support at least the following enhancements in order to access the SNPN or PNI- NPN:

(1) SNPN:

According to the clause 5.30.2.3 of TS 23.501, an SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the subscribed SNPN

-	Subscription identifier (SUPI) and credentials for the subscribed SNPN;

-  Some other optional information as defined in this clause.

(2) PNI-NPN:

According to the clause 5.30.3.3 of TS 23.501, the UE needs to support the following configuration for the access to PNI-NPN:

To use CAG, the UE, that supports CAG as indicated as part of the UE 5GMM Core Network Capability, may be pre-configured or (re)configured with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	 an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	each entry of the Allowed CAG list may be associated with time validity information containing one or more time periods;

In general, considering these enhancements on the UE side, the uses need to upgrade the terminals in order to access the SNPN or PNI-NPN for local service access. This is a problem in the real business case, since the cost for the deployment of SNPN or PNI-NPN is relatively high.

In 6G, in order to better support the deployment of localized service access, it’s better to have less impacts on the UE side.

Proposal 1: To support the localized service access in 6G system, the related feature should have less impacts on the UE side. The UE impact should be considered during the evaluation.

Considering that the network proving the localized service access should be provided by the operators and under the MNO control, it is proposed to clarify that the network providing localized service access should be within the PLMN network.

Proposal 2:  It needs to be clarified that the network providing localized service access should be within the PLMN network. The NOTE 3b is needed for the clarification.

**** First Change (All text new, changes shown w.r.t S2-2509789 by penholder)****

Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1: Support for access to Localized Services as specified in 5G (refer to TS 23.501) is the starting point for the discussion on support of 6G use cases and requirements related to localized service access.

- Authorization and authentication of UE for localized service access including subscription aspects.

  - Void

NOTE 2: Authorization, authentication aspects need to be coordinated with SA3

- Void

- Whether and how to support UE mobility, service continuity between PLMN network and network providing localized service access, or between the networks providing localized service access, including interface and interaction procedure between PLMN network and networks providing localized service access, or between the networks providing localized service access.

NOTE 3a: Service continuity refers to accessing the same service (e.g. internet connectivity) that is provided by both the initial/source network and the latter/target network.

NOTE 3b: Localized service access can be provided by a part of a PLMN as well.

- Void

- Void

- Void

- [Further disc.] Whether and how to perform the network access control in the network providing localized service access.

NOTE 4: This WT should have less impacts on the UE side. The UE impact should be considered during the evaluation.

NOTE 5:	Void

**** Second Change (All new text)****

5.X. Key Issue #X: Localized service access

The study for 6G localized service access includes the following aspects:

-     How to authorize and authenticate the UE for localized service access including subscription aspects.

NOTE 2: Authorization, authentication aspects need to be coordinated with SA3

- How to support UE mobility, service continuity between PLMN network and network providing localized service access, or between the networks providing localized service access, including interface and interaction procedure between PLMN network and networks providing localized service access, or between the networks providing localized service access.

NOTE 3a: Service continuity refers to accessing the same service (e.g. internet connectivity) that is provided by both the initial/source network and the latter/target network.

NOTE 3b: Localized service access can be provided by a part of a PLMN as well.

-   How to simplify the network providing localized service access.

NOTE 4: This WT should have less impacts on the UE side. The UE impact should be considered during the evaluation.

**** End of Changes ****